3GPP TSG-RAN WG4 Meeting #110	R4-2402731

# Athens, Greece, February 26 – March 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM Performance Requirements for Enhanced RedCap

Agenda item:	8.23.3

Document for:	Discussion

Introduction

The revised Rel-18 WID [1] for enhanced RedCap contains the enhancements for performance requirements:

Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above-mentioned enhancements and core requirements [RAN4].

For above depicted enhancements, RRM performance requirements are discussed in this contribution and a proposal on the further proceeding is made.

Discussion

Impacts to RRM core requirements for the above enhancements in the WID objective are discussed in this section.

Test cases for eDRX cycle > 10.24 sec in FR1 and FR2

It has been studied for the requirements on Rel-18 RedCap UE in RRC_INACTIVE state when both eDRX IDLE and eDRX INACTIVE cycles are greater than 10.24 sec. In particular, the core requirements are specified according to the agreements on how and when to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s and when the PTWs are partially-overlapping/ non-overlapping (not-coinciding) for serving/neighbor cell measurements. It is also assumed that the requirements for RRC_INACTIVE state with eDRX cycle > 10.24 sec use the same number of samples needed for the intra-frequency/inter-frequency measurement for RRC_IDLE state with eDRX cycle > 10.24 sec. Based on the specified core requirements, it is needed to discuss whether new test cases need to be introduced. It is observed that Annex A of TS38.133 does not define any test case on the extended eDRX to verify the behavior of eRedCap UE in Rel-18. Thus, RAN4 should discuss whether to specify new test cases for extended eDRX requirements in RRC_INACTIVE state.

RAN4 to discuss whether to specify new test cases for extended eDRX requirements in RRC_INACTIVE state.

Test cases transition requirements

In Rel-18, the transition requirements have been specified for the transition between short INACTIVE eDRX (≤ 10.24s) and long INACTIVE eDRX ( 20.48s). Therefore, the transition requirements are appliable to the cases when UE is changed with its eDRX_IDLE cycle length, eDRX_INACTIVE cycle length, INACTIVE RAN DRX length, or PTW configuration. In those cases, the UE will follow the less stringent requirement of the two requirements of two states. For the core requirements on transition, no additional performance requirement needs to be specified.

No additional performance requirement needs to be specified for transition.

Conclusion

This contribution has investigated the impact on RRM performance requirements from the objectives in the Rel-18 enhanced RedCap WID [1]. The following proposals are made.

- RAN4 to discuss whether to specify new test cases for extended eDRX requirements in RRC_INACTIVE state.

No additional performance requirement needs to be specified for transition.

- References

- RP-232671, “Revised WID on Enhanced support of reduced capability NR devices”, Ericsson, RAN#101.