3GPP TSG-RAN WG4 Meeting #110	R4-2402735

# Athens, Greece, February 26 – March 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM Performance Requirements for RedCap Positioning

Agenda item:	8.14.3.3

Document for:	Discussion

Introduction

The specification of positioning for RedCap devices is one of the objectives in the Rel-18 WID on expanded and improved positioning [1]. The WID depicts the RAN4 work related to this accuracy improvement technology.

This contribution investigates impacts to RRM performance requirements for NR positioning enhancements.

Discussion

The impacts to RRM performance requirements for NR positioning are discussed in this section.

RedCap Positioning

Accuracy Requirements for RedCap UE with FH

In [2], the RRM core requirements for PRS measurement with FH are discussed how UE should report the measurements with FH. In particular, UE can report the measurements based on either multiple hops or a single hop. In [3], it is agreed to define requirements for positioning measurement based on multiple hops. When multiple hops are used in a single MG occasion, the requirements on the number of hops needs to be specified.

When UE is reporting based on multiple hops, the accuracy requirements need to be defined. For example, the accuracy requirements have to be different depending on the bandwidth of the reported measurement. Consequently, it is needed to discuss whether to define the same or different accuracy requirements depending on the number of hops within a single MG occasion. The bandwidth size is a dominant factor to the accuracy of the RSTD and UE Rx-Tx time difference measurement. Based on Cramer-Rao bound, it is clear that the timing measurement accuracy is improved as BW increases. Depending on the number of frequency hops, the accuracy requirement could not be the same, so RAN4 should define separate accuracy requirement at least for two cases such as a single frequency hop and multiple frequency hops.

RAN4 to discuss whether to define the same or different accuracy requirements depending on the number of hops within a single MG occasion.

RAN4 to define separate accuracy requirement at least for two cases such as a single frequency hop and multiple frequency hops.

For simulations, PRB overlapping is considered between the frequency hops for 1 Rx and 2 Rx antennas at the UE. For the alignment of simulation results, RAN4 should discuss the modeling of the frequency impairments due to phase shifts between frequency hops and include the PRB overlap as a parameter in the simulation assumptions.

RAN4 to discuss the modeling of the frequency impairments due to phase shifts between frequency hops and include the PRB overlap as a parameter in the simulation assumptions.

Conclusion

The scope of RRM impacts on performance requirements as part of the Rel-18 WI on Expanded and Improved NR positioning is investigated in this contribution.

The following proposal is made.

- RAN4 to discuss whether to define the same or different accuracy requirements depending on the number of hops within a single MG occasion.

RAN4 to define separate accuracy requirement at least for two cases such as a single frequency hop and multiple frequency hops.

RAN4 to discuss the modeling of the frequency impairments due to phase shifts between frequency hops and include the PRB overlap as a parameter in the simulation assumptions.

- References

- RP-230328, “Revised WID on Expanded and Improved NR Positioning”, Intel Corporation, RAN#99

- R4-2317207, “Topic summary for [108-bis][215] NR_pos_enh2_part1,” Moderator (Ericsson), RAN4#108bis.

- R4-2321543, “WF on R18 NR positioning – RedCap positioning and PRS/SRS BW aggregation,” Ericsson, RAN4#109.