3GPP TSG-RAN4 Meeting #110	R4-2402764

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.113 | CR | 0097 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf) CR to TS 36.113 on update of FRC used in performance criteria |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Fixed reference channel models used in Performance Criteria for continuous and transient phenomena for BS need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 6.1, 6.2: update of fixed reference channel models used for E-UTRA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Fixed reference channel models used in Performance Criteria for continuous and transient phenomena for BS are not up to date. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------- start of changes----------

## 6.1	Performance criteria for continuous phenomena for BS

The test should, where possible, be performed using a bearer with the characteristics of data rate and throughput defined in Table 6.1.1 and Table 6.1.1a. If the test is not performed using one of these bearers (for, example, of none of them are supported by the BS), the characteristics of the bearer used shall be recorded in the test report.

The throughput in Table 6.1.1 or Table 6.1.1a is stated relative to the maximum throughput of the FRC. For E-UTRA the maximum throughput for an FRC is equal to the payload size * the number of uplink subframes per second. For NB-IoT the Maximum throughput for an FRC equals the Payload size / (Number of Resource Unit * time to send one Resource Unit).

The BS Uplink and Downlink paths shall each meet the performance criteria defined in Table 6.1.1 and Table 6.1.1a during the test. If the Uplink and Downlink paths are evaluated as a one loop then the criteria is two times the throughput reduction shown in Table 6.1.1 and Table 6.1.1a. After each test case BS shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.1.1: BS E-UTRA Performance Criteria for continuous phenomena for BS

| E-UTRA Channel Bandwidth [MHz] | Bearer Information Data Rate | Performance Criteria 1,2 |
| --- | --- | --- |
| 1.4 | FRC A1-1 in Annex A.1 in TS 36.104 [2] | Throughput > 95 % No loss of service |
| 3 | FRC A1-2 in Annex A.1 in TS 36.104 [2] | Throughput > 95 % No loss of service |
| 3 | FRC A1-6 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation | Throughput > 95 % No loss of service |
| 5 | FRC A1-3 in Annex A.1 in TS 36.104 [2] | Throughput > 95 % No loss of service |
| 5 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation | Throughput > 95 % No loss of service |
| 10 | FRC A1-3 in Annex A.1 in TS 36.104 [2] 3 FRC A1-8 in Annex A.1 in TS 36.104 [2]4 | Throughput > 95 % No loss of service |
| 10 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation | Throughput > 95 % No loss of service |
| 15 | FRC A1-3 in Annex A.1 in TS 36.104 [2] 3 | Throughput > 95 % No loss of service |
| 15 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation | Throughput > 95 % No loss of service |
| 20 | FRC A1-3 in Annex A.1 in TS 36.104 [2] 3 FRC A1-9 in Annex A.1 in TS 36.104 [2]4 | Throughput > 95 % No loss of service |
| 20 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation | Throughput > 95 % No loss of service |

Table 6.1.1a: NB-IoT BS Performance criteria for continuous phenomena for BS

| NB-IoT Sub-carrier spacing [kHz] | Bearer Information Data Rate | Performance Criteria 1, 2 |
| --- | --- | --- |
| 15 | FRC A14-1 in Annex A.14 in TS 36.104 [2] | Throughput > 95 % No loss of service |
| 3.75 | FRC A14-2 in Annex A.14 in TS 36.104 [2] | Throughput > 95 % No loss of service |

NOTE 1:	The performance criteria, Throughput > 95 % / No loss of service, applies also if a bearer with another characteristics is used in the test.

NOTE 2:	The performance criteria, Throughput > 90 % / No loss of service, applies instead if the Uplink and Downlink paths are evaluated as a one loop.

NOTE 3:	This is the information data rate of a single instance of the bearer mapped to 25 resource blocks. The performance criteria shall be met for each consecutive application of a single instance of the bearer mapped to disjoint frequency ranges with a width of 25 resource blocks each. This reference measurement channel is not applied for Band 46 and Band 49.

NOTE 4:	This is the information data rate of a single instance of the bearer mapped to a single interlace. The performance criteria shall be met for each application of a single instance of the bearer mapped to each single interlace. This reference measurement channel is only applied for Band 46 and Band 49.

## 6.2	Performance criteria for transient phenomena for BS

The test should be, where possible, be performed using a bearer with the characteristics of data rate and throughput defined in Table 6.2.1 and Table 6.2.1a. If the test is not performed using one of these bearers (for, example, of none of them are supported by the BS), the characteristics of the bearer used shall be recorded.

The throughput in Table 6.2.1 or Table 6.2.1a is stated relative to the maximum throughput of the FRC. For E-UTRA the maximum throughput for an FRC is equal to the payload size * the number of uplink subframes per second. For NB-IoT the Maximum throughput for an FRC equals the Payload size / (Number of Resource Unit * time to send one Resource Unit).

The BS Uplink and Downlink paths shall each meet the performance criteria defined in Table 6.2.1 and Table 6.2.1a during the test. If the Uplink and Downlink paths are evaluated as a one loop then the criteria is two times the throughput reduction shown in Table 6.2.1 and Table 6.2.1a. After each test case BS shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.2.1: BS Performance Criteria for transient phenomena for BS

| E-UTRA Channel Bandwidth [MHz] | Bearer Information Data Rate | Performance Criteria 1, 2 |
| --- | --- | --- |
| 1.4 | FRC A1-1 in Annex A.1 in TS 36.104 [2] | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 3 | FRC A1-2 in Annex A.1 in TS 36.104 [2] | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 3 | FRC A1-6 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 5 | FRC A1-3 in Annex A.1 in TS 36.104 [2] | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 5 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 10 | FRC A1-3 in Annex A.1 in TS 36.104 [2] 3 FRC A1-8 in Annex A.1 in TS 36.104 [2]4 | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 10 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 15 | FRC A1-3 in Annex A.1 in TS 36.104 [2] 3 | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 15 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 20 | FRC A1-3 in Annex A.1 in TS 36.104 [2] 3 FRC A1-9 in Annex A.1 in TS 36.104 [2]4 | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 20 | FRC A1-7 in Annex A.1 in TS 36.104 [2] for E-UTRA with NB-IoT in-band operation | Throughput < 95 % temporarily, however the communication link shall be maintained |

Table 6.2.1a: NB-IoT BS Performance Criteria for transient phenomena for BS

| NB-IoT Sub-carrier spacing [kHz] | Bearer Information Data Rate | Performance Criteria 1, 2 |
| --- | --- | --- |
| 15 | FRC A14-1 in Annex A.14 in TS 36.104 [2] | Throughput < 95 % temporarily, however the communication link shall be maintained |
| 3.75 | FRC A14-2 in Annex A.14 in TS 36.104 [2] | Throughput < 95 % temporarily, however the communication link shall be maintained |

NOTE 1:	The performance criteria, Throughput < 95 % temporarily / however the communication link shall be maintained, applies also if a bearer with another characteristics is used in the test.

NOTE 2:	The performance criteria, Throughput < 90 % temporarily / however the communication link shall be maintained, applies instead if the Uplink and Downlink paths are evaluated as a one loop.

NOTE 3:	This is the information data rate of a single instance of the bearer mapped to 25 resource blocks. The performance criteria shall be met for each consecutive application of a single instance of the bearer mapped to disjoint frequency ranges with a width of 25 resource blocks each. This reference measurement channel is not applied for Band 46.

NOTE 4:	This is the information data rate of a single instance of the bearer mapped to a single interlace. The performance criteria shall be met for each application of a single instance of the bearer mapped to each single interlace. This reference measurement channel is only applied for Band 46.

## 6.3	Performance criteria for continuous phenomena for Ancillary equipment

---------- end of changes----------