3GPP TSG-RAN WG4 Meeting #110 	R4-2402802

Athens, GR, 26 February – 01 March, 2024

Agenda Item:	8.14.3.1

Source: 	Ericsson

Title:	On SL positioning performance

Document for:	Discussion

# Introduction

The latest agreed WF is in [15].

In this contribution, we discuss issues related to SL positioning performance.

# Discussion

## Side conditions for SL PRS measurements in RAN4 requirements

Side conditions need to be defined for core requirements and accuracy requirements.

- Proposal 1 (side conditions): For SL RSTD, the side conditions assumed in RAN4 requirements are: ≥0 dB for the reference link and ≥-6 dB for the measured link.

- Proposal 2 (side conditions): For SL Rx-Tx, the requirements are defined down to -6 dB.

- Proposal 3 (side conditions): Given that SL PRS-RSRP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRP can be defined down to -6 dB.

- Proposal 4 (side conditions): Given that SL PRS-RSRPP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRPP can be defined down to -6 dB.

- Proposal 5 (side conditions): For SL AoA, the requirements apply down to -6 dB, regardless of whether accuracy requirements are specified or not.

- Proposal 6 (side conditions): For SL RTOA, the requirements apply down to -6 dB, regardless of whether accuracy requirements are specified or not.

## Measurement accuracy requirements

- Proposal 7 (measurement accuracy requirements): RAN4 will define measurement accuracy requirements for:

  - PRS RSTD

  - PRS Rx-Tx

  - PRS RSRP

  - PRS RSRPP

  - FFS: PRS AoA

  - FFS: PRS RTOA

- Proposal 8 (measurement accuracy requirements): Two sets of accuracy requirements are to be defined for:

  - 1 sample for SL-PRS BW>48 PRBs,

  - 4 samples for 24 PRBs ≤SL-PRS BW≤48 PRBs.

## Test cases for SL positioning measurements

- Proposal 9 (test cases): Test cases for all SL positioning measurements are specified (SL RSTD, RSRP/RSRPP, Rx-Tx, AoA, and RTOA).

- Proposal 10 (test cases): For each SL positioning measurement type (for which accuracy requirements are defined), test cases for measurement accuracy requirements cover:

  - SL-PRS BW>48 PRBs, and

  - 24 PRBs ≤SL-PRS BW≤48 PRBs.

## Specification structure

- Proposal 11 (specification structure): A new separate section is created for SL positioning measurement performance in clause 10, e.g.:

  - 10.4A		NR Sidelink Measurements for Positioning

# Summary

The following have been proposed in the current contribution:

- Proposal 1 (side conditions): For SL RSTD, the side conditions assumed in RAN4 requirements are: ≥0 dB for the reference link and ≥-6 dB for the measured link.

- Proposal 2 (side conditions): For SL Rx-Tx, the requirements are defined down to -6 dB.

- Proposal 3 (side conditions): Given that SL PRS-RSRP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRP can be defined down to -6 dB.

- Proposal 4 (side conditions): Given that SL PRS-RSRPP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRPP can be defined down to -6 dB.

- Proposal 5 (side conditions): For SL AoA, the requirements apply down to -6 dB, regardless of whether accuracy requirements are specified or not.

- Proposal 6 (side conditions): For SL RTOA, the requirements apply down to -6 dB, regardless of whether accuracy requirements are specified or not.

- Proposal 7 (measurement accuracy requirements): RAN4 will define measurement accuracy requirements for:

  - PRS RSTD

  - PRS Rx-Tx

  - PRS RSRP

  - PRS RSRPP

  - FFS: PRS AoA

  - FFS: PRS RTOA

- Proposal 8 (measurement accuracy requirements): Two sets of accuracy requirements are to be defined for:

  - 1 sample for SL-PRS BW>48 PRBs,

  - 4 samples for 24 PRBs ≤SL-PRS BW≤48 PRBs.

- Proposal 9 (test cases): Test cases for all SL positioning measurements are specified (SL RSTD, RSRP/RSRPP, Rx-Tx, AoA, and RTOA).

- Proposal 10 (test cases): For each SL positioning measurement type (for which accuracy requirements are defined), test cases for measurement accuracy requirements cover:

  - SL-PRS BW>48 PRBs, and

  - 24 PRBs ≤SL-PRS BW≤48 PRBs.

- Proposal 11 (specification structure): A new separate section is created for SL positioning measurement performance in clause 10, e.g.:

  - 10.4A		NR Sidelink Measurements for Positioning

# References

[1] RP-230328, Revised WID on Expanded and Improved NR Positioning, Intel Corp, March, 2023.

[2] R4-2303190, WF on Expanded and Improved NR Positioning RRM requirements, Ericsson, March, 2023.

[3] R1-2301950, Summary #1 on Measurements and reporting for SL positioning, vivo, March 2023.

[4] R1-2301951, Summary #2 on Measurements and reporting for SL positioning, vivo, March 2023.

[5] R1-2301952	Summary #3 on Measurements and reporting for SL positioning, vivo, March 2023.

[6] R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT, April 2023.

[7] R4-2310163, WF on NR Positioning – SL positioning and CPP, CATT, May 2023.

[8] R4-2310077, Simulation assumptions for SL positioning, CATT, May 2023.

[9] R4-2314343, Ad-hoc minutes on R18 NR Positioning RRM requirements, Ericsson, Aug. 2023.

[10] R4-2314354, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Aug. 2023.

[11] R4-2314358, LS on SL positioning and CPP measurements report mapping, Aug. 2023.

[12] R4-2312724, On SL positioning, Ericsson, Aug. 2023.

[13] RP-232670, Revised WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, MediaTek, Sep. 2023.

[14] R4-2317380, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Oct. 2023.

[15] R4-2321524, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Nov. 2023.