3GPP TSG RAN WG4 Meeting #110	R4-2402845

Athens, Greece, 26 Feb. 2024 – 01 Mar. 2024

Agenda Item:	8.3.2.6

Source: 	Ericsson

Title:	Discussion on RTD requirements

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the remaining open issues in TCI state switching delay requirements for the multi-rx.

# Discussion

In RAN4#109 meeting it was discussed how to capture RTD requirements for UE supporting multi-Rx. There were few proposals to capture it as separate section and some other proposals that RTD condition can be mentioned as a side condition in the relevant requirements.

In the meanwhile, in MIMO evolution WI, we introduced following new section [2] for capturing the timing requirements for multi-TRP. We think capturing the requirements in this section is a reasonable approach as multi-Rx can be considered as a MIMO multi-TRP feature.

7.6.8 Minimum Requirements for Multi-TRPs A UE supporting [FG 40-2-1 or FG 40-2-2] shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in Table 7.6.8-1 below. A UE supporting [FG 40-2-1 or FG 40-2-2] shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in Table 7.6.8-2 below, provided that the UE indicates that it is capable of [FG 40-2-6]. Table 7.6.8-1: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE not capable of [FG 40-2-6] Frequency Range; Maximum receive timing difference. FR1; CP lengthnote 1. FR2-1; CP lengthnote 1. Note 1: CP length of the maximum SCS on the carrier.. Table 7.6.8-2: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE capable of [FG 40-2-6] Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8.

- Capture RTD requirements for multi-rx in section 7.6.8

# Summary

The following are proposed in this contribution.

- Capture RTD requirements for multi-rx in section 7.6.8

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] TS 38.133 18.4.0