3GPP TSG-RAN WG4 Meeting # 110				                              R4-2402876

Athens, GR, 26 Feb – 01 Mar, 2024

Title: 	Way forward on [110][332] NR_MIMO_OTA_enh

Agenda Item:	8.12.4

Source: 	CAICT

Document for:	Approval

Topic #1: FR1 MIMO OTA

Sub-topic 1-1 FR1 MIMO OTA test methodology

Issue 1-1-1: UE positioning for FR1 MIMO OTA

Agreement:

- To avoid misalignment and ambiguity, align the three coordinate systems, i.e. DUT coordinate system, Channel model coordinate system and chamber coordinate system, for FR1 MIMO OTA test methodology.

Issue 1-1-2: Power validation systematic offset

Agreement:

- Remove the statement in TS 38.151 C3.6 that allows the power validation result to be considered a systematic offset.

- The corresponding CR is agreed in R4-2400534.

Issue 1-1-3: Update to FR1 Calibration Procedure

Agreement:

- The CR on FR1 calibration procedure is agreed in R4-2403016.

Sub-topic 1-2 FR1 MIMO OTA channel model validation results

Issue 1-2: Completing channel model validation for band n1 4x4 MIMO

Agreement:

- Encourage Rel-17 aligned labs to complete CDL-C UMa channel model validation for 4x4 MIMO at band, and contribute to Measurement Campaign for band n1.  The deadline for channel model validation and data submission for measurement campaign is RAN4 #111 (May 2024).  Labs are allowed to submit channel model validation results for band n1 together with the measurement data for performance requirements development at RAN4 #111 (May 2024).

- Each volunteer lab shall use the same measurement setup and equipment for channel model validation, lab alignment, and measurement campaign.

Sub-topic 1-3 FR1 MIMO OTA lab alignment

Issue 1-3-1: Updated pass/fail limit for Rel-18 FR1 MIMO OTA lab alignment

Background: RAN5 finalized the definition of measurement uncertainty (MU) for FR1 MPAC system (in Table B.1-1 of TS 38.551).

Agreement:

- The pass/fail limit (defined as 0.75*MU) for Rel-18 FR1 MIMO OTA lab alignment is updated based on the latest MU agreed in RAN5. For bands≤3GHz, the value is updated to 0.75*2.61=1.96 dB.

Issue 1-3-2: Effect of power validation systematic offset on lab alignment

Agreements:

- With the agreement in Issue 1-1-2, the lab alignment outcome should be based on the measurement results without power validation systematic offsets.

Issue 1-3-3: Outcome of the 1st round Rel-18 FR1 MIMO OTA lab alignment

For information: The summary of 1st round lab alignment results is shown in Table 1 and Figs. 1~2.

Table 1. Summary of the 1st round FR1 MIMO OTA lab alignment results (without systematic offsets)

| Device | Band | TRMS measurement result [dBm/15kHz] |  |  |  |  |  | Average approach | Average value | Max-Min deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Lab 1 | Lab 2 | Lab 3 | Lab 4 | Lab 5 | Lab 6 |  |  |  |
| PAD_1 | n28 | -89.34 | -90.12 | -90.43 | -88.26 | -89.22 | -91.75 | Linear average | -89.85 | 3.49 |
| PAD_2 | n28 | -86.46 | -87.93 | -89.09 | -85.50 | -85.86 | -87.74 |  | -87.10 | 3.60 |
| PAD_3 | n28 | -86.26 | -89.34 | -89.23 | -86.19 | -86.49 | -88.36 |  | -87.65 | 3.15 |
| Device | Band | TRMS offset [dBm/15kHz] |  |  |  |  |  | Pass/fail limit |  |  |
|  |  | Lab 1 | Lab 2 | Lab 3 | Lab 4 | Lab 5 | Lab 6 |  |  |  |
| PAD_1 | n28 | 0.52 | -0.27 | -0.58 | 1.59 | 0.63 | -1.90 | +/- 0.75 MU, i.e., +/- 1.96 dB |  |  |
| PAD_2 | n28 | 0.64 | -0.83 | -2.00 | 1.60 | 1.24 | -0.64 |  |  |  |
| PAD_3 | n28 | 1.38 | -1.69 | -1.59 | 1.46 | 1.15 | -0.71 |  |  |  |

Figure 2. TRMS measurement results from 6 labs (without systematic offsets)

Figure 3. Deviations between TRMS measurement results from each lab and reference values (without systematic offsets)

Agreement:

- Based on the analysis in R4-2400033, the 1st round Rel-18 FR1 MIMO OTA lab alignment is concluded with the following outcome:

  - 5 labs are aligned in the 1st round.

- The remaining lab can resubmit the PAD measurement result at the next meeting and be confirmed as an aligned lab, without affecting the conclusion that the 5 labs are aligned.

Sub-topic 1-4 FR1 MIMO OTA requirements related work

Issue 1-4-1: Thresholds of data pool for specifying FR1 MIMO OTA requirements

Agreements:

- Thresholds for specifying FR1 MIMO OTA performance requirement:

  - Minimum number of data: 15

  - Minimum number of UE models: 12

- As long as the thresholds are met, all the data submitted by aligned labs will be taken into the data pool for specifying FR1 MIMO OTA performance requirements.

- Volunteer labs should submit the information of the UEs that are planned to be measured using the template in R4-2321191 to RAN4 secretary as soon as possible, in order to monitor if the above thresholds can be met.

Issue 1-4-2: Template for FR1 MIMO OTA Measurement Campaign

Agreements:

- The template in R4-2403019 is agreed for FR1 MIMO OTA Measurement Campaign data submission.

Issue 1-4-3: Effect of power validation systematic offset on the data pool for performance requirements

Agreements:

- With the agreement in Issue 1-1-2, the performance requirements should be based on the measurement data with no systematic offset applied.

Issue 1-4-4: Whether IEs can be used to identify 2Rx UE and 4Rx UE

Agreements:

- Not to use IE srs-TxSwitch or srs-TxSwitch-v1610 to differentiate 2Rx capable UE from 4Rx capable UE in a target testing RF band for FR1 MIMO OTA testing.

Topic #2: FR2 MIMO OTA

Sub-topic 2-1 Figure of Merit (FoM) for PC1 UE

Issue 2-1: FoM of FR2 MIMO OTA for PC1 UE

Agreement:

- For FR2 PC1 MIMO OTA, the performance metric is to be specified with 5 of 36 test points, i.e.,

MASC70=10log5110P70,110+110P70,210+⋅⋅⋅+110P70,510

where {P70,1, …, P70,5} are the best 5 sensitivity values from all the 36 constant density measurement points.

Sub-topic 2-2 FR2 MIMO OTA Lab Alignment Activity

Issue 2-2: FR2 MIMO OTA lab alignment activity

Agreement:

- Complete the 1st round lab alignment at the next meeting with the results from at least three labs. Allow remaining labs to submit measurement results afterwards.

- Return the PADs to ETS-L by RAN4 #110bis (Apr. 2024).

- All labs can submit data for Measurement Campaign, whether the data can be included in the data pool for specifying requirements depends on lab alignment outcome.

- The deadline to submit lab alignment results and the data for Measurement Campaign is RAN4 #111 (May 2024).

Sub-topic 2-3 FR2 MIMO OTA Measurement Campaign

Issue 2-3-1: Devices provision for FR2 MIMO OTA Measurement Campaign

Agreements:

- Further encourage companies to volunteer FR2 devices to test lab before measurement campaign is finished. It is suggested for companies to provide devices at least two weeks before April meeting starts.

Way forward:

- Volunteer companies are encouraged to provide FR2 smartphones for Measurement Campaign for specifying performance requirements. Preferably the smartphones can be available at Apr. meeting.

| Company Name | How many FR2 devices can be provided | To which volunteer lab(s) [pending on the lab alignment outcome] |
| --- | --- | --- |
|  |  | e.g., x to Lab A, y to Lab B |

Issue 2-3-2: Template for FR2 MIMO OTA Measurement Campaign

Agreements:

- The template in R4-2403014 is agreed for FR2 MIMO OTA Measurement Campaign data submission.

References

- R4-2402672, “Topic summary for [110][332] NR_MIMO_OTA_enh”, Moderator (CAICT), 3GPP TSG-RAN WG4 Meeting #110, Feb. 2024.

- R4-2403011, “Ad-hoc meeting minutes for [110][332] NR_MIMO_OTA_enh”, Moderator (CAICT), 3GPP TSG-RAN WG4 Meeting #110, Feb. 2024.