3GPP TSG-RAN WG4 Meeting # 110 	R4-2402885

Athens, Greece, Feb. 26th – March 1st, 2024

Agenda Item: 8.5.2.1

Source: MediaTek Inc.

Title: Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR

Document for: Discussion

# Introduction

In Rel-18, further work objective to complete the requirements for measurement without gaps is given in the work item description (WID) [1] as below:

- Define RRM requirements for measurement without gaps for the following cases - NR SSB-based inter-frequency and intra-frequency measurements without gaps for UEs reporting NeedForGapsInfoNR IE [RAN4] - Study whether the additional interruption is allowed when UE reporting ‘NeedForGapsInfoNR'. Further define the interruption length, occasion and ratio, if the interruption is allowed - Define related requirements, such as CSSF, measurement period, scheduling restriction etc.

Furthermore, the open issues from the previous meeting (meeting RAN4#109) are given below [2]:

# Discussion on DRX specific issues

The open issues from WF:

Issue 1-4-1: Interruption caused when DRX is configured larger than 320ms - Proposals - Option 1: No interruption is expected when DRX is configured larger than 320ms on the serving cell. - Option 2: Interruption is allowed, and it is according to Tcycle,i. - Option 3: No interruption is expected during DRX activity time, including DRX ON duration extended by inactivity-timer after each PDCCH reception. - Agreement: - RAN4 discusses interruption requirements for cases when DRX is configured in the maintenance part of the work item.

Issue 1-4-1: In general, we don’t understand the motivation to have no interruption for some scenarios when DRX is larger than 320ms or no interruption is expected when SMTC is during DRX-off and UE uses such SMTC to measure NFG measurements with interruption on a certain MO. The fundamental UE behaviour is to follow the DRX provided by the NW, which means the UE is not expected to wake up in the middle of DRX period to perform measurements. Thus, RAN4 shall follow the existing requirements of NCSG and MG as baseline

- For DRX based interruption ratio when DRX is configured larger than 320ms, RAN4 shall follow the existing requirements of NCSG or MG as baseline.

- For DRX based interruption ratio when DRX is configured smaller than 320ms, RAN4 shall follow the existing requirements of NCSG or MG as baseline.

# Summary

In this contribution, discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR is provided and we have the following proposals:

Proposal 1: For DRX based interruption ratio when DRX is configured larger than 320ms, RAN4 shall follow the existing requirements of NCSG or MG as baseline.

Proposal 2: For DRX based interruption ratio when DRX is configured smaller than 320ms, RAN4 shall follow the existing requirements of NCSG or MG as baseline.

# References

- RP-231477, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation, June 2023, Taipei Taiwan.

- R4-2321632, “WF on NR_MG_enh2_part2”, Intel Corporation, Chicago, USA, Nov 2023.