3GPP TSG-RAN WG4 Meeting # 110 	R4-2402886

Athens, Greece, Feb. 26th – March 1st, 2024

Agenda Item: 8.5.2.2

Source: MediaTek Inc.

Title: Discussion on inter-RAT measurements

Document for: Discussion

# Introduction

In Rel-18, a work objective to complete the requirements for measurement without gaps is given in the work item description (WID) [1] as below:

- Define RRM requirements for measurement without gaps for the following cases - Inter-RAT measurements without gaps [RAN4] - Inter-RAT NR measurements - Inter-RAT LTE measurement

In meeting RAN4#108bis, the above was discussed and the open issues were captured in the WF [2]. The analysis and discussion on the open issues are provided in the next section.

# Discussion on inter-RAT measurements without gaps – UE capabilities

The analysis and discussion are provided in the next section.

### Sub-topic 2-5 UE capabilities Issue 2-5-1: Reporting of UE capability interRAT-NeedForIntrNR-r18 - Proposals - Option 1: Reporting of interRAT-NeedForIntrNR-r18 capability should be done based on network request. - Option 2: Do not further discuss this issue. - Agreement: - RAN4 discusses upon this issue in the maintenance part of the work item.

Effective measurement window (EMW):

Regarding scheduling restrictions for inter-RAT EUTRAN, it is not clear how to define the scheduling restriction without the use of SMTC, therefore, RAN4 agreed to define alike SMTC window for LTE or effective measurement window. Effective measurement window value is equal to 5 ms as agreed in the previous meetings with effective measurements window defined as 40 ms and 80 ms. In addition, RAN4 agreed to introduce additional effective measurement window duration: 2ms and 5.5ms with periodicity 40ms, 80ms, yet UE capability for window duration of 2ms shall be introduced in case of sync is needed.

Nevertheless, the EMW is only needed when the UE causes scheduling restrictions. Besides, the UE can determine whether the UE in need for scheduling restrictions. Therefore, the following reporting capabilities for case b-1 and case b-2 can applied:

- When the UE require scheduling restriction for a specific carrier: the UE shall report to the NW that the UE can measure the a given/specific carrier frequency of inter-RAT EUTRAN without gap and without interruption; the UE shall not report EMW.

- When the UE require NO scheduling restriction for a specific carrier: the UE reports to the NW that the UE can measure the a given/specific carrier frequency of inter-RAT EUTRAN without gap and without interruption, also, the UE shall report the support of effective measurement window (EMW) for inter-RAT EUTRAN capability.

  - Such that network and UE can align the understand on when to expect scheduling restriction based on EMW configuration.

- When the UE reports for inter-RAT EUTRAN measurements without gap and without interruption and the UE cannot guarantee no scheduling restrictions, the UE shall report NCSG, or gaps.

- For Cases b-1 and b-2: When the UE require scheduling restriction for a specific carrier: the UE shall report to the NW that the UE can measure the a given/specific carrier frequency of inter-RAT EUTRAN without gap and without interruption; the UE shall not report EMW.

- For Cases b-1 and b-2: When the UE require NO scheduling restriction for a specific carrier: the UE reports to the NW that the UE can measure the a given/specific carrier frequency of inter-RAT EUTRAN without gap and without interruption, also, the UE shall report the support of effective measurement window (EMW) for inter-RAT EUTRAN capability.

- For Cases b-1 and b-2: When the UE reports for inter-RAT EUTRAN measurements without gap and without interruption and the UE cannot guarantee no scheduling restrictions, the UE shall report NCSG, or gaps.

# Summary

In this contribution, discussion on inter-RAT measurement without gaps is provided and we have the following proposals:

Proposal 1: For Cases b-1 and b-2: When the UE require scheduling restriction for a specific carrier: the UE shall report to the NW that the UE can measure the a given/specific carrier frequency of inter-RAT EUTRAN without gap and without interruption; the UE shall not report EMW.

Proposal 2: For Cases b-1 and b-2: When the UE require NO scheduling restriction for a specific carrier: the UE reports to the NW that the UE can measure the a given/specific carrier frequency of inter-RAT EUTRAN without gap and without interruption, also, the UE shall report the support of effective measurement window (EMW) for inter-RAT EUTRAN capability.

Proposal 3: For Cases b-1 and b-2: When the UE reports for inter-RAT EUTRAN measurements without gap and without interruption and the UE cannot guarantee no scheduling restrictions, the UE shall report NCSG, or gaps.

# References

- RP-231477, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation, June 2023, Taipei Taiwan.

- R4-2321632, “WF on NR_MG_enh2_part2”, Intel Corporation, Chicago, USA, Nov 2023.