3GPP TSG-RAN5 Meeting # 102	R4-2403065

# Athens, Greece, 26th February – 3rd March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0035 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UL RMCs updates for IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Perf |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UL Reference Measurement Channels for IoT NTN devices are not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Leverage applicable eMTC and NB-IoT UL Reference measurement channels from TS 36.101. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, A.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.521-4 CR 0015 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is a revision of R4-2402907 where Table A.4.2.2.1-1 and Table A.4.2.2.2-1 have been corrected to remove non applicable channel bandwidths and not applicable RMC. MediaTek has been added as co-sourcing company. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.108: "Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node (SAN) radio transmission and reception".

[3]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[4]	ITU-R Recommendation M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[5]	3GPP TS 36.307: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements on User Equipments (UEs) supporting a release-independent frequency band".

[6]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[7]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

[8]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[9]	ITU-R Recommendation SM.329-10, "Unwanted emissions in the spurious domain"

[10]	[ANSI C63.26-2015, American National standard for Compliance Testing of Transmitters Used in Licensed Radio Services, Accredited Standards Committee C63 – Electromagnetic compatibility]

[11]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[12]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[13]	3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks"

[14]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

[15]	3GPP TS 36.212: " Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

# 3	Definitions of terms, symbols and abbreviations

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

# A.3	Testing related to Satellite Access

## A.3.1	General

The following test conditions should be maintained for Satellite Access

-	The same ephemeris info will be maintained during each test.

-	A set of ephemeris information are pre-defined for each satellite corresponding to respective epoch times in TS 36.508 [14].

-	The range of the selected constant delay shift is as follows:

-	For NGSO an altitude of 600km and 1200km on a circular orbit are considered. The range of the one-way delay between UE and satellite is from 2ms (lowest value for LEO orbit 600km) to 6.67ms (highest value for LEO orbit 1200km).

-	Constant delay value is derived from ephemeris info (SIB31) and UE location associated to zero Doppler or non-zero Doppler value under test.

## A.3.2	Test condition for transmitter characteristics

All requriements in section 6 for transmitter characteristics, other than frequency error in clauses 6.4A.1 and 6.4B.1, shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

Frequency error requirement in clauses 6.4A.1 and 6.4B.1 shall be verified for at least two cases: one with zero Doppler condition and the other one with constant Doppler (different from zero) up to [0.93] ppm for GSO satellites and up to 24 ppm for NGSO satellites.

## A.3.3	Test condition for receiver characteristics

All requirements in section 7 for receiver characteristics shall be verified when Doppler conditions related to satellite motion for DL in service link are set to zero and delay conditions are set to constant for all types of satellites.

## A.3.4	Test condition for performance requirements

All requirements in section 8 for performance requirements shall be verified when Doppler conditions related to satellite motion for DL in service link are set to zero and delay conditions are set to constant for all types of NGSO satellites. The one-way delay between UE and satellite for NGSO at an altitude of 600km is 2ms.

# A.4	UL reference measurement channels

## A.4.1	General

The measurement channels in the following subclauses are defined to derive the requirements in clause 6 (Transmitter Characteristics) and clause 7 (Receiver Characteristics). The measurement channels represent example configurations of physical channels for different data rates.

### A.4.1.1	Applicability and common parameters

The UL reference measurement channels comprise transmission of PUSCH and Demodulation Reference signals only.

The following conditions apply:

-	1 HARQ transmission

-	Cyclic Prefix normal

-	PUSCH hopping off

-	Link adaptation off

-	Demodulation Reference signal as per TS 36.211 [3] subclause 5.5.2.1.2.

Where ACK/NACK is transmitted, it is assumed to be multiplexed on PUSCH as per TS 36.212 [15] subclause 5.2.2.6.

-	ACK/NACK 1 bit

-	ACK/NACK mapping adjacent to Demodulation Reference symbol

-	ACK/NACK resources punctured into data

-	Max number of resources for ACK/NACK: 4 SC-FDMA symbols per subframe

-	No CQI transmitted, no RI transmitted

### A.4.1.2	Determination of payload size

The algorithm for determining the payload size A is as follows; given a desired coding rate R and radio block allocation NRB:

1.	Calculate the number of channel bits Nch that can be transmitted during the first transmission of a given sub-frame.

2.	Find A such that the resulting coding rate is as close to R as possible, that is,

,

subject to

a)	A is a valid TB size according to clause 7.1.7 of TS 36.213 [12] assuming an allocation of NRB resource blocks.

b)	C is the number of Code Blocks calculated according to section 5.1.2 of TS 36.212 [15].

c)	For RMC-s, which at the nominal target coding rate do not cover all the possible UE categories for the given modulation, reduce the target coding rate gradually (within the same modulation), until the maximal possible number of UE categories is covered.

3.	If there is more than one A that minimises the equation above, then the larger value is chosen per default and the chosen code rate should not exceed 0.93.

### A.4.1.3	Overview of UL reference measurement channels

In Table A.4.1.3-1 to A.4.1.3-2 are listed the UL reference measurement channels specified in Annexes A.4.2 and A.4.3 of this release of TS 36.102. This table is informative and serves only to a better overview. The reference for the concrete reference measurement channels and corresponding implementation’s parameters as to be used for testing are annexes A.4.2 and A.4.3 as appropriate.

Table A.4.1.3-1: Overview of UL reference measurement channels

| Duplex | Table | Name |  | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD, Full RB allocation, QPSK |  |  |  |  |  |  |  |  |  |  |
| FDD / HD-FDD |  | Table A.4.2.1.1-1 |  |  | 1.4 | QPSK | 1/3 | 6 |  | M1 |  |
| FDD, Full RB allocation, 16-QAM |  |  |  |  |  |  |  |  |  |  |
| FDD / HD-FDD |  | Table A.4.2.1.2-1 |  |  | 1.4 | 16QAM | 1/3 | 6 |  | M1 |  |
| FDD, Partial RB allocation, QPSK |  |  |  |  |  |  |  |  |  |  |
| FDD / HD-FDD |  | Table A.4.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 1 |  | M1 |  |
| FDD / HD-FDD |  | Table A.4.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 2 |  | M1 |  |
| FDD / HD-FDD |  | Table A.4.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 3 |  | M1 |  |
| FDD / HD-FDD |  | Table A.4.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 4 |  | M1 |  |
| FDD / HD-FDD |  | Table A.4.2.2.1-1 |  | 1.4 |  | QPSK | 1/3 | 5 |  | M1 |  |
| FDD, Partial RB allocation, 16-QAM |  |  |  |  |  |  |  |  |  |  |
| FDD / HD-FDD |  | Table A.4.2.2.2-1 |  | 1.4 |  | 16QAM | 1/2 | 1 |  | M1 |  |
| FDD / HD-FDD |  | Table A.4.2.2.2-1 |  | 1.4 |  | 16QAM | 1/2 | 2 |  | M1 |  |
| FDD / HD-FDD |  | Table A.4.2.2.2-1 |  | 1.4 |  | 16QAM | 1/2 | 3 |  | M1 |  |
| FDD / HD-FDD |  | Table A.4.2.2.2-1 |  | 1.4 |  | 16QAM | 2/5 | 4 |  | M1 |  |
| FDD, SubPRB allocation |  |  |  |  |  |  |  |  |  |  |  |
| FDD / HD-FDD |  | Table A.4.2.3-1 |  | 1.4 |  | π/2 BPSK | 1/3 | 1 |  | M1 | 2 out of 3 subcarriers |
| FDD / HD-FDD |  | Table A.4.2.3-1 |  | 1.4 |  | QPSK | 1/3 | 1 |  | M1 | 3 subcarriers |
| FDD / HD-FDD |  | Table A.4.2.3-1 |  | 1.4 |  | QPSK | 1/3 | 1 |  | M1 | 6 subcarriers |

Table A.4.1.3-2: Overview of UL reference measurement channels (HD-FDD, NB-IoT)

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HD-FDD | Table A.4.3-1 |  | 0.2 | π/2 BPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.4.3-1 |  | 0.2 | π/4 QPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.4.3-1 |  | 0.2 | π/2 BPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.4.3-1 |  | 0.2 | π/4 QPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.4.3-1 |  | 0.2 | QPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.4.3-1 |  | 0.2 | QPSK | 1/3 | 1 |  | NB1 |  |
| HD-FDD | Table A.4.3-1 |  | 0.2 | QPSK | 1/3 | 1 |  | NB1 |  |

## A.4.2	Reference measurement channels for FDD

### A.4.2.1	Full RB allocation

#### A.4.2.1.1	QPSK

Table A.4.2.1.1-1: Reference Channels for QPSK with full/maximum RB allocation for UE category M1

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | QPSK |
| Target Coding rate |  | 1/3 |
| Payload size | Bits | 600 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame |  | 1 |
| Total number of bits per Sub-Frame | Bits | 1728 |
| Total symbols per Sub-Frame |  | 864 |
| UE Category |  | M1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th and 6th subframes every 10ms for the channel bandwidth 5MHz/10MHz/15MHz/20MHz. For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. is total number of absolute subframes a PUSCH with repetition spans [4]. NOTE 3: For HD-FDD UE with , MPDCCH are scheduled at 0th DL subframe every +5 subframes (starting from the 0th subframe). The associated PUSCH is scheduled at the 4th to (+3)-th UL subframes every +5 subframes. Information bit payload is available if uplink subframe is scheduled. |  |  |

#### A.4.2.1.2	16-QAM

Table A.4.2.1.2-1: Reference Channels for 16-QAM with maximum RB allocation for UE category M1

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 1.4 |
| Allocated resource blocks |  | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 |
| Modulation |  | 16QAM |
| Target Coding rate |  | 1/3 |
| Payload size | Bits | 872 |
| Transport block CRC | Bits | 24 |
| Number of code blocks per Sub-Frame |  | 1 |
| Total number of bits per Sub-Frame | Bits | 2880 |
| Total symbols per Sub-Frame |  | 720 |
| UE Category |  | M1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |

### A.4.2.2	Partial RB allocation

For each channel bandwidth, various partial RB allocations are specified. The number of allocated RBs is chosen according to values specified in the Tx and Rx requirements. The single allocated RB case is included.

The allocated RBs are contiguous and start from one end of the channel bandwidth. A single allocated RB is at one end of the channel bandwidth.

#### A.4.2.2.1	QPSK

Table A.4.2.2.1-1: Reference Channels for QK with partial RB allocation for UE category M1

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Trans-port block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 | 1 | 12 | QPSK | 1/3 | 72 | 24 | 1 | 288 | 144 | M1 |
|  | 1.4 | 2 | 12 | QPSK | 1/3 | 176 | 24 | 1 | 576 | 288 | M1 |
|  | 1.4 | 3 | 12 | QPSK | 1/3 | 256 | 24 | 1 | 864 | 432 | M1 |
|  | 1.4 | 4 | 12 | QPSK | 1/3 | 392 | 24 | 1 | 1152 | 576 | M1 |
|  | 1.4 | 5 | 12 | QPSK | 1/3 | 424 | 24 | 1 | 1440 | 720 | M1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

#### A.4.2.2.2	16-QAM

Table A.4.2.2.2-1: Reference Channels for 16-QAM with partial RB allocation for UE category M1

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 | 1 | 12 | 16QAM | 1/2 | 256 | 24 | 1 | 576 | 144 | M1 |
|  | 1.4 | 2 | 12 | 16QAM | 1/2 | 552 | 24 | 1 | 1152 | 288 | M1 |
|  | 1.4 | 3 | 12 | 16QAM | 1/2 | 840 | 24 | 1 | 1728 | 432 | M1 |
|  | 1.4 | 4 | 12 | 16QAM | 2/5 | 904 | 24 | 1 | 2304 | 576 | M1 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 2: For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10ms for the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

### A.4.2.3	subPRB allocation

The location of allocated RB for subPRB allocation is chosen according to values specified in the Tx requirements.

Table A.4.2.3-1: Reference Channels for SubPRB allocation

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 1.4 | 1.4 |
| Allocated resource blocks |  | 1 | 1 | 1 |
| Number of subcarriers |  | 2 out of 3 | 3 | 6 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 |
| Modulation |  | π/2 BPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 |
| Payload size | Bits | 32 | 72 | 72 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| Number of code blocks |  | 1 | 1 | 1 |
| Total number of bits per resource unit | Bits | 192 | 288 | 288 |
| Total symbols per resource unit |  | 192 | 144 | 144 |
| Tx time | ms | 8 | 4 | 2 |
| UE UL Category |  | M1 | M1 | M1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |

## A.4.3	Reference measurement channels for category NB1

Table A.4.3-1 Reference Channels for category NB1

| Parameter | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 3.75 | 3.75 | 15 | 15 | 15 | 15 | 15 |
| Number of tone | 1 | 1 | 1 | 1 | 3 | 6 | 12 |
| Modulation | π/2 BPSK | π/4 QPSK | π/2 BPSK | π/4 QPSK | QPSK | QPSK | QPSK |
| Number of NPUSCH repetition (NOTE 5) | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| IMCS / ITBS | 0 / 0 | 3 / 3 | 0 / 0 | 3 / 3 | 5 / 5 | 5 / 5 | 5 / 5 |
| Payload size (bits) | 32 | 40 | 32 | 40 | 72 | 72 | 72 |
| Allocated resource unit | 2 | 1 | 2 | 1 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.33 | 0.29 | 0.33 | 0.33 | 0.33 | 0.33 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 192 | 96 | 192 | 288 | 288 | 288 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 144 | 144 | 144 |
| Tx time (ms) | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: Parameters related to NPUSCH format 1 scheduling are defined in Table A.4.3-2. NOTE 3: NPDCCH is not transmitted in the subframes used for transmission of SI messages. NOTE 4: SI messages transmission should be prioritized over NPDCCH transmission in case of collision. NPDCCH transmission is postponed until the next NB-IoT downlink subframe in case NPDCCH transmission occurs in a non NB-IoT downlink subframe, where an NB-IoT downlink subframe is a subframe that does not contain NPSS/NSSS/NPBCH/SIB1-NB transmission. NOTE 5: Number of repetition NRep as defined in table 16.5.1.1-3 in TS 36.213 [12]. |  |  |  |  |  |  |  |

Table A.4.3-2: NPDCCH configuration for NPUSCH format 1 scheduling

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N0 |
| NPDCCH format |  | 1 |
| Scheduling delay () |  | 0 |
| DCI subframe repetition number |  | 00 |
| (npdcch-NumRepetitions) |  | 1 |
| G (NPDCCH-startSF-USS) |  | 8 |
| (npdcch-Offset-USS) |  | 1/4 |

Annex B (normative): 
Downlink physical channels

## <<< END OF CHANGES 2>>>