3GPP TSG-RAN WG4 Meeting # 110	R4-2403289

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on carrier phase positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The reasons for changes in this CR are summarized as below: - RAN1 agreed to support Nsample = 1,2,4 for carrier phase measurements. 4 samples for RSCPD measurement in clasue 9.9.7.5 is missing in the latest spec. - For the calculation of Kp,PRS, only PRS periodicity and MGRP are considered in the existing requirements. However for carrier phase measurements, periodic time window(s) may be configured and UE is required to measure PRS witin the time window(s) only. In this case, such the time window(s) should also taken into account. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes in this CR includes: - Add 4 samples for RSCPD measurments in clasue 9.9.7.5. - For the calculation of Kp,PRS, take the CPP time window(s) into account if configured. - Correction of some signallings and clause numbers |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for RSCPD and RSCP measurment will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.7, 9.9.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------ START OF CHANGE 1--------------

<unchanged sections omitted>

9.9.7	Measurement requirements for DL RSCPD reported with RSTD

9.9.7.1	Introduction

The requirements in clause 9.9.7 shall apply provided the UE has received NR-DL-TDOA-RequestLocationInformation message with nr-DL-PRS-RSCPD-Request from LMF via LPP [34] requesting the UE to measure and report RSCPD measurement together with DL RSTD measurements defined in TS 38.215 [4].

9.9.7.2	Requirements Applicability

The requirements in clause 9.9.7 apply for periodic and triggered reporting of RSCPD with RSTD measurements, provided:

-	PRS-RSTD related side conditions given in clause 10.1.23 for FR1 and FR2 are fulfilled, for a corresponding Band.

-	RSCPD related side conditions given in clause 10.1.x for FR1 and FR2 are fulfilled, for a corresponding Band.

<unchanged sections omitted>

9.9.7.5	Measurements Period Requirements for DL RSCPD reported with RSTD

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation  message with nr-DL-RSCPD-Request from the LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.7.3) DL RSTD and RSCPD measurements, defined in TS 38.215 [4], during the time window configured to UE via nr-DL-PRS-MeasurementTimeWindowsConfig but the time window periodicity is not configured, and the start of the measurement period is the start of the window.

Otherwise, the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.7.3) RSTD and DL RSCPD measurements, defined in TS 38.215 [4], during the measurement period TRSCPD with RSTD,Total defined as:

TRSCPD with RSTD,Total=i=1LTRSCPD with RSTD,i+ L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSCPD with RSTD,i is the measurement period for PRS RSTD with RSCPD measurement in positioning frequency layer i as specified below:TRSCPD with RSTD

TRSCPD with RSTD=kmultiTEG×CSSFPRS×ceil( Kp,PRS)×NRxBeam×NPRS,islotN'Lavailable_PRS,iN×Nsample-1*Teffect+Tlast,

where:

- DL-RSCPD measurement performed during TRSCPD with RSTD is a single sample measurement. The DL-RSCPD measurement of reference TRP and the target TRP is performed on the same PFL.

- NRxBeam, CSSFPRS, kmultiTEG, Kp,PRS NPRS,islot, N, N’, Teffect and Tlast are defined in clause 9.9.2.5.

- Nsample = 1 or 2 or 4 as defined in clause 9.9.2.5.

- Lavailable_PRS is the time duration of available PRS in the positioning frequency layer to be measured during Tavailable_PRS, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26].

- When periodic time window(s) are configured by the LMF, Tavailable_PRS=LCM(Tprs, MGRP, Twindow), otherwise  Tavailable_PRS=LCM(Tprs, MGRP). Twindow is the maximum periodicity of the configured time window(s). Tprs and MGRP are defined in clause 9.9.2.5.

- Lavailable_PRS and Tprs are calculated by only considering the PRS resources in the indicated resources sets overlapping with both the MG and the indicated time window(s).

- Kp,PRS is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS = 1 for UE not configured with concurrent measurement gap.

- When periodic time window(s) are configured by the LMF, for a window W of duration max(TPRS,  MGRP_max, Twindow), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion and the periodic time window:

  - Ntotal is the total number of associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window W, and

  - Navailable is the number of non-dropped associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, after further accounting for MG collisions by applying the selected gap collision rule

  - Requirements do not apply if Navailable =0.

- Otherwise, for a window W of duration max(TPRS,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

  - Ntotal is the total number of associated gap occasions covering PRS occasions within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window, and

  - Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TRSCPD with RSTD starts from the first time window (TRSCPD) configured by LMF within MG instance aligned with a DL PRS resource(s) in the assistance data after both the NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34].

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TRSCPD with RSTD starts from the first time window (TRSCPD) configured by LMF within MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the RSCPD with RSTD measurement in each reporting period within the time window(TRSCPD) configured by LMF within MG instance and activate the location report at the time when the periodic timer expires.

If during the measurement period, the MG pattern is reconfigured or time window (TRSCPD) for RSCPD measurement is reconfigured, the measurement period can be longer. When PRS-RSRP is also configured to UE, RSCPD with RSTD and RSRP are performed over the same measurement period.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TRSCPD with RSTD within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 9.9.7 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while RSTD and RSCPD measurements are being performed together by UE, then the UE shall continue and complete the on-going RSTD and RSCPD measurements. The measurement period for RSCPD with RSTD measurement can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23. The UE shall also meet the RSCPD measurement accuracy requirements in clause 10.1.x.

-------------END OF CHANGE 1--------------

------------ START OF CHANGE 2--------------

<unchanged sections omitted>

9.9.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference

9.9.8.1	Introduction

The requirements in this clause shall apply, provided the UE has received nr-Multi-RTT-RequestLocationInformation message with nr-UE-RSCP-Request from LMF via LPP [34] requesting the UE to measure and report one or more DL RSCP measurements together with UE Rx-Tx time difference measurements defined in TS 38.215 [4].

9.9.8.2	Requirements Applicability

The requirements in clause 9.9.8 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25 are met for a corresponding band.

- 	DL RSCP related side conditions given in clause 10.1.x for FR1 and FR2 are fulfilled, for a corresponding Band.

-	SRS is configured on at least one of the PCell, PSCell and SCell.

-	The UE transmits SRS within [-160, 160] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

<unchanged sections omitted>

9.9.8.5	Measurement Period Requirements for DL RSCP and UE Rx-Tx time difference

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message with nr-UE-RSCP-Request from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.8.3) UE Rx-Tx and RSCP measurements, defined in TS 38.215 [4], during the time window configured to UE via nr-DL-PRS-MeasurementTimeWindowsConfig but the time window periodicity is not configured, and the start of the measurement period is the start of the window.

Otherwise, the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.8.3) UE Rx-Tx and DL RSCP measurements, defined in TS 38.215 [4], during the measurement period TRSCP with UERxTx,Total defined as:

TRSCP with UERxTx,Total=i=1LTDL RSCP with UERxTx,i+ L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i

TDL RSCP with UERxTx,i is the measurement period for RSCP with UE Rx-Tx measurement in positioning frequency layer i as specified below:

TDL RSCP with UERxTx

TDL RSCP with UERxTx=kmultiTEG*CSSFPRS*ceil( Kp,PRS)*NRxBeam*NPRSslotN'Lavailable_PRSN*Nsample-1*Teffect+Tlast ,

where:

- DL-RSCP performed during TDL RSCP with UERxTx is a single sample measurement where DL-RSCP and UE Rx-Tx measurements are performed on the same PFL.

- NRxBeam, CSSFPRS, kmultiTEG, Nsample, NPRS,islot, N, N’, Teffect and Tlast are defined in clause 9.9.4.5.

- Nsample = 1 or 2 or 4 as defined in clause 9.9.4.5.

- CSSFPRS

- kmultiTEG

kmultiTEG

kmultiTEGNTEG

kmultiTEGNTEGkTEG,simul

NTEGNTEG

kTEG,simul

-	Kp,PRS is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS = 1 for UE not configured with concurrent measurement gap.

- When periodic time window(s) are configured by the LMF, for a window W of duration max(TPRS,  MGRP_max, Twindow), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion and the periodic time window:

Ntotal is the total number of associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window W, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

- Otherwise, ror a window W of duration max(TPRS,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window W, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

NRxBeam NRxBeamNRxBeamNRxBeam

-	Lavailable_PRS is the time duration of available PRS resources in the positioning frequency layer, to be measured during Tavailable_PRS, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. Lavailable_PRS

- When periodic time window(s) are configured by the LMF, Tavailable_PRS=LCM(Tprs, MGRP, Twindow), otherwise  Tavailable_PRS=LCM(Tprs, MGRP). Twindow is the maximum periodicity of the configured time window(s). Tprs and MGRP are defined in clause 9.9.8.5.

- Lavailable_PRS and Tprs are calculated by only considering the PRS resources in the indicated resources sets overlapping with both the MG and the indicated time window(s).

NPRSslot

{N,T}

N’

Nsample

Nsample

Nsample

Nsample

Nsample

TlastTlastTTavailable_PRS

Teffect

Teffect= TTavailable_PRS*Tavailable_PRS

T

Tavailable_PRS= LCMTPRS,MGRP, TwindowTPRSMGRP

Lavailable_PRSTPRS

Tavailable_PRS= LCMTPRS, MGRPTPRS

MGRP

TPRSTperPRS with muting=Nmuting*TperPRSTperPRS with mutingTPRS

TperPRS

NmutingNmuting=TmutingPRS*LmutingTmutingPRSLmutingb1

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TDL RSCP with UERxTx starts from the first time window (TDL RSCP) configured by LMF within MG instance aligned with DL PRS resource(s) in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TDL RSCP with UERxTx starts from the first time window (TDL RSCP) configured by LMF within MG instance aligned with DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the DL RSCP with UE Rx-Tx time difference measurement in each reporting period and activate the location report at the time when the periodic timer expires.

The DL RSCP with UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of the positioning frequency layer, the MG pattern is reconfigured either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TDL RSCP with UERxTx within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 9.9.8 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

When PSCell or SCell addition or release does not cause SRS reconfiguration during the measurement period, UE shall continue and complete the DL RSCP and UE Rx-Tx time difference measurements, and the measurement period requirements apply.

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the DL RSCP and UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

When SRS is reconfigured without serving cell change during the measurement period, UE shall restart the DL RSCP and UE Rx-Tx time difference measurement after the SRS reconfiguration is complete.

When a serving cell change occurs during the measurement period, the UE shall restart DL RSCP and UE Rx-Tx time difference measurements.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the measurement period, then the DL RSCP and UE Rx-Tx time difference measurement is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the measurement period, then the DL RSCP and UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the measurement period, then:

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The UE Rx-Tx time difference measurement period may be restarted in such case.

If UE uplink transmission timing changes due to the network-configured Timing Advance command or due to the change in the NTA_offset defined in Table 7.1.2-2 or due to the UE autonomous timing adjustment defined in clause 7.1.2 during the DL RSCP with UE Rx-Tx measurement period, the UE may continue and complete the DL RSCP with UE Rx-Tx measurement.

-------------END OF CHANGE 2--------------