3GPP TSG-RAN4 Meeting #110	R4-2403346

# Athens, Greece,  February 26 – Mars 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4202 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_SL_enh2-Core) Corrections to SL unlicensed requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR contains corrections sidelink unlicensed requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | The current section for sidelink requirements contains several editorial types of errors including requirements within squarebrackets which are corrected in this CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uncertainties in the requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1, 12.2, 12.3, 12.3A, 12.4A, 12.7.9, 12.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------- Start of Change <1> --------------

## 12.1	Introduction

This clause contains the requirements for the UE capable of V2X sidelink communication when the UE is out of coverage on the carrier used for V2X sidelink operation, as defined in TS 38.304 [1]. The requirements apply when the UE is:

-	in any cell selection state, or,

-	configured for V2X SL operation on a V2X carrier which is dedicated to only V2X SL operation and configured with only a PCell on WAN carrier.

-	configured for SL operation on a sidelink carrier with CCA and configured with only a PCell on WAN carrier.

-	configured for inter-band con-current V2X operation.

-	configured for intra-band con-current V2X operation with different carriers.

-	configured in co-channel coexistence for LTE SL and NR SL operation with same carrier.

Note:	Any cell selection state refers to a UE that is out of network coverage and is not associated with a serving cell on any carrier as defined in TS 38.304 [1].

Note:	When a UE in RRC_CONNECTED state is performing transmissions and/or reception for V2X sidelink communication, the UE shall meet all the requirements specified in Clause 9 assuming that UE has a dedicated RX/TX chain for V2X sidelink communication. Otherwise, the UE may interrup the V2X sidelink communication in order to meet the measurement requirements specified in Clause 9.

This clause also contains the requirements for the UE capable of V2X sidelink communication when the UE is in coverage on the carrier used for V2X sidelink operation, as defined in TS 38.304 [1]. The requirements apply when the UE is:

-	configured for intra-band con-current NR V2X cooperation with same carrier.

-	configured in co-channel coexistence for LTE SL and NR SL operation with same carrier，

For UE capable of Public Safety sidelink communication and/or other commercial sidelink commnunication, unless explicitly stated, V2X requirements apply.

For sidelink communication in unlicensed spectrum,

-	the term SyncRef UE subject to CCA is not available at the UE refers to when all the candidate S-SSB positions monitored in every S-SSB period are not available during the last 1280 ms. Otherwise, the SyncRef UE subject to CCA is considered as available at the UE.

-	the term S-SSB period subject to CCA is not available at the UE refers to the S-SSB period in which all the candidate S-SSB positions are not available. Otherwise the S-SSB period subject to CCA is considered as available at the UE.

-------------- End of Change <1> --------------

-------------- Start of Change <2> --------------

## 12.2	UE Transmit Timing

### 12.2.1	Introduction

This clause contains requirements of transmission timing for V2X sidelink communication when:

-	GNSS is used as the synchronization reference source;

-	NR Cell is used as the synchronization reference source;

-	E-UTRAN Cell is used as the synchronization reference source;

-	SyncRef UE is used as the synchronization reference source.

The requirements for 60 kHz SCS of sidelink signal defined in this caluse do not apply to the sidelink communication in unlicensed spectrum.

### 12.2.2	GNSS as synchronization reference source

The requirements in this subclause are applicable when the reference timing used by the UE for V2X sidelink communication is derived from GNSS.

The sidelink transmissions takes place  before the subframe starting boundary as defined in TS 38.331 [2], where  = 0 and=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.2-1.

Table 12.2.2-1: Te Timing Error Limit

| Frequency Range of sidelink | Te_ |
| --- | --- |
| FR1 | 12*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |

### 12.2.3	NR Cell as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for sidelink transmissions is a NR serving cell on a non-V2X sidelink carrier or a V2X sidelink carrier.

The sidelink transmissions takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell, where  = 0. If uplink transmission and sidelink transmission are in the same band,  is defined in Table 7.1.2-2, otherwise  is 0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.3-1.

Table 12.2.3-1: Te Timing Error Limit

| Frequency Range of sidelink | SCS of SSB signals ( kHz) | SCS of sidelink signals (kHz) | Te |
| --- | --- | --- | --- |
| FR1 | 15 | 15 | 14*64*Tc |
|  |  | 30 | 12*64*Tc |
|  |  | 60 | 12*64*Tc |
|  | 30 | 15 | 10*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 9*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |  |

### 12.2.4	E-URTAN Cell as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for sidelink transmissions is an E-UTRAN serving cell on a non-V2X sidelink carrier.

The sidelink transmissions takes place  before the reception of the first detected path (in time) of the corresponding E-UTRAN downlink frame from the reference cell, where  = 0 and=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.4-1.

Table 12.2.4-1: Te Timing Error Limit

| Frequency Range of sidelink | E-UTRAN downlink bandwidth (MHz) | Te_ |
| --- | --- | --- |
| FR1 | ≥3 | 14*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |

### 12.2.5	SyncRef UE as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for deriving sidelink transmission is from SyncRef UE transmitting sidelink synchronization signals.

The sidelink transmissions takes place  before the reception of the first detected path (in time) of the corresponding timing reference frame from the SyncRef UE, where  = 0 and=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.5-1.

Table 12.2.5-1: Te Timing Error Limit

| Frequency Range of sidelink | SCS of sidelink signals (kHz) | Te |
| --- | --- | --- |
| FR1 | 15 | 12*64*Tc |
|  | 30 | 8*64*Tc |
|  | 60 | 5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |

If the UE uses SyncRefUE on a carrier frequency subject to CCA for deriving the UE transmit timing, then the UE shall meet all the transmit timing requirements defined in clause 12.2.5 provided that the SyncRefUE is available at the UE. The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.5-1 provided that at least one S-SSB is available at the UE durting the last 160 ms.

-------------- End of Change <2> --------------

-------------- Start of Change <3> --------------

## 12.3	Initiation/Cease of SLSS Transmissions

### 12.3.1	Introduction

The requirements in this subclause are applicable to the UE capable of V2X sidelink communication when:

-	GNSS is used as the synchronization reference source;

-	NR Cell is used as the synchronization reference source;

-	EUTRAN Cell is used as the synchronization reference source;

-	SyncRef UE is used as the synchronization reference source.

#### 12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source

The requirements apply when the NR Cell is used as synchronization reference source and when the UE is

-	out of coverage on the V2X NR sidelink carrier and in coverage with a serving cell on a NR non-V2X sidelink carrier, or

-	 in coverage with a serving cell on a NR V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 38.331[2] are met; networkControlledSyncTx is not configured; and syncTxThreshIC is included in SystemInformationBlockType12. The UE shall be capable of measuring the RSRP of the cell used as synchronization reference source to evaluate to initiate/cease SLSS transmissions within Tevaluate,SLSS

where,

-	Tevaluate,SLSS is as specified in Table 12.3.1.1-1 when UE performs SSB based measurements without measurement gaps.

-	Tevaluate,SLSS is as specified in Table 12.3.1.1-2 when UE performs SSB based measurements with measurement gaps.

Table 12.3.1.1-1: Tevaluate,SLSS for measurements without measurement gaps when NR cell is used as synchronization reference source (FR1)

| DRX cycle in NR cell | Tevaluate,SLSS |
| --- | --- |
| No DRX | max(400ms, ceil( 2 x 5 x Kp) x SMTC period)Note 1 |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5 x 2 x 5 x Kp) x max(SMTC period, DRX cycle)) |
| DRX cycle>320ms | ceil( 7 x Kp ) x DRX cycle |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 12.3.1.1-2: Tevaluate,SLSS for measurements with measurement gaps when NR cell is used as synchronization reference source (FR1)

| DRX cycle in NR cell | Tevaluate,SLSS |
| --- | --- |
| No DRX | max(400ms, 2 x 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(2 x 1.5x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x CSSFintra |

If higher layer filtering is configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the NR cell as synchronization reference source:

-	SS-RSRP related side conditions given in clauses 10.1.2 for FR1, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.7 for FR1, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.12 for FR1, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

#### 12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source

The requirements apply when the EUTRAN Cell is used as synchronization reference source and when the UE is

-	out of coverage on the V2X NR sidelink carrier and in coverage with a serving cell on a LTE non-V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 36.331[16] are met; networkControlledSyncTx is not configured; and syncTxThreshIC is included in SystemInformationBlockType28. The UE shall be capable of measuring the RSRP of the cell used as synchronization reference source to evaluate to initiate/cease SLSS transmissions within Tevaluate,SLSS

where,

-	Tevaluate,SLSS = 0.4 seconds when UE is not configured with DRX.

-	Tevaluate,SLSS = as specified in Table 12.3.1.2-1 when UE is configured with DRX.

Table 12.3.1.2-1: Tevaluate,SLSS when EUTRAN cell is used as synchronization reference source

| DRX cycle length in EUTRAN cell[s] | Tevaluate,SLSS [s] (number of DRX cycles) |
| --- | --- |
| ≤0.04 | 0.4 (Note 1) |
| 0.04<DRX-cycle≤2.56 | Note 2 (6) |
| Note1: Number of DRX cycles depends upon the DRX cycle in use Note2: Time depends upon the DRX cycles in use |  |

If higher layer filtering is configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the cell as synchronization reference source:

-	RSRP related side conditions given in TS 36.133[15] Clauses 9.1.2.1 and 9.1.2.2 and RSRQ related side conditions given in TS 36.133[15] Clause 9.1.5.1 for a corresponding Band are fulfilled,

-	SCH_RP and SCH Ês/Iot according to TS 36.133[15] Annex B.2.1 for a corresponding Band are fulfilled.

#### 12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source

The requirements apply when GNSS is used as synchronization reference source and when the UE is

-	out of coverage on the V2X sidelink carrier and in coverage with a serving cell on a non-V2X sidelink carrier, or

-	in coverage with a serving cell on a NR V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 38.331[2] are met; networkControlledSyncTx is not configured; and syncTxThreshIC is included in SystemInformationBlockType12 in a NR cell.

When the conditions for SLSS transmissions specified in TS 36.331[16] are met; networkControlledSyncTx is not configured; and syncTxThreshIC is included in SystemInformationBlockType28 in a EUTRAN cell.

The requirements in Clause 12.3.1.1 shall apply if the serving cell is a NR cell.

The requirements in Clause 12.3.1.2 shall apply if the serving cell is a EUTRAN cell.

#### 12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source

The requirements apply when SyncRef UE is used as synchronization reference source and when the UE is

-	in any cell selection state, or

-	out of coverage on the V2X sidelink carrier and is associated with a serving cell on a non-V2X sidelink carrier, or

-	in coverage with a serving cell on a NR V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 38.331[2] are met and when SyncRef UE is used as synchronization reference source and if syncTxThreshOoC is included in the preconfigured V2X parameters.

The UE shall be capable of measuring the PSBCH-RSRP of the selected SyncRef UE used as synchronization reference source and evaluate it to initiate/cease SLSS transmissions within Tevaluate,SLSS, as shown in Table 12.3.1.4-1.

Table 12.3.1.4-1: Tevaluate,SLSS when SyncRef UE is used as synchronization reference source

| SL-DRX cycleNote 1 [ms] | Tevaluate,SLSS [ms] |
| --- | --- |
| No SL-DRX | 4 x S-SSB periods |
| SL-DRX cycle ≤ 160 | 4 x S-SSB periods |
| SL-DRX cycle > 160 | 4 x SL-DRX cycle |
| Note 1: If multiple SL-DRX cycles are configured for SL UE, the SL-DRX cycle in the requirement is the shortest one. When the shortest SL-DRX cycle UE used changes, the requirements do not apply to the time of transition. |  |

If higher layer filtering for PSBCH-RSRP measurements is pre-configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the selected SyncRef UE as defined in TS 38.331 [2] used to derive transmission timing for V2X sidelink communication:

-	PSBCH-RSRP related side conditions given in Clause 12.4 for a corresponding Band are fulfilled,

-	V2X S-SSB_RP and S-SSB Ês/Iot according to Annex B. 4 for a corresponding Band are fulfilled.

## 12.3A	Initiation/Cease of SLSS Transmissions with CCA

### 12.3A.1	Introduction

The requirements in this subclause are applicable to the UE capable of sidelink communication in unlicensed spectrum when:

-	GNSS is used as the synchronization reference source;

-	NR Cell is used as the synchronization reference source;

-	EUTRAN Cell is used as the synchronization reference source;

-	SyncRef UE is used as the synchronization reference source on a carrier frequency subject to CCA.

#### 12.3A.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source

The requirements defined in subclause 12.3.1.1 apply when the NR Cell is used as synchronization reference source and when the UE is

-	out of coverage on the NR sidelink carrier and in coverage with a serving cell on a NR non- sidelink carrier, or

-	in coverage with a serving cell on a NR sidelink carrier.

#### 12.3A.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source

The requirements defined in subclause 12.3.1.2 apply when the EUTRAN Cell is used as synchronization reference source and when the UE is

-	out of coverage on the NR sidelink carrier and in coverage with a serving cell on a LTE non-sidelink carrier.

#### 12.3A.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source

The requirements defined in subclause 12.3.1.3 apply when GNSS is used as synchronization reference source and when the UE is

-	out of coverage on the sidelink carrier and in coverage with a serving cell on a non-sidelink carrier, or

-	in coverage with a serving cell on a NR sidelink carrier.

#### 12.3A.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source

The requirements apply when SyncRef UE is used as synchronization reference source and when the UE is

-	in any cell selection state, or

-	out of coverage on the sidelink carrier and is associated with a serving cell on a non-sidelink carrier, or

-	in coverage with a serving cell on a NR sidelink carrier,

and when the conditions for SLSS transmissions specified in TS 38.331[2] are met and when SyncRef UE is used as synchronization reference source and if syncTxThreshOoC is included in the preconfigured sidelink parameters.

The UE shall be capable of measuring the PSBCH-RSRP of the selected SyncRef UE used as synchronization reference source and evaluate it to initiate/cease SLSS transmissions within Tevaluate,SLSS_CCA, as shown in Table 12.3A.1.4-1 when the SyncRef UE is transmitting S-SSB on a carrier frequency subject to CCA.

Table 12.3A.1.4-1: Tevaluate,SLSS_CCA when SyncRef UE is transmitting S-SSB on a carrier subject to CCA and is used as synchronization reference source

| SL-DRX cycleNote 1 [ms] | Tevaluate,SLSS_CCA [ms] |
| --- | --- |
| No SL-DRX | (4 + LSLSS) x S-SSB periods |
| SL-DRX cycle ≤ 160 | (4 + LSLSS) x S-SSB periods |
| SL-DRX cycle > 160 | (4 + LSLSS) x SL-DRX cycle |
| Note 1: If multiple SL-DRX cycles are configured for SL UE, the SL-DRX cycle in the requirement is the shortest of all the configured SL-DRX cycles. When the shortest SL-DRX cycle UE used changes, the requirements do not apply to the time of transition. Note 2: LSLSS is the number of unavailable S-SSB period during Tevaluate,SLSS_CCA due to the CCA failures; where LSLSS ≤ LSLSS, max and LSLSS, max = 4. |  |

The UE shall initiate the procedure for selection/reselection of different synchronization reference source as defined in TS38.331 [2] when the requirements cannot be met due to that LSLSS exceeding LSLSS, max during Tevaluate,SLSS_CCA.

If higher layer filtering for PSBCH-RSRP measurements is pre-configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the selected SyncRef UE as defined in TS 38.331 [2] used to derive transmission timing for sidelink communication:

-	PSBCH-RSRP related side conditions given in Clause 12.4 for a corresponding Band are fulfilled,

-	sidelink S-SSB_RP and S-SSB Ês/Iot according to Annex B. 4 for a corresponding Band are fulfilled.

-------------- End of Change <3> --------------

-------------- Start of Change <4> --------------

### 12.7.9	Interruptions to WAN due to sidelink carrier addition/release

This sub-clause contains the requirements related to the interruptions on the PCell/serving cell due to sidelink component carrier addition/release. It is applicable for UE is in sidelink resource allocation mode 2.

A UE capable of V2X sidelink communication may indicate its interest (initiation or termination) in V2X sidelink communication to the connected gNodeB using IE SidelinkUEInformationNR in TS38.331[2].

The UE is allowed an interruption of up to the duration shown in table 12.7.9-1 on the PCell/serving cell when any number of sidelink component carriers is added or released. This interruption is for both uplink and downlink of the PCell/serving cell.

Table 12.7.9-1: Interruption length due to sidelink component carrier addition/release

|  | NR Slot length (ms) | Interruption length (number of slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |
| 3 | 0.125 | 9 |

For sidelink component carrier addition/release, when the UE is in RRC_IDLE/RRC_INACTIVE mode, the interruptions in this clause shall not apply when one of the following conditions is met:

-	While the UE is receiving paging,

-	While the UE is receiving system information.

-------------- End of Change <4> --------------

-------------- Start of Change <5> --------------

## 12.12	Selection / Reselection of Synchronization Reference Source for NR SL Carrier Aggregation

Requirements in this clause are applicable to UE supporting NR sidelink carrier aggregation.

When the UE is synchronized to a SyncRef UE in a carrier and required only to search other SyncRef UEs in the synchronized carrier, the UE shall be able to identify a newly detectable NR SL SyncRef UE within Tdetect,SyncRef UE_V2X if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331 [2]. The UE shall be capable of performing PSBCH-RSRP measurements for 3 identified NR SL SyncRef UE with the measurement period of Tmeasure,PSBCH-RSRP in Table 12.4-1.

When the synchronization reference source for NR sidelink carrier aggregation is lost and UE has to search SyncRef UE on the aggregated carriers which are configured as synchronization carrier, the UE shall be able to identify a newly detectable NR SL SyncRef UE within N×Tdetect,SyncRef UE_V2X if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331 [2]. The UE shall be capable of performing PSBCH-RSRP measurements for 3 identified NR SL SyncRef UE per carrier with the measurement period of N×Tmeasure,PSBCH-RSRP. N is the number of aggregated carriers configured as synchronization carrier.

It is assumed that the identified NR SL SyncRef UE does not drop or delay any SLSS transmission within the measurement period. Otherwise, the measurement period may be extended.

When GNSS synchronization reference source is configured as the highest priority and

-	UE is synchronized to a SyncRef UE that is synchronized to GNSS directly or in-directly,

-	The value of Tdetect,SyncRef UE_V2X is as 1.6 seconds at S-SSB Es/Iot ≥0 dB, provided that the UE is allowed to drop a maximum of 30% of its SLSS transmissions on each carrier operating NR SL sidelink communication during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

-	in other case

-	When UE is in non-SL-DRX

-	The value of Tdetect,SyncRef UE_V2X is as 8 seconds at S-SSB Es/Iot ≥0 dB, provided that the UE is allowed to drop a maximum of 6%  of its SLSS transmissions on each carrier operating SL sidelink communication during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its SL data reception on each carrier operating SL sidelink communication per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its SL data reception during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	When UE is in SL-DRX

-	The value of Tdetect,SyncRef UE_V2X is as 8 seconds at S-SSB Es/Iot ≥0 dB, provided that the UE is allowed to drop a maximum of 6%  of its SLSS transmissions on each carrier operating SL sidelink communication during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its SL data reception on each carrier operating SL sidelink communication per PSBCH monitoring occasion and UE is allowed to drop at most an aggregated window of 24 ms of its SL data reception during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	The UE is allowed to extend Tdetect,SyncRef UE_V2X to max(4×50 SL-DRX cycle length, 8s) when the following conditions are satisfied over an evaluation period Tevaluate,SLSS in clause 12.3.1.1 if an NR cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.2 if an EUTRA cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.4 if an SLSS is used as synchronization reference source. If multiple SL-DRX cycles are configured, the SL-DRX cycle length is the longest one.

-	SS-RSRP is larger than syncTxThreshOoC.

When serving cell/PCell synchronization reference source is configured as the highest priority,

-	When UE is in non-SL-DRX

-	The value of Tdetect,SyncRef UE_V2X is as 8 seconds at S-SSB Es/Iot ≥0 dB, provided that the UE is allowed to drop a maximum of 6%  of its SLSS transmissions on each carrier operating NR sidelink communication during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its SL data reception on each carrier operating NR SL sidelink communication per PSBCH monitoring occasion and overall drop rate shall not exceed 0.3% of its SL data reception during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	When UE is in SL-DRX

-	The value of Tdetect,SyncRef UE_V2X is as 8 seconds at S-SSB Es/Iot ≥0 dB, provided that the UE is allowed to drop a maximum of 6%  of its SLSS transmissions on each carrier operating NR SL sidelink communication during Tdetect,SyncRef UE_V2X at most in an aggregated window of 480 ms during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	UE is allowed to drop up to 2 slots of its SL data reception per PSBCH monitoring occasion and UE is allowed to drop at most an aggregated window of 24 ms of its SL data reception during Tdetect,SyncRef UE_V2X for the purpose of selection/reselection to the SyncRef UE.

-	The UE is allowed to extend Tdetect,SyncRef UE_V2X to max(4×50 SL-DRX cycle length, 8s) when the following conditions are satisfied over an evaluation period Tevaluate,SLSS in clause 12.3.1.1 if an NR cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.2 if an EUTRA cell is used as synchronization reference source, or Tevaluate,SLSS in clause 12.3.1.4 if an SLSS is used as synchronization reference source. If multiple SL-DRX cycles are configured, the SL-DRX cycle length is the longest one.

-	SS-RSRP is larger than syncTxThreshOoC.

-------------- End of Change <5> --------------