3GPP TSG-RAN4 Meeting #110		R4-2403353

Athens, Greece, Feb 26 – Mar 01, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on correction for L3 measurement requirements due to R18 MUSIM gaps |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DualTxRx_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2024-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Correction on L3 measurement requirements impacted by MUSIM gaps, which include correcting errors, missing corresponding parts for MUSIM and correcting the references to other clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections on eight clauses with similar changes for correcting errors |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccurate L3 requirements due to MUSIM gaps |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5, 9.2.6, 9.3.4, 9.3.9, 9.4.2, 9.4.3, 9.10.2 and 9.10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2402900 |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE 1----------------------------

### 9.2.5	Intrafrequency measurements without measurement gaps

#### 9.2.5.1	Intrafrequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TPSS/SSS_sync_intra is given in Table 9.2.5.1-11; otherwise, TPSS/SSS_sync_intra is given in Table 9.2.5.1-2.

-	For UE indicating [no gap without interruption], TPSS/SSS_sync_intra is given in Table 9.2.5.1-1 for FR1 and  Table 9.2.5.1-2 for FR2. For UE indicating [no gap with interruption], TPSS/SSS_sync_intra is given in Table  9.2.5.1-X1 for FR1 and  Table 9.2.5.1-X2 for FR2.

-	Otherwise, TPSS/SSS_sync_intra is given in table 9.2.5.1-1, 9.2.5.1-2, 9.2.5.1-4 (deactivated SCell) or 9.2.5.1-5 (deactivated SCell) or 9.2.5.1-9 (deactivated SCell) or 9.2.5.1-11 or 9.2.5.1-12 (deactivated PSCell) or 9.2.5.1-13 (deactivated PSCell).

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured

-	For UE indicatting [no gap without interruption], TSSB_time_index_intra is given in Table 9.2.5.1-3 for FR1 and Table 9.2.5.1-15 for FR2-2. For UE indicating [no gap with interruption], TSSB_time_index_intra is given in Table 9.2.5.1-X3 for FR1 and Table 9.2.5.1-X4 for FR2-2.

-	Otherwise, TSSB_time_index_intra is given in table 9.2.5.1-3, 9.2.5.1-15 (FR2-2), 9.2.5.1-6 (deactivated SCell), 9.2.5.1-10(deactivated SCell) or 9.2.5.1-14 (deactivated PSCell).

-	T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TSSB_measurement_period_intra is given in Table 9.2.5.2-7; otherwise, T SSB_measurement_period_intra is given in Table 9.2.5.2-2.

-	For UE indicating [no gap without interruption], TSSB_measurement_period_intra is given in Table 9.2.5.2-1 for FR1 and  Table 9.2.5.2-2 for FR2. For UE indicating [no gap with interruption], TSSB_measurement_period_intra is given in Table 9.2.5.2-Y1 for FR1 and  Table Table 9.2.5.2-Y2 for FR2.

-	For UE indicating [no gap without interruption], TSSB_measurement_period_intra is given in Table 9.2.5.2-1 for FR1 and  Table 9.2.5.2-2 for FR2. For UE indicating [no gap with interruption], TSSB_measurement_period_intra is given in Table 9.2.5.2-Y1 for FR1 and  Table Table 9.2.5.2-Y2 for FR2.

-	For power class 6 UE supporting [measurementEnhancementCAInterFreqFR2-r18] when [highSpeedMeasFlagFR2] is configured, the T SSB_measurement_period_intra given in Table 9.2.5.2-7 (if SMTC <= 40ms) and Table 9.2.5.2-2 (if SMTC > 40ms) shall apply for SCC.

-	Otherwise, T SSB_measurement_period_intra is given in table 9.2.5.2-1, table 9.2.5.2-2 table 9.2.5.2-3 (deactivated SCell), 9.2.5.2-4(deactivated SCell), 9.2.5.2-5 or 9.2.5.2-6(deactivated SCell), 9.2.5.2-8(deactivated PSCell) or 9.2.5.2-9(deactivated PSCell).

-	CSSFintra: it is a carrier specific scaling factor and is determined

-	according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e.

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP, or

-	 when intra-frequency SMTC is fully non overlapping with GAP for UE indicating [no gap with interruption],  or

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP for UE indicating [no gap without interruption], or

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

if the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24.  For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24.	For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36.

MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra = 48 samples.

When UE supports concurrent measurement GAPs, i.e., supports the following capability or capabilities’ combination:

-	concurrentMeasGap-r17, or

-	[concurrent gap with Pre-MG capability], or

-	[concurrent gap with NCSG capability],

Or when UE supports musim-GapPreference-r17 or both concurrent measurement GAPs and musim-GapPreference-r17

and UE concurrent measurement GAPs or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured

Kp is the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gaps, and/or per-FR measurement GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in section 9.1.8.3, 9.1.10.4 and 9.1.10.5, respectively.

Kp = 1 when Navailable = 0.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

Editor Note: FSS for the case when Navailable = 0 due to fully overlapping between SMTC occasions and the union of MUSIM gap and measurement gap occasions within the window W.

When UE supports [MUSIM-GapConfig-17] and the SMTC occasion of the target frequency layer is overlapping with the configured aperiodic MUSIM gap, longer cell identification period for the target frequency layer is expected.

-	Otherwise, when the UE is not configured with or UE does not support concurrent measurement GAPs and the UE is not configured with periodic MUSIM gaps or UE does not support MUSIM gaps:

When intra-frequency SMTC is fully non overlapping with measurement gaps or NCSG, or intra-frequency SMTC is fully overlapping with MGs or NCSG, Kp=1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When intra-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.	If the higher layer signaling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in Table 9.2.5.1-1, Table 9.2.5.1-2, Table 9.2.5.1-3, Table 9.2.5.1-4, Table 9.2.5.1-5 and Table 9.2.5.1-6 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in Table 9.2.5.1-1, Table 9.2.5.1-2, Table 9.2.5.1-3, Table 9.2.5.1-4, Table 9.2.5.1-5, Table 9.2.5.1-6, Table 9.2.5.1-12, Table 9.2.5.1-13 and Table 9.2.5.1-14 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

-	When the target SSB is completely contained in active BWP of UE or the active downlink BWP is initial BWP, the intra-frequency measurement should be without gap without interruption regardless of the NeedForGaps’ status reporting.

----------------------END OF CHANGE 1----------------------------

----------------------START OF CHANGE 2----------------------------

### 9.2.6	Intra-frequency measurements with measurement gaps

#### 9.2.6.1	Void

#### 9.2.6.2	Intra-frequency cell identification

When a measurement gap is provided or an activated Pre-MG is provided without any pre-MG status changed during the measurement period, the UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra  ms

Tidentify_intra_with_index = TPSS/SSS_sync_ntra + T SSB_measurement_period_intra + TSSB_time_index_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2.6.2-1, 9.2.6.2-2 or 9.2.6.2-9.

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TPSS/SSS_sync_intra is given in Table 9.2.6.2-9; otherwise, TPSS/SSS_sync_intra is given in Table 9.2.6.2-2.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2.6.2-3 or 9.2.6.2-10 (for FR2-2).

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.6.3-1 or 9.2.6.3-2.

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TSSB_measurement_period_intra is given in Table 9.2.6.3-4; otherwise, T SSB_measurement_period_intra is given in Table 9.2.6.3-2.

-	For power class 6 UE supporting [measurementEnhancementCAInterFreqFR2-r18] when [highSpeedMeasFlagFR2] is configured, the T SSB_measurement_period_intra given in Table 9.2.6.3-4 (if SMTC <= 40ms) and Table 9.2.6.3-2 (if SMTC > 40ms) shall apply for SCC.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with or the UE does not support concurrent measurement GAPs or MUSIM gaps. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gaps and per-FR measurement GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other GAP and MUSIM gap occasions within the window, and

Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in section 9.1.8.3, 9.1.10.4  and 9.1.10.5, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements in this clause do not apply if Navailable =0.

When UE supports [MUSIM-GapConfig-17] and the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in section 9.1.10.4 is applied, longer cell identification period for the target intra-frequency is expected.

Mpss/sss_sync_with_gaps : For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync with_gaps=40. For a UE supporting FR2-1 power class 2, Mpss/sss_sync with_gaps =24.  For a UE supporting FR2-1 power class 3, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync with_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync with_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync with_gaps = 36.

Mmeas_period_ with_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_ with_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 4, Mmeas_period with_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_ with_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_ with_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_ with_gaps = 36.

-	MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48. For a UE supporting FR2 power class 3, MSSB_index_intra = 48.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in Table 9.2.6.2-1, Table 9.2.6.2-2, and Table 9.2.6.2-3 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in Table 9.2.6.2-1, Table 9.2.6.2-2, and Table 9.2.6.2-3 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

----------------------END OF CHANGE 2----------------------------

----------------------START OF CHANGE 3----------------------------

### 9.3.4	Inter-frequency measurement with measurement gaps

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured) or deriveSSB-IndexFromCellInter-r17 is configured for the FR1 and FR2-1 target frequency layers and and UE supporting deriveSSB-IndexFromCellInterNon-NCSG-r17. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.4-1, table 9.3.4-2, table 9.3.4-5 when highSpeedMeasInterFreq-r17 is configured and UE supports measurementEnhancementInterFreq-r17 and table 9.3.4-9 when highSpeedMeasFlagFR2-r17 is configured and UE supports [measurementEnhancementCAInterFreqFR2-r18]. When the SCG is deactivated, table 9.3.4-7 applies for an inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.4-2 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.4-2 applies for an inter-frequency carrier configured only by MCG.

-	For UE supporting power class 6 and [measurementEnhancementCAInterFreqFR2-r18] with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TPSS/SSS_sync_inter is given in Table 9.3.4-9; otherwise, TPSS/SSS_sync_inter is given in Table 9.3.4-2.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.4-3,table 9.3.4-6 when highSpeedMeasInterFreq is configured and UE supports measurementEnhancementInterFreq-r17, and table 9.3.4-10 when highSpeedMeasFlagFR2-r17 is configured and UE supports [measurementEnhancementCAInterFreqFR2-r18]. When the SCG is deactivated, table 9.3.4-8 applies for an inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.4-4 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.4-4 applies for an inter-frequency carrier configured only by MCG.

-	For UE supporting power class 6 and [highSpeedFR2measurementEnhancementInterFreq-r18] with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TSSB_measurement_period_inter is given in Table 9.3.5-5; otherwise, TSSB_measurement_period_inter is given in Table 9.3.5-2.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.5-1, table 9.3.5-2, table 9.3.5-3 when highSpeedMeasInterFreq is configured and UE supports measurementEnhancementInterFreq-r17, and in table 9.3.5-5 when highSpeedMeasFlagFR2-r17 is configured and UE supports [measurementEnhancementCAInterFreqFR2-r18]. When the SCG is deactivated, table 9.3.5-4 applies for an inter-frequency carrier  configured by SCG and not configured by MCG and table 9.3.5-2 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.5-2 applies for an inter-frequency carrier configured only by MCG.

-	For UE supporting power class 6 and [measurementEnhancementCAInterFreqFR2-r18] with highSpeedMeasFlagFR2-r17 configured, TSSB_measurement_period_inter is given in Table 9.3.5-5; otherwise, TSSB_measurement_period_inter is given in Table 9.3.5-2.

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_inter = 64 samples. For a UE supporting FR2-1 power class 2, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_inter = 96. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 60. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_inter = 60.

MSSB_index_inter: For a UE supporting FR2-1 power class 1 or 5, MSSB_index_inter = 40 samples. For a UE supporting FR2 power class 2, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 3, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 4, MSSB_index_inter = 24 samples. For a UE supporting FR2-2 power class 2 or 3, MSSB_index_inter = 48 samples. For a UE supporting FR2 power class 1, MSSB_index_inter = 72 samples.

Mmeas_period_inter: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_inter =64. For a UE supporting FR2-1 power class 2, Mmeas_period_inter=40. For a UE supporting FR2-1 power class 3, Mmeas_period_inter =40. For a UE supporting FR2-1 power class 4, Mmeas_period_inter = 40. For a UE supporting FR2-2 power class 1, Mmeas_period_inter = 96. For a UE supporting FR2-2 power class 2, Mmeas_period_inter = 60. For a UE supporting FR2-2 power class 3, Mmeas_period_inter = 60.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Kgap is a scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement GAPs or MUSIM gaps. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gaps, and/or per-FR measurement GAPs within the same FR, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement GAP and MUSIM gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for the GAP and MUSIM gap in section 9.1.8.3 and 9.1.10.5, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Kgap is only applicable for UE supporting concurrent measurement GAPs or MUSIM gaps. When concurrent measurement GAPs or MUSIM gaps are configured, requirements in this clause do not apply if Navailable =0.

When UE supports [musim-GapPreference-r17] and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in section 9.1.10.4 is applied, longer cell identification period for the target inter-frequency is expected.

----------------------END OF CHANGE 3----------------------------

----------------------START OF CHANGE 4----------------------------

### 9.3.9	Inter frequency measurements without measurement gaps

#### 9.3.9.1	Inter frequency Cell identification

UE satisfying the applicability conditions specified in 9.3.1 on the requirement in this clause shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured) or deriveSSB-IndexFromCellInter-r17 is configured for the FR1 and FR2-1 target frequency layers and and UE supporting deriveSSB-IndexFromCellInterNon-NCSG-r17. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index.

-	For inter-frequency SSB based measurements without measurement gaps in active BWP, it is assumed that when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1 and FR2, SFN and frame boundary across serving cell and inter-frequency neighbor cells is aligned

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection

-	For inter-frequency SSB based measurements without measurement gaps in active BWP, and UE supports interFrequencyMeas-Nogap-r16,  TPSS/SSS_sync_inter is given in table 9.3.9.1-1 and table 9.3.9.1-2.

-	For UE indicating [nogap-nointerurption], TPSS/SSS_sync_inter is given in Table 9.3.9.1-1 for FR1 and Table 9.3.9.1-2 for FR2

-	For UE indicating [nogap-interruption], TPSS/SSS_sync_inter is given in Table 9.3.9.1-1a for FR1 and Table 9.3.9.1-2a for FR2.

-	TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured

-	For inter-frequency SSB based measurements without measurement gaps in active BWP, and UE supports interFrequencyMeas-Nogap-r16,  TSSB_time_index_inter is given in table 9.3.9.1-3 and table 9.3.9.1-4.

-	For UE indicating [nogap-nointerurption], TSSB_time_index_inter is given in Table 9.3.9.1-3 for FR1 and Table 9.3.9.1-4 for FR2

-	For UE indicating [nogap-interruption], TSSB_time_index_inter is given in Table 9.3.9.1-3a for FR1 and Table 9.3.9.1-4a for FR2.

-	T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement

-	For inter-frequency SSB based measurements without measurement gaps in active BWP, and UE supports interFrequencyMeas-Nogap-r16,  T SSB_measurement_period_inter is given in table 9.3.9.2-1, table 9.3.9.2-2, table 9.3.9.2-3 and table 9.3.9.2-3a when highSpeedMeasInterFreq-r17 is configured and UE supports measurementEnhancementInterFreq-r17, and table 9.3.9.2-4 when highSpeedMeasFlagFR2-r17 is configured and UE supports [measurementEnhancementCAInterFreqFR2-r18].

-	For UE indicating [nogap-nointerurption], T SSB_measurement_period_inter is given in Table 9.3.9.2-1 for FR1, table 9.3.9.2-2 for FR2, and table 9.3.9.2-3 when highSpeedMeasInterFreq-r17 is configured and UE supports measurementEnhancementInterFreq-r17.

-	For UE indicating [nogap-interruption], T SSB_measurement_period_inter is given in Table 9.3.9.2-1a for FR1 and table 9.3.9.2-2a for FR2, and table 9.3.9.2-3b when highSpeedMeasInterFreq-r17 is configured and UE supports measurementEnhancementInterFreq-r17.

-	For UE supporting power class 6 and [measurementEnhancementCAInterFreqFR2-r18] with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TSSB_measurement_period_inter is given in Table 9.3.9.2-x; otherwise, TSSB_measurement_period_inter is given in Table 9.3.9.2-2.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside GAP, i.e. when interfrequency SMTC is fully non overlapping or partially overlapping with GAPs.

-	when inter-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps for UE indicating [nogap-nointerruption] or

-	when inter-frequency SMTC is fully non overlapping with measurement gaps for UE indicating [no gap with interruption],

For inter-frequency SSB based measurements without measurement gaps in active BWP

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_inter = 40. For a UE supporting FR2-1 power class 2, Mpss/sss_sync_inter = 24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter = 24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync = 24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_inter = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_inter = 36. For FR1, Mpss/sss_sync_inter = 5.

MSSB_index_inter: For a UE supporting FR2-2 power class 1, MSSB_index_inter = 72. For a UE supporting FR2-2 power class 2, MSSB_index_inter = 48. For a UE supporting FR2-2 power class 3, MSSB_index_inter = 48. For FR1, MSSB_index_inter = 3.

Mmeas_period_inter: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_inter = 40. For a vehicle mounted UE supporting FR2-1 power class 2, Mpss/sss_sync_inter=24. For a UE supporting FR2-1 power class 3, Mmeas_period_inter = 24. For a UE supporting FR2-1 power class 4, Mmeas_period_inter = 24. For a UE supporting FR2-2 power class 1, Mmeas_period_inter = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_inter = 36. For FR1, Mmeas_period_inter = 5.

If the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement or the UE indicates either [nogap-intrruption] or [nogap-nointerruption] via [NeedForGap-InfoNR-R18],

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_inter = 64 samples. For a UE supporting FR2-1 power class 2, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_inter = 96. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 60. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_inter = 60. For FR1, Mpss/sss_sync_inter = 8.

MSSB_index_inter: For a UE supporting FR2-1 power class 1 or 5, MSSB_index_inter = 40 samples. For a UE supporting FR2 power class 2, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 3, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 4, MSSB_index_inter = 24 samples. For a UE supporting FR2-2 power class 2 or 3, MSSB_index_inter = 48 samples. For a UE supporting FR2 power class 1, MSSB_index_inter = 72 samples. For FR1, MSSB_index_inter = 3.

Mmeas_period_inter: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_inter =64. For a UE supporting FR2-1 power class 2, Mmeas_period_inter=40. For a UE supporting FR2-1 power class 3, Mmeas_period_inter =40. For a UE supporting FR2-1 power class 4, Mmeas_period_inter = 40. For a UE supporting FR2-2 power class 1, Mmeas_period_inter = 96. For a UE supporting FR2-2 power class 2, Mmeas_period_inter = 60. For a UE supporting FR2-2 power class 3, Mmeas_period_inter = 60. For FR1, Mmeas_period_inter = 8.

When UE supports concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrent measurement GAP and musim-GapPreference-r17 and the UE is configured with concurrent measurement GAPs or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps,

Kp is a scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and per-FR GAPs within the same FR as the SSB frequency layer, and starting at the beginning of any SMTC occasion:

Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and

Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for GAP and MUSIM gap collisions by applying the collision rules for the measurement GAP and MUSIM gap in section 9.1.8.3, 9.1.10.4 and 9.1.10.5, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Kp = 1 when Navailable = 0.

Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

Editor Note: FSS for the case when Navailable = 0 due to fully overlapping between SMTC occasions and the union of MUSIM gap and measurement gap occasions within the window W.

When UE supports [musim-GapPreference-r17] and the SMTC occasions of the target frequency layer is fully or partially overlapping with the configured aperiodic MUSIM gap, longer cell identification period for the target frequency layer is expected.

Otherwise, when UE is not configured with or UE does not support concurrent measurement GAPs and the UE is not configured with MUSIM gaps:

When interfrequency SMTC is fully non overlapping with measurement gaps or NCSG, or interfrequency SMTC is fully overlapping with MGs or NCSG, Kp =1.

When interfrequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When inter-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP.

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by inter-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by inter-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by SSB-ToMeasure and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

Table 9.3.9.1-1: Time period for PSS/SSS detection, (FR1)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, ceil(Mpss/sss_sync_inter x Kp) x SMTC period )Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2x Mpss/sss_sync_inter x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mpss/sss_sync_inter x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.3.9.1-1a: Time period for PSS/SSS detection, when UE indicate [nogap-interruption] (FR1)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, Mpss/sss_sync_inter x max(80ms, SMTC period) )Note 1 x CSSFinter |
| [DRX cycle≤ 320ms] | max( 600ms, ceil(M2 x Mpss/sss_sync_inter) x max(80ms, SMTC period,DRX cycle)) x CSSFinter |
| [DRX cycle>320ms] | Mpss/sss_sync_inter x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.3.9.1-2: Time period for PSS/SSS detection, (FR2)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_inter x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mpss/sss_sync_inter x Kp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void |  |

Table 9.3.9.1-2a: Time period for PSS/SSS detection, when UE indicate [nogap-interruption] (FR2)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_inter x Klayer1_measurement) x max(80ms, SMTC period))Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_inter x Klayer1_measurement) x [max(80ms, SMTC period, DRX cycle)]) x CSSFinter |
| DRX cycle>320ms | ceil(Mpss/sss_sync_inter x Klayer1_measurement) x [DRX cycle x] CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void |  |

Table 9.3.9.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil(MSSB_index_inter x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 x MSSB_index_inter x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil(MSSB_index_inter x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.3.9.1-3a: Time period for time index detection, when UE indicate [nogap-interruption] (FR1)

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, MSSB_index_inter x max(80ms, SMTC period))Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 x MSSB_index_inter) x [max(80ms, SMTC period, DRX cycle)]) x CSSFinter |
| DRX cycle>320ms | MSSB_index_inter x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.3.9.1-4: Time period for time index detection (FR2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200ms, Ceil(MSSB_index_inter x Kp) SMTC period) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(1.5 MSSB_index_inter x Kp) Max(SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(MSSB_index_inter x Kp) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: Kp is applicable for UE supporting concurrentMeasGap-r17 |  |

Table 9.3.9.1-4a: Time period for time index detection, when UE indicate [nogap-interruption] (FR2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200ms, MSSB_index_inter max(80ms, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(1.5 MSSB_index_inter) Max(80ms, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | MSSB_index_inter DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: Kp is applicable for UE supporting [concurrent gaps] and MUSIM gaps |  |

Table 9.3.9.1-5: Time period for time index detection (FR1) [for a target cell with 12 or 15 PRB SSB]

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( [6] x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 x [6] x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil( [6] x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: FFS When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

----------------------END OF CHANGE 4----------------------------

----------------------START OF CHANGE 5----------------------------

### 9.4.2	NR − E-UTRAN FDD measurements

#### 9.4.2.1	Introduction

The requirements are applicable for NR−E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements.

In the requirements, an E-UTRAN FDD cell is considered to be detectable when:

-	RSRP related conditions in the accuracy requirements in clause 10.2.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [15],

-	RSRQ related conditions in the accuracy requirements in clause 10.2.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [15],

-	RS-SINR related conditions in the accuracy requirements in clause 10.2.5 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.19 of TS 36.133 [15].

#### 9.4.2.2	Requirements when no DRX is used

When the UE requires measurement gaps or NCSG to identify and measure inter-RAT cells and an appropriate measurement gap pattern or NCSG is scheduled, or when the UE is capable of concurrent measurement gap patterns and concurrent measurement gap patterns are scheduled, or an appropriate pre-MG is scheduled and activated, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable FDD cell within TIdentify, E-UTRAN FDD according to the following expression:

TIdentify,  E-UTRAN FDD=TBasicIdentify∙480TInter1∙CSSFinterRAT      ms,

where:

TBasicIdentify = 480 ms,

TInter1 is defined in clause 9.4.1,

CSSFinterRAT = CSSFwithin_gap,i when measurement gaps are configured, or CSSFwithin_ncsg,i when NCSGs are configured, is the scaling factor for the measured inter-RAT E-UTRA carrier i which is calculated as specified in clause 9.1.5.2.

For a UE supporting and configured with concurrent measurement GAPs or MUSIM gaps or both concurrent measurement gaps and MUSIM gaps, Kgap_EUTRA: it is the scaling factor for an E-UTRAN frequency layer to be measured within the associated GAP pattern. Kgap = 1 when the UE is not configured with concurrent measurement GAPs nor MUSIM gaps. Otherwise, Kgap_EUTRA = Ntotal / Navailable for UE configured with concurrent measurement GAPs.

For a window W of duration xRP_max, where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gaps, and per-FR measurement GAPs for FR1, and starting from the beginning of any associated gap occasion:

Ntotal is the total number of associated GAP occasions within the window, including those dropped and non-dropped ocassions within the window, and

Navailable is the number of non-dropped associated GAP occasions after accounting for measurement GAP and MUSIM gap collisions  by applying the collision rules for the GAP and MUSIM gap in section 9.1.8.3 and 9.1.10.5, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements do not apply for UE configured with concurrent measurement GAPs or MUSIM gaps, if Navailable =0

When UE supports [musim-GapPreference-r17] and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in section 9.1.10.4 is applied, longer cell identification period for the target inter-RAT carrier is expected.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of TMeasure, E-UTRAN FDD defined in Table 9.4.2.2-1.

Table 9.4.2.2-1: Measurement period and measurement bandwidth

| Configuration | Physical Layer Measurement period: TMeasure, E-UTRAN FDD [ms] | Measurement bandwidth [RB] |
| --- | --- | --- |
| 0 | 480 x [CSSFinterRAT x Ceil(Kgap_EUTRA)] | 6 |
| 1 (Note 1) | 240 x [CSSFinterRAT x Ceil(Kgap_EUTRA)] | 50 |
| NOTE 1: This configuration is optional. NOTE 2: Kgap_EUTRA is only applicable for a UE supporting concurrent measurement gaps. Otherwise Kgap_EUTRA =1 |  |  |

When measurement gaps are scheduled for E-UTRAN FDD inter-RAT measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting RSRP, RSRQ, and RS-SINR measurements to higher layers with measurement period TMeasure, E-UTRAN FDD given by table 9.4.2.2-1.

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN FDD RSRP measurement accuracy for all measured cells shall be as specified in clause 10.2.2. The NR – E-UTRAN FDD RSRQ measurement accuracy for all measured cells shall be as specified in clause 10.2.3. The NR – E-UTRAN FDD RS-SINR measurement accuracy for all measured cells shall be as specified in clause 10.2.5.

#### 9.4.2.3	Requirements when DRX is used

When DRX is in use and an appropriate measurement gap pattern or NCSG is configured, or when the UE is capable of concurrent measurement gap patterns and concurrent measurement gap patterns are configured, or an appropriate pre-MG is scheduled and activated, the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in Table 9.4.2.3-1. When highSpeedMeasFlag-r16 is configured and UE supports the enhanced inter-RAT E-UTRAN measurement requirements, the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in Table 9.4.2.3-2.

For a UE supporting and configured with concurrent measurement GAPs, or MUSIM gaps or both concurrent measurement gaps and MUSIM gaps, Kgap_EUTRA: it is the scaling factor for an E-UTRAN frequency layer to be measured within the associated GAP pattern. Kgap = 1 when the UE is not configured with concurrent measurement GAPs nor MUSIM gaps. Otherwise, Kgap_EUTRA = Ntotal / Navailable for UE configured with concurrent measurement GAPs.

For a window W of duration xRP_max, where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gap(s), and per-FR GAPs for FR1, and starting from the beginning of any associated gap occasion:

Ntotal is the total number of associated GAP occasions within the window, including both dropped and non-dropped instances of the associated GAP within the window, and

Navailable is the number of non-dropped associated GAP occasions after accounting for measurement gap and MUSIM gap collisions  by applying the collision rules for the GAP and MUSIM gap in section 9.1.8.3 and 9.1.10.5, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements do not apply for UE configured with concurrent measurement GAPs or MUSIM gaps, if Navailable =0

When UE supports [musim-GapPreference-r17] and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in section 9.1.10.4 is applied, longer cell identification period for the target inter-RAT carrier is expected.

----------------------END OF CHANGE 5----------------------------

----------------------START OF CHANGE 6----------------------------

### 9.4.3	NR − E-UTRAN TDD measurements

#### 9.4.3.1	Introduction

The requirements are applicable for NR−E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements.

In the requirements, an E-UTRAN TDD cell is considered to be detectable when:

-	RSRP related conditions in the accuracy requirements in clause 10.2.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [15],

-	RSRQ related conditions in the accuracy requirements in clause 10.2.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [15],

RS-SINR related conditions in the accuracy requirements in clause 10.2.5 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.19 of TS 36.133 [15].

#### 9.4.3.2	Requirements when no DRX is used

When the UE requires measurement gaps or NCSG to identify and measure inter-RAT cells and an appropriate measurement gap pattern or NCSG is scheduled, or when the UE is capable of concurrent measurement gap patterns and concurrent measurement gap patterns are scheduled, or an appropriate pre-MG is scheduled and activated or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable TDD cell within TIdentify, E-UTRAN TDD according to the following expression:

-	When configuration 0 or configuration 1 in Table 9.4.3.2-1 is applied,

TIdentify,  E-UTRAN TDD=TBasicIdentify∙480TInter1∙CSSFinterRAT      ms,

-	When configuration 2 or configuration 3 in Table 9.4.3.2-1 is applied,

TIdentify,  E-UTRAN TDD=TBasicIdentify∙480TInter1∙CSSFinterRAT+240∙CSSFinterRAT      ms,

where:

TBasicIdentify = 480 ms,

TInter1 is defined in clause 9.4.1,

CSSFinterRAT = CSSFwithin_gap,i when measurement gaps are configured, or CSSFwithin_ncsg,i when NCSGs are configured, is the scaling factor for the measured inter-RAT E-UTRA carrier i which is calculated as specified in clause 9.1.5.2.

For a UE supporting and configured with concurrent measurement GAPs, or MUSIM gaps or both concurrent measurement gaps and MUSIM gaps, Kgap_EUTRA: it is the scaling factor for an E-UTRAN frequency layer to be measured within the associated GAP pattern. Kgap = 1 when the UE is not configured with concurrent measurement GAPs nor MUSIM gaps. Otherwise, Kgap_EUTRA = Ntotal / Navailable for UE configured with concurrent measurement GAPs or MUSIM gaps..

-	For a window W of duration xRP_max , where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gaps,and per-FR measurement GAPs for FR1, and starting from the beginning of any associated gap occasion:

-	Ntotal is the total number of associated GAP occasions within the window, including those overlapped with other measurement gap and MUSIM gap ocassions within the window, and

-	Navailable is the number of non-dropped associated GAP occasions after accounting for measurement gap and MUSIM gap collisions by applying the GAP and MUSIM gap in section 9.1.8.3 and 9.1.10.5, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

-	Requirements do not apply for UE configured with concurrent measurement GAPs or MUSIM gaps, if Navailable =0

When UE supports [musim-GapPreference-r17] and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in section 9.1.10.4 is applied, longer cell identification period for the target inter-RAT carrier is expected.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of TMeasure, E-UTRAN TDD defined in Table 9.4.3.2-1.

Table 9.4.3.2-1: TMeasure, E-UTRAN TDD for different configurations

| Configuration | Measurement bandwidth (RB) | Number of UL/DL sub-frames per half frame (5 ms) |  | DwPTS |  | TMeasure, E-UTRAN TDD (ms) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DL | UL | Normal CP | Extended CP |  |
| 0 | 6 | 2 | 2 |  |  | 480 x CSSFinterRAT x Ceil(Kgap_EUTRA) |
| 1 (Note 1) | 50 | 2 | 2 |  |  | 240 x CSSFinterRAT x Ceil(Kgap_EUTRA) |
| 2 | 6 | 1 | 3 |  |  | 720 x CSSFinterRAT x Ceil(Kgap_EUTRA) |
| 3 (Note 1) | 50 | 1 | 3 |  |  | 480 x CSSFinterRAT x Ceil(Kgap_EUTRA) |
| NOTE 1: This configuration is optional. NOTE 2: Void NOTE 3: Kgap_EUTRA is only applicable for a UE supporting concurrent measurement gaps and/or MUSIM gaps. Otherwise Kgap_EUTRA =1 |  |  |  |  |  |  |

When measurement gaps are scheduled for E-UTRAN TDD inter-RAT measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting RSRP, RSRQ, and RS-SINR measurements to higher layers with measurement period Tmeasure, E-UTRAN TDD given by table 9.4.3.2-1.

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN TDD RSRP measurement accuracy for all measured cells shall be as specified in clause 10.2.2. The NR – E-UTRAN TDD RSRQ measurement accuracy for all measured cells shall be as specified in clause 10.2.3. The NR – E-UTRAN TDD RS-SINR measurement accuracy for all measured cells shall be as specified in clause 10.2.5.

#### 9.4.3.3	Requirements when DRX is used

When DRX is in use and an appropriate measurement gap pattern or NCSG is configured, or when the UE is capable of concurrent measurement gap patterns and concurrent measurement gap patterns are configured, or an appropriate pre-MG is scheduled and activated, the UE shall be able to identify a new detectable E-UTRAN TDD cell within TIdentify, E-UTRAN TDD specified in Table 9.4.3.3-1. When highSpeedMeasFlag-r16 is configured and UE supports the enhanced inter-RAT E-UTRAN measurement requirements, the UE shall be able to identify a new detectable E-UTRAN TDD cell within TIdentify, E-UTRAN TDD specified in Table 9.4.3.3-2.

For a UE supporting and configured with concurrent measurement GAPs, or MUSIM gaps or both concurrent measurement gaps and MUSIM gaps, Kgap_EUTRA: it is the scaling factor for an E-UTRAN frequency layer to be measured within the associated GAP pattern. Kgap = 1 when the UE is not configured with concurrent measurement GAPs nor MUSIM gaps.. Otherwise, Kgap_EUTRA = Ntotal / Navailable for UE configured with concurrent measurement GAPs or MUSIM gaps.

For a window W of duration xRP_max, where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gap(s), and per-FR measurement GAPs for FR1, and starting from the beginning of any associated gap occasion:

Ntotal is the total number of associated GAP occasions within the window, including those overlapped with other measurement GAP and MUSIM gap instances within the window, and

Navailable is the number of non-dropped associated GAP occasions after accounting for measurement gap and MUSIM gap collisions by applying the  collision rules for the GAP and MUSIM gap in section 9.1.8.3 and 9.1.10.5, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements do not apply for UE configured with concurrent measurement GAPs or MUSIM gaps, if Navailable =0

When UE supports [musim-GapPreference-r17] and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in section 9.1.10.4 is applied, longer cell identification period for the target inter-RAT carrier is expected.

----------------------END OF CHANGE 6----------------------------

----------------------START OF CHANGE 7----------------------------

### 9.10.2	CSI-RS based intra-frequency measurements

...

#### 9.10.2.5	Intra-frequency measurements without measurement gaps

If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the CSI-RS based measurement shall include PSS/SSS detection time of associatedSSB, the time period used to acquire the SFN information and CSI-RS based measurement period without gap.

-	PSS/SSS detection time of associatedSSB is the intra-frequency TPSS/SSS_sync_intra in Clause 9.2.5.1.

-	The time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, the time period used to acquire the SFN information is TCSI-RS_SFN_intra as shown in Table 9.10.2.5-3 for FR1. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

-	If the associatedSSB, which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2.5.1, becomes undetectable for a period ≤ 5 seconds and then the associatedSSB becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than ± 3200/2µ Tc, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3], PSS/SSS detection time and time period used to acquire the SFN information are equal to 0.

The measurement period for CSI- RS based intra-frequency measurements without gaps is as shown in table 9.10.2.5-1and Table 9.10.2.5-2.

Additionally, for a given CSI-RS resource, if the associated SS/PBCH block is configured but not detected by the UE, or if CSI-RS is configured with associated SSB but not QCL-ed to the associated SSB, the UE is not required to monitor the corresponding CSI-RS resource.

Table 9.10.2.5-1: Measurement period for intrafrequency CSI-RS based measurements without gaps(FR1)

| DRX cycle | T CSI-RS_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp_CSI-RS) x CSI-RS period) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp_CSI-RS) x max(CSI-RS period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp_CSI-RS) x DRX cycle x CSSFintra |
| NOTE 1: The requirements apply assuming CSI-RS configuration with {D=3 with PRBs ≥ 48}. D is frequency domain density for the 1-port CSI-RS for L3 mobility defined in clause 7.4.1 of TS38.211 [6]. |  |

Table 9.10.2.5-2: Measurement period for intrafrequency CSI-RS based measurements without gaps(FR2)

| DRX cycle | T CSI-RS_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp_CSI-RS) x CSI-RS period) x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp_CSI-RS) x max(CSI-RS period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Mmeas_period_w/o_gaps x DRX cycle x CSSFintra |
| NOTE 1: The requirements apply assuming CSI-RS configuration with {D=3 with PRBs ≥ 48}. D is frequency domain density for the 1-port CSI-RS for L3 mobility defined in clause 7.4.1 of TS38.211 [6]. |  |

Mmeas_period_w/o_gaps : For a UE supporting power class 1, Mmeas_period_w/o_gaps =40. For a UE supporting FR2 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1.5.

For a UE not supporting concurrentMeasGap-r17 or for a UE is supporting concurrentMeasGap-r17 but not configured with concurrent measurement gaps, and the UE does not support [musim-GapPreference-r17] or is supporting [musim-GapPreference-r17] but not configured with periodic MUSIM gaps,

-	if the intra-frequency CSI-RS resource does not overlap with any measurement gaps, Kp_CSI-RS=1;

-	if some occasions of the intra-frequency CSI-RS resource is overlapped with any measurement gaps, Kp_CSI-RS = 1/(1- (CSI-RS resource period /MGRP)) , where CSI-RS resource period < MGRP, and the MGRP is the periodicity of the measurement gap.

-	Otherwise, when UE supports concurrent measurement gaps or [musim-GapPreference-r17] or both concurrent measurement gaps and [musim-GapPreference-r17],  and concurrent measurement gaps or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured, Kp_CSI-RS is the scaling factor for a CSI-RS frequency layer to be measured outside gap which is defined as Kp_CSI-RS = Ntotal / Navailable

For a window W of duration max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG, periodic MUSIM gaps, and per-FR MG within the same FR as the CSI-RS frequency layer, and starting at the beginning of any gap occasions covering the CSI-RS resources:

Ntotal is the total number of CSI-RS resource occasions within the window, including those overlapped with other MG and MUSIM gap occasions within the window, and

Navailable is the number of CSI-RS resource occasions that are not overlapped with any other non-dropped MG or non-dropped MUSIM gap occasions within the window W, after accounting for MG and MUSIM gap collisions by applying the collision rules for the measurement gap and MUSIM gap in section 9.1.8.3, 9.1.10.4 and 9.1.10.5, respectively.

Kp_CSI-RS = 1 when Navailable = 0

Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between CSI-RS occasions and MUSIM gap occasions within the window W.

Editor Note: FSS for the case when Navailable = 0 due to fully overlapping between CSI-RS occasions and the union of MUSIM gap and measurement gap occasions within the window W.

When UE supports [MUSIM-GapConfig-17] and the CSI-RS occasions of the target frequency layer is overlapping with the configured aperiodic MUSIM gap, longer cell identification period for the target frequency layer is expected.

----------------------END OF CHANGE 7----------------------------

----------------------START OF CHANGE 8----------------------------

### 9.10.3	CSI-RS based Inter-frequency measurements

…

#### 9.10.3.5	Inter frequency measurements with measurement gaps

When measurement gaps are provided, if configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the UE shall be able to identify a new detectable CSI-RS based inter frequency cell within T CSI-RS_identify_inter,

T CSI-RS_identify_inter = (TPSS/SSS_sync + T CSI-RS_measurement_period_inter + TCSI-RS_SFN_inter) ms

Where:

TPSS/SSS_sync is the time period used in PSS/SSS detection which is determined according to TPSS/SSS_sync_inter in clause 9.3.4,

TCSI-RS_SFN_inter is the time period used to acquire the SFN information of the cell being measured, which is shown in Table 9.10.3.5-3 for FR1 and equals inter-frequency TSSB_time_index_inter in Clause 9.3.4 for FR2,

TCSI-RS_measurement_period_inter: equal to a measurement period of CSI-RS based measurement given in table 9.10.3.5-1 and table 9.10.3.5-2.

Mmeas_period_inter: For a UE supporting FR2 power class 1 or 5, Mmeas_period_inter =8N samples. For a UE supporting FR2 power class 2, Mmeas_period_inter=5N samples. For a UE supporting FR2 power class 3, Mmeas_period_inter =5N samples. For a UE supporting FR2 power class 4, Mmeas_period_inter = 5N samples.  Note that scaling factor N = [8].

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5 for measurement conducted within measurement gaps.

When UE supports concurrent measurement gaps or [musim-GapPreference-r17] or both concurrent measurement gap and [musim-GapPreference-r17], and concurrent measurement gaps or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured, Kp_CSI-RS is the scaling factor for a CSI-RS frequency layer to be measured within the associated measurement gap which is defined as Kp_CSI-RS = Ntotal / Navailable. Kp_CSI-RS = 1 for UE not configured with concurrent measurement gaps and MUSIM gaps.

-	For a window W of duration max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG, periodic MUSIM gaps, and per-FR MG within the same FR as the CSI-RS frequency layer, and starting at the beginning of any gap occasions covering the CSI-RS resources.:

-	Ntotal is the total number of associated gap occasions covering CSI-RS resources within the window, including those overlapped with other measurement gap and MUSIM gap instances within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering CSI-RS resources within the window W, after accounting for MG and MUSIM gap collisions by applying the collision rules for the measurement gap and MUSIM gap in section 9.1.8.3 and 9.1.10.5, respectively.

-	Requirements do not apply if Navailable = 0

When UE supports [musim-GapPreference-r17] and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in section 9.1.10.4 is applied, longer cell identification period for the target inter-frequency is expected.

Additionally, for a given CSI-RS resource, if the associated SSB is configured but not detected by the UE, or if CSI-RS configured with associated SSB but not QCL-ed to the associated SSB, the UE is not required to monitor the corresponding CSI-RS resource.

----------------------END OF CHANGE 8----------------------------