3GPP TSG-WG4 Meeting #110 	R4-2403355

# Athens, Greece, February 26 - March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS 38.133 on SCell Activation Requirement in HST FR2 Enhanced |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | R18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Editor Notes: FFS additional condition/capability is needed is present in the SCell activation requirements in HST FR2 sceanrio. |  |  |  |  |  |  |  |  |
| Summary of change: |  | scellWithoutSSB is added as an additional condition for SCell activation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements is not compelte. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2402555. |  |  |  |  |  |  |  |  |

## <Start of Change #1>

### 8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time is the SCell activation delay in millisecond.

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms.

-	TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met

-	 ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

-	 ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

-	TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB;

-	TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise.

otherwise, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

-	6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

-	3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

-	However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	the SCell is contiguous to an active serving cell in the same band, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell

-	its RTD with contiguous FR1 active serving cell is larger than 260ns with respect to the to-be-activated SCell’s SSB numerology, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB;

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided

-	The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and

-	The difference of the reception power with the contiguous active serving cell is <= 6dB, and

-	The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

If the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled for UE supporting [scellWithoutSSB-interband],

-	The RTD between the target SCell and the colocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and

-	The [EPRE] difference at UE side is smaller than or equal to [9] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and

-	The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

where the reference serving cell can be indicated by higherlayer parameter [SSB-less-Referencecell]. If UE is not indicated with [SSB-less-Referencecell], the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured.

Editor notes: FFS whether and how to capture if there are more than one QCL source cell.

Editor notes: FFS whether and how to capture the wording “after the compensation for AGC”.

Tactivation_time is

-	Tfirst_TRS + TTRS +[5] ms, [if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support [ATRS based SSB-less operation]]

-	Tfirst_ATRS + Tgap + TATRS + [5] ms [if aperiodic CSI-RS resources are configured for Scell activation for UE supporting [ATRS based SSB-less operation]]

Editor notes: Further check the above [5]ms on processing time for fine time tracking.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms provided:

-	The UE is provided with SMTC for the target SCell, and

-	The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and

-	The parameter ssb-PositionsInBurst is same for the serving cell(s) and the Scell, and

-	SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the FR2 power class 6 UE supporting scellWithoutSSB and [Enhanced FR2 HST RRM requirements for intra-band CA and inter-frequency measurements in connected mode] is configured with highSpeedMeasFlagFR2-r17, Tactivation_time is 3 ms , provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2:

## <End of Change #1>