3GPP TSG-RAN WG4 Meeting #110	R4-2403372

Athens, Greece, 26 Feb – 01 Mar, 2024

Title: 	WF on RRM requirements for NR_MIMO_evo_DL_UL

Agenda Item:	8.21.5

Source: 	Samsung

Document for:	Approval

<Topic 1 RRM core requirements maintenance #1: Timing requirements for UL multi-DCI multi-TRP with two TAs >

Issue 1-1-2: Clarification of association of UL TCI states and DL RS

< Agreement>:

- [One of] downlink reference signal(s) associated with the same coresetPoolIndex as uplink signal

<Topic 2 RRM core requirements maintenance #2: Unified TCI Framework extended to M-TRP>

Issue 2-1-3: For sDCI mTRP, how to specify DL MAC CE based dual TCI state switch the switching delay requirements for Case 3, if SSB are adjacent in FR2?

< Agreement>:

- Longer delay is expected

Issue 2-1-4-a: For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE supporting two TAs and but not supporting RTD>CP?

<Way forward >

- Option 1

  - Known case: THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms)

  - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + 2ms)+ Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms

- Option 2

  - Known case: THARQ + 3Nslotsubframe,µ + NM*( Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms)

  - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP +  Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms

- Option 3

  - RAN4 shall discuss which reference signal is considered as the one associated with the UL/joint TCI state, and whether it can be assumed that the associated reference signal is always monitored by UE.

- Option 4

  - Make a down-selection from the three options for separate UL TCI state switching in mDCI mTRP:

    - Option 1: Add additional time for T/F tracking for separate TCI state switching.

    - Option 2: Not add additional time for T/F tracking and UE is not mandatory to meet uplink timing requirements for separate TCI state switching.

    - Option 3: Align the same rule as joint UL TCI state switching: UE is not expected to transmit on UL based on the target TCI state unless DL TCI state switch has also been activated yet.

- Option 5

  - if PL-RS is maintained, use legacy Rel-17 requirements. If PL-RS is not maintained, add further SSB timing and frequency tracking.

Issue 2-1-4-b: For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE supporting two TAs and supporting RTD>CP in FR1?

<Way forward >

- Option 1

  - Known case: THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms)

  - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + 2ms)+ Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms

- Option 2

  - Known case:

    - n+ THARQ + 3Nslotsubframe,µ + TOk(Tfirst-SSB + 2ms) + OL *TSSB when PL-RS is maintained

    - n+HARQ+ 3Nslotsubframe,µ+ TOk*TfirstSSB+ Tfirst_PL-RS + 4*TPL-RS +2ms+ OL*TSSB

Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE;

Tfirst_PL-RS is time to first pathloss RS transmission after first SSB transmission for T/F tracking. Tfirst_PL-RS should be larger than 2ms when PL-RS is CSI-RS.

OL=1 if SSB overlaps or adjacent to SSB from other TRP in FR2 and SSB periodicity is less than that of other TRP, 0 otherwise

  - Unknown case:

    - No Requirements.

- Option 3

  - RAN4 shall discuss which reference signal is considered as the one associated with the UL/joint TCI state, and whether it can be assumed that the associated reference signal is always monitored by UE.

- Option 4

  - Make a down-selection from the three options for separate UL TCI state switching in mDCI mTRP:

    - Option 1: Add additional time for T/F tracking for separate TCI state switching.

    - Option 2: Not add additional time for T/F tracking and UE is not mandatory to meet uplink timing requirements for separate TCI state switching.

    - Option 3: Align the same rule as joint UL TCI state switching: UE is not expected to transmit on UL based on the target TCI state unless DL TCI state switch has also been activated yet.

- Option 5

  - if PL-RS is maintained, use legacy Rel-17 requirements. If PL-RS is not maintained, add further SSB timing and frequency tracking.

Issue 2-1-6: Applicability of L1-RSRP measurement when RTD>CP in FR1

It is the common understanding of measurement multiple TRP (maximum is 2 depends on RAN1 feature) of RTD>CP is only for the same carrier.

Issue 2-1-7: Measurement restriction of L1-RSRP measurement when RTD>CP in FR1

< Agreement >

- If UE not support twoTA or UE support twoTAs but not support RTD>CP, the same as legacy.

- if the two TA is supported and UE supported RTD>CP,

  - For serving cell SSB L1-RSRP, when [the SSB for L1-RSRP measurement and CSI-RS for RLM, BFD, CBD or L1-RSRP measurement are overlapping or partially overlapping in OFDM symbols]

      - SSB and CSI-RS: same SCS, no restriction;

      - SSB and CSI-RS: different SCS,

        - With simultaneousRxDataSSB-DiffNumerology, no restriction

        - No simultaneousRxDataSSB-DiffNumerology, with restriction to measure just one

  - For a cell with different PCI from serving cell, when [the SSB for L1-RSRP measurement and SSB transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement are overlapping or partially overlapping in OFDM symbols],

      - no restriction

  - For cell with different PCI from serving cell, when [the SSB for L1-RSRP measurement and CSI-RS transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement are overlapping or partially overlapping in OFDM symbols],

      - SSB and CSI-RS: same SCS, no restriction;

      - SSB and CSI-RS: different SCS,

        - With simultaneousRxDataSSB-DiffNumerology, no restriction

        - No simultaneousRxDataSSB-DiffNumerology, with restriction to measure just one

Issue 2-1-8: Scheduling restriction of L1-RSRP measurement when RTD>CP in FR1

<Agreement>:

For serving cell or cell with different PCI from serving cell:

- If UE not support twoTA or UE support twoTAs but not support RTD>CP, the same as legacy.

- If two TA and RTD>CP is supported:

  - Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1：no restriction

  - Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing as PDSCH/PDCCH on FR1：

    - Option 1:

      - Support simultaneousRxDataSSB-DiffNumerology: no restriction

      - not support simultaneousRxDataSSB-DiffNumerology

        - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols and 1 symbol before and after symbols corresponding to the SSB indexes configured for L1-RSRP measurement

    - Option 2:

        - no restriction

    - Option 3:

      - Support simultaneousRxDataSSB-DiffNumerology: no restriction

      - not support simultaneousRxDataSSB-DiffNumerology

        - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols which are overlapped or partially overlapped with SSB symbols configured for L1-RSRP measurement

  - For cell with different PCI from serving cell, Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1

    - Option 1:

        - The UE is not expected to transmit PUCCH/PUSCH/SRS on the same symbols and 1 symbol before or after the OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, where the transmission of PUCCH/PUSCH/SRS may be on serving cell(s) and cell(s) with PCI different from serving cell(s), and restricted symbols may partially or fully overlap with UL symbols

    - Option 2:

        - no restriction

    - Option 3:

        - The UE is not expected to transmit PUCCH/PUSCH/SRS on the symbols configured for L1-RSRP measurement which are overlapped or partially overlapped with SSB symbols configured for L1-RSRP measurement.

Issue 2-1-9: when RTD>CP in FR1, update of other requirements as RLM/BFD/…?

<Way forward>

-	Option 1: RAN4 to discuss and confirm whether other RRM requirements can be applicable, if needed, how to clarify the side condition of RTD>CP

Issue 2-1-12: Add clarification of joint TCI state activation delay in sDCI?

<Way forward>

-	Option 1:

- UE is not expected to receive or transmit on the target TCI state before UE completes the DL and UL TCI state switch of both TRPs

Issue 2-1-13: Add scheduling restriction for dual TCI state activation delay in mDCI

<Way forward>

-	Option 1:

- before UE finish two TCI state activations, UE is not expected to receive or transmit data in conflicting RS which is used for T/F tracking or pathloss calculation.

<Topic 3 RRM performance part >

Issue 3-1-1: Whether to define TDCP accuracy requirement/test case for TDCP?

< Agreement>:

Define test cases for TDCP under certain conditions, and the conditions can be discussed in Issue 3-1-3.

Issue 3-1-2: Whether to define additional criteria for TDCP?

< Agreement>:

Not to introduce additional L1-SINR or L1-RSRP reporting along with TDCP reporting in Rel-18.

Issue 3-1-3: Design for the test case of TDCP

< Agreement>:

  - Test metric:

    - Step 1: Determine the configuration (e.g., SNR, doppler spread) for the test.

    - Step 2: for the configuration agreed in step 1, determine the TDCP accuracy range (i.e., TDCP accuracy value X1 to X2) from the simulation results from different companies.

    - Step 3: Define the test case for the configurations agreed in step 1 and the TDCP value UE should report to pass the test is determined from the range of values determined in step 2 (i.e., X1 to X2). [For the repeated tests UE should pass the test at least [Y2] % of the times. Y2 can be FFS.]

      - The text in [] is the baseline, and other proposal is not precluded.

  - Test Parameters:

    - Channel model: fading channel

      - As baseline, TDL-A30

    - Pick one value from SNR = 10dB, 15dB, 20dB

    - The distance between two TRSs: 1 slot as baseline, further discuss on 10 slots

Issue 3-1-4: TDCP Measurement Report Mapping – amplitude

< Agreement>:

Define TDCP mapping table of amplitude in section 10 of RRM spec.

- Option 1:

| ki | TDCP Range |
| --- | --- |
| 0 | 0.9945< TDCP <=1 |
| 1 | 0.9922< TDCP <=0.9945 |
| 2 | 0.9890< TDCP <=0.9922 |
| 3 | 0.9844< TDCP <=0.9890 |
| … | … |
| 12 | 0.6464< TDCP <=0.75 |
| 13 | 0.5< TDCP <=0.6464 |
| 14 | 0.2929< TDCP <=0.5 |
| 15 | 0≤ TDCP <=0.2929 |

- Option 2:

- use the RAN1 points as the middle (but not the centre due to non-uniform) of each range as

| Estimated TDCP value | Report index |
| --- | --- |
| 0.9953≤Estimated TDCP≤1 | 0 |
| 0.99335<Estimated TDCP<0.9953 | 1 |
| … | … |
| 0≤Estimated TDCP<0.39645 | 15 |

- Other options are not precluded

Issue 3-1-5: TDCP Measurement Report Mapping - phase

< Agreement>:

Define TDCP mapping table of phase in section 10 of RRM spec.

- Option 1:

- For mapping table of TDCP phase reporting, the 16 points divided [0 2pi] to 16 segments. the mapping table can use ej2πci16 as the boundary of each range.

- Option 2:

- the RAN1 points are in the middle of each range

- Other options are not precluded

Issue 3-2-1: Whether to define TCs for two TAs?

< Agreement>:

Define TC for two TAs

FFS: both of UE uplink transmit timing and UE timing advance adjustment accuracy or just one of them.

Issue 3-2-2: Whether to define TCs for m-DCI mTRP cases?

<Way forward>

- Option 1: Define TC for m-DCI mTRP cases

- Option 2: Do not define TC for m-DCI mTRP cases

Issue 3-2-3: Whether to define TCs for s-DCI mTRP cases?

< Agreement>:

- Define TCs for s-DCI mTRP cases:

  - Including separate DL TCI state switch and UL TCI state switch.

  - All are known TCI state

  - Joint TCI state switch. Note: further check of simultaneous reception of the TC.

  - Dual TCI state. Note: further check the testability