3GPP TSG-RAN WG4 Meeting # 110						      					            R4-2403446

Athens, Greece, February 26 - March 1, 2024

Agenda item:			8.12.4

Source:	Samsung

Title:	Ad-hoc minutes on Reply LS

Document for:	Approval

# Introduction

xxx

# Topic #1: Reply LS to RAN5

## Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2401899 | Discussion on the necessity of defining the missing testing parameter for PC6 and reply LS | Samsung |  |
| R4-2402570 | On LS Reply to RAN5 on Missing Parameters | Nokia, Nokia Shanghai Bell |  |
| R4-2402571 | CR to 38.133 CatF R17 on PC1,5,6 RRM Parameters for RAN5 | Nokia, Nokia Shanghai Bell |  |
| R4-2402572 | CR to 38.133 CatA R18 on PC1,5,6 RRM Parameters for RAN5 | Nokia, Nokia Shanghai Bell |  |
| R4-2401607 | Missing Parameters for FR2 RRM Testing of Different Power Classes | Qualcomm Incorporated |  |

## Open issues summary

### Sub-topic 2-1

Issue 2-1-1: RAN4 to clarify whether requested missing parameters need to be added from Rel-17 version of TS 38.133？

- Proposals

  - Option 1: Yes

  - Option 2: No

- Discussion:

  - R&S: In RAN5, there are test cases for PC1 with missing parameters, so the change to PC1 should be from R15.

  - Nokia: for PC6, introduced from R17.

  - Nokia RAN5: RAN5 just use the lastest spec from RAN4.

- Agreement:

  - RAN4 shall introduce the requested missing parameters for different PCs from different releases:

    - PC1: from Rel-15

    - PC5: from Rel-17 (and release independent to early release)

    - PC6: from Rel-17

Issue 2-1-2: RAN4 to discuss whether the missing parameters to be considered are release independent or not?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - N/A

Issue 2-1-3: do you agree to exclude 90 degrees relative offset between active probes in Table A.3.15.3-1 for PC6 UEs?

- Proposals

  - Option 1: Yes

  - Option 2: No

- Discussion:

  - QC: In Rel-17, for uni-directional deployment, what we used is 30 degrees relative angle between AoAs. Maybe not preclude other options, but we just 30degree

  - Nokia: 150 degree is used for measurement reporting

  - QC: not needed for drafting the reply LS

- Agreement:

  - For PC6, the relative offset between active probes in Table A.3.15.3-1:

    - FFS to eliminate the option of 90 degrees

Issue 2-1-4: For HST FR2 PC6 test case parameters, which relative angular offset between active probes in Setup 3 according to clause A.3.15.3 are applicable?

- Proposals

  - Option 1: AWGN for AoA1, and AWGN with 19444 Hz for AoA2

  - Option 2: AWGN for AoA1, and AWGN with 9722 Hz for AoA2

- Discussion:

  - Chair: not needed for drafting the reply LS

Issue 2-1-5: For PC5, do you agree to define angular offset between active probes as for PC1, e.g., 30°, 60°, 90° and 120 degrees

- Proposals

  - Option 1: yes

  - Option 2: no

- Discussion:

  - QC: no need for 120, and maybe not needed for 90

  - Nokia: keep the same as PC1

- Agreement:

  - For PC5, the relative offset between active probes in Table A.3.15.3-1:

    - 30°, 60°, 90° and 120 degrees

Issue 2-1-6: How to consider Gain difference Y between fine and rough beams at Rx beam peak direction?

- Proposals

  - Option 1: Nokia

use 12dB gain difference between fine and rough beams as a starting point for the values of Y for PC 1, 5, and 6 UEs.

  - Option 2: Qualcomm

| Value “Y” in dB, for each UE power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 7 |
| 18 (10*log(64)) | 9.0 | 7.0 | FFS | 15.5 (10*log(36)) | FFS |

- Discussion for PC1/5:

  - Chair: for PC1 value from QC, it means 64 elements panel and 1 element for rough beam

  - QC: for PC1, it can still have similar coverage as PC3.

  - R&S: have these number checked with design?

- Discussion for PC6:

  - QC: use PC5 as starting point

  - Nokia: 4x4 is assumed for PC6

- Agreement:

|  | Value “Y” in dB, for each UE power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| [18] | 9.0 | 7.0 | FFS | [15.5] | [15.5] | FFS |

Issue 2-1-7: How to consider Gain difference Z between fine and rough beams at Spherical coverage directions?

- Proposals

  - Option 1: Nokia

use 12bB gain difference between fine and rough beams as a starting point for the values of Z for PC 1, 5, and 6 UEs.

  - Option 2: Qualcomm

| Value “Z” in dB, for each UE power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 7 |
| FFS (10*log(64)) | 9.0 | 7.0 | FFS | 15.5 (10*log(36)) | FFS |

  - Option 3: Samsung

All the Rel-17 FR2 HST TCs are irrelevant to gain difference Z, and there is no need to define the gain difference Z for PC6.

- Recommended WF

- Agreement:

|  | Value “Z” in dB, for each UE power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| [18] | 9.0 | 7.0 | FFS | [15.5] | [15.5] | FFS |

Issue 2-1-8: How to consider SSB_RP side condition?

- Proposals

  - Option 1: Nokia

| Parameter | Angle of arrival | NR operating bands |  |  | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 1, 2, 3, 4, 5, 6 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.4+Y5 | -123.4+ Y6 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.6+Y5 | -123.6+ Y6 |  |  |
|  |  | n259 |  |  | -105.5 |  | -117.5+Y5 |  |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  | -123.4+ Y6 |  |  |
|  |  | n262 | -120.3+Y1 | -105.6 | -103.6 | -118.8+Y4 |  |  |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.4+Z5 | -110.5+Z6 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.6+Z5 | -110.5+Z6 |  |  |
|  |  | n259 |  |  | -92.7 |  |  |  |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  | -110.5+Z6 |  |  |
|  |  | n262 | -112.3+Z1 | -93.7 | -90.5 | -106.7+Z4 |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |

  - Option 2: Samsung

    - Suggest RAN4 to discuss the definition of minimum SSB_RP side condition for Rx beam peak if the necessity of defining Y is confirmed.

    - Suggest not to define minimum SSB_RP side condition for spherical coverage

- Agreement:

  - For SSB_RP side condition, add new column for PC6.

    - FFS the value during this meeting.

Issue 2-1-9: whether there is a need to define the CSI-RS_RP (Section B.2.4.2) for PC6

- Proposals

  - Option 1: No, current PC6 UE test cases in RAN4 are defined for SSB-based measurements only.

  - Option 2: other, please specify.

- Discussion:

  - Chair: Based on Rel-17/18 test cases, CSI-RS_RP is not used in all relevant test cases.

Issue 2-1-10: whether the values of Minimum SSB_RP in Table B.2.2-2: Conditions for intra-frequency measurements in FR2 also need to be defined for PC6 devices.

- Proposals

  - Option 1: Yes

  - Option 2: No

  - Option 3: check with RAN5

- Agreement:

  - FFS the necessity of Minimum SSB_RP in Table B.2.2-2:

    - If needed, companies prepare CR to complete Minimum SSB_RP in Table B.2.2-2 in April/May meetings.

Issue 2-1-11: UE gain G

- Proposals

  - Option 1: Qualcomm

UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 7 |
| Minimum, dBi | 0 | FFS | -10 | FFS | -5 | FFS |
| Maximum, dBi | 57 | FFS | +20 | FFS | 57 | FFS |

  - Option 2: Samsung

There is no need to define the UE gain (B 2.1.5 and B 2.1.6) for PC6.

  - Option 2: Nokia

UE gain is needed for PC6. Exact value FFS.

- Agreement:

  - FFS PC6 shall have the same UE gain as PC5

  - FFS the value of UE gain for PC1, 5, and 6

Issue 2-1-12: UE gain difference between inter-frequencies Ginter

- Proposals

  - Option 1: Qualcomm

UE gain difference between inter-frequencies Ginter

|  | UE Power class |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 7 |
| Maximum difference, dB | 3 | FFS | 3 | 3 | FFS |

  - Option 2: Nokia

Check with RAN5 whether UE gain difference between inter-frequencies Ginter needs to be defined in Rel-17 TSs.

- Agreement:

  - For UE gain difference between inter-frequencies Ginter

    - [3dB] for PC1, PC5 and PC6

Issue 2-1-13: the values of Rough Beam gain reduction “D”

- Proposals

  - Option 1: check with RAN5 whether the values of Rough Beam gain reduction “D” needs to be defined in RAN4 in B.2.1.5.

  - Option 2: other，please specify

- Agreement:

  - RAN4 continue to discuss on the values of rough beam gain reduction “D”