3GPP TSG-RAN WG4 Meeting #110 			                						             R4-2403456

Athens, Greece, February 26 – March 1, 2024

Agenda item:			8.16.3

Source:	Apple

Title:	WF on R18 Further NR mobility enhancement – part 2

Document for:	Approval

# Core

Issue 1-1: UE feature list of R18 Further NR mobility enhancement part 2

Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-x1 | Measurement validation based on EMR measurement during connection setup/resume | Indicate UE supporting measurement validation based on EMR measurement during IDLE/INACTIVE state and reporting for fast CA/DC setup | idleInactiveNR-MeasReport-r16 | Yes | N/A | UE does not support EMR measurement validation during connection setup/resume and reporting for fast CA/DC | Per-UE | No | FFS | N/A |  | Optional with capability signaling |
|  | 39-x2 | Measurement validation based on non-EMR measurement during connection setup/resume | Indicate UE supporting measurement validation based on non-EMR measurement during IDLE/INACTIVE state and reporting for fast CA/DC setup |  | Yes | N/A | UE does not support non-EMR measurement validation during connection setup/resume and reporting for fast CA/DC | Per-UE | No | FFS | N/A |  | Optional with capability signaling |

Issue 1-2: applicability of validity check

FFS the following solution:

- The validity check is not applicable to

  - Any measurements during IDLE/INACTIVE but no included in configuration of R18 CA/DC setup improvement.

  - Any measurements during CONNECTED mode.

- Support of R18 CA/DC setup improvement shall have no impact on R16 EMR reporting behaviour.

# Performance

Issue 2-1: test case list

Agreement:

| No. | Category | Configuration | Responsible company |
| --- | --- | --- | --- |
|  | NR-DC with selective activation of cell groups Note: For UE capable of both FR1-FR1 DC and FR1-FR2 DC, UE only needs to pass either 1)+2) or 3)+4) | Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC | Apple |
|  |  | Inter-frequency CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC | ZTE |
|  |  | Intra-frequency CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC | CATT |
|  |  | Inter-frequency CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC | CMCC |
|  | FFS: Improvement on SCell/SCG setup delay | TBA |  |
|  | CHO including target MCG and target SCG in NR-DC (obj.3) Note: For UE capable of both FR1-FR1 DC and FR1-FR2 DC, UE only needs to pass either 6) or 7) | FR1-FR1 NR-DC to FR1-FR1 NR-DC | Huawei |
|  |  | FR1-FR2 NR-DC to FR1-FR2 NR-DC | vivo |
|  | CHO including target MCG and candidate SCG in NR-DC (obj.4) Note: For UE capable of both FR1-FR1 DC and FR1-FR2 DC, UE only needs to pass either 8) or 9) | FR1-FR1 NR-DC to FR1-FR1 NR-DC | Nokia |
|  |  | FR1-FR2 NR-DC to FR1-FR2 NR-DC | OPPO |

Issue 2-2: whether test case for CHO-only is needed

Agreement:

- Introduce one single new test case for CHO-only, selected from existing CHO test case.

- UE capable of R18 enhanced CHO configuration only needs to pass the new R18 test case, i.e. legacy CHO test case in R16 can be skip.

Issue 2-3: Whether define test case for Improvement on SCell/SCG setup delay

FFS the following options:

- Option 1: do not define test case (QC)

  - Msg1 transmission occasion is unknown to TE. RSRP variation from X sec before T1 based test design is not definable.

  - If test is for validating UE not to report, TE does not differentiate wrong UE who is doing nothing and good UE who is performing validity check. Both good and wrong UE will pass by not reporting.

- Option 2: define test case

# Annex

Test procedure (for information only).

- Subsequent CPA

- Subsequent CPC

- CHO including target MCG and target SCG in NR-DC (obj.3)

- CHO including target MCG and candidate SCG in NR-DC (obj.4)