3GPP TSG-RAN WG4 Meeting # 110											             R4-2403475

Xiamen, China, October 9 – October 13, 2023

Agenda item:			8.14.4

Source:	Huawei, HiSilicon

Title:	WF on R18 NR positioning – LPHAP

Document for:	Approval

- Sub-Topic #1: Core requirements maintenance

Issue 1-1-1: Start of PRS measurement

- Agreement (from adhoc session)

  - For Case 1, PRS measurement start is not limited to PTW when the PRS resource indicated in the assistance data is not within PTW.

Issue 1-1-2: PRS and (e)DRX alignment

- Conclusion

  - The issue is merged into Issue 1-1-1, no further discussion needed.

Issue 1-1-3: Multiple times of autonomous TA adjustments

- Agreement

  - Negative TA value is not expected for one-shot autonomous TA adjustment within the positioning validity area

Issue 1-1-4: Clarification for autonomous TA adjustment

- Conclusion

  - Companies can bring clarification for autonomous TA adjustment in contribution driven manner.

Issue 1-1-5: RRM measurement for Case 2

- Conclusion

  - RAN4 already defined RRM requirements for Case 2. Companies can bring discussions, if any, on whether additional requirements are needed.

# Sub-Topic #2: Performance requirements

Issue 1-2-1: Accuracy requirements

- Agreements (from adhoc session)

  - Existing accuracy requirements are applicable for PRS measurement in INACTIVE with eDRX, and PRS measurement in IDLE.

Issue 1-2-2: RRM testing for PRS measurement

Question 1: whether to define test cases for measurement accuracy

- Wayforward

  - FFS whether to define test cases for measurement accuracy for LPHAP.

Question 2: for which measurement types to define RRC test cases

- Wayforward

  - Option 1:

    - Define test cases for RSTD, PRS RSRP and PRS RSRPP measurement in RRC_IDLE state and for UE Rx-Tx time difference in RRC_INACTIVE state.

  - Option 2:

    - Define test cases for all applicable measurement in RRC_IDLE state and RRC_INACTIVE state, respectively.

Question 3: what eDRX configuations are to be tested

- Background

  - For RAN eDRX > 10.24s, we have 2 cases for PRS measurement

    - Case 1: eDRX cycle is smaller or equal to configured PRS measurement reporting periodicity. In this case, UE starts positioning measurement within PTW.

    - Case 2: eDRX cycle is longer than configured PRS measurement reporting periodicity. In this case, positioning measurement start is not limited to PTW.

- Wayforward

  - Option 1:

    - Both RAN eDRX ≤ 10.24s and RAN eDRX ＞ 10.24s

    - For RAN eDRX <= 10.24s, both CN eDRX <= 10.24 s and CN eDRX >10.24 s

  - Option 2:

    - Case 2 for RAN eDRX ＞ 10.24

  - Option 3:

    - Both Case 1 and Case 2 for RAN eDRX ＞ 10.24

Question 4: what PRS measurement features are to be tested

- Wayforward

  - Option 1:

    - Reduced number of PRS samples

    - parallelPRS-MeasRRC-Inactive-r17 capability

    - Lower Rx beam sweeping in FR2

  - Option 2:

    - 4 sample baseline

Issue 1-2-3: RRM testing for other requirements

REQ 1: RRM measurement (cell reselection)

- Background

  - For RAN eDRX > 10.24s, we have we have 2 cases for RRM measurement

    - Case 1: eDRX cycle is smaller or equal to configured PRS measurement reporting periodicity (if PRS measurement is configured) and configured SRS transmission periodicity. In this case, RRM measurements for positioning needs are performed within PTW, and requirements are reused from eRedCap WI.

    - Case 2: eDRX cycle is larger than configured PRS measurement reporting periodicity (if PRS measurement is configured) or configured SRS transmission periodicity (if SRS transmission for positioning is configured). In this case, RRM measurements for positioning needs are not limited to PTW, and new requirements are defined.

- Agreement

  - Define test cases for RRM measurement (cell reselection) in Case 2

REQ 2: Transmit timing

- Wayforward

  - Option 1:

    - Test UE transmit timing for positioning measurements

  - Option 2:

    - Test UE autonomous TA adjustment

REQ 3: TA validation

- Wayforward

  - FFS whether to define test cases for TA validation

Issue 1-2-4: UE types to be tested

- Wayforward

  - Option 1:

    - Define separate sets of TCs for normal (non-RedCap) and RedCap type of UEs.

  - Option 2:

    - Use same set of TCs for normal (non-RedCap) and RedCap type of UEs.

Issue 1-2-5: Test case list

- Conclusion

  - Test case list will be defined in next meeting based on the outcome of open issues. Companies can bring proposals on the detailed test case list.