3GPP TSG-RAN4 Meeting#110	 R4-2403613

# Athens, Greece, 26 Feb- 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | ❌ | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 Cat F draft CR for 38.101-1 Add the missing harmonic mixing requirements for CA_n7-n77 with PC2 and PC1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, TELUS, Bell Mobility, Skyworks, CHTTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_FR1_TDD_NR_CADC_SUL_R18-Core |  |  |  |  |  | Date: |  |  | 2024-01-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Harmonic mixing (UL2/DL3) requirements are missing for CA_n7-n77 with PC2/ PC1.5 on n77, while according to the following agreement captured in latest TR 38.846 it should be specified. |  |  |  |  |  |  |  |  |
| , |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing harmonic mixing requirements for CA_n7-n77 with PC2/PC.15 on n77 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Harmonic mixing requirements are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 7.3A.4-4a, Table 7.3A.4-4b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### << Start of changes >>

Table 7.3A.4-4a: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n186 | 5 | 15 | 25 (RBstart=0) | 5 | 27.3 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 (RBstart=0) | 5 | 27.3 | NOTE 7 | UL1/DL3 |
| n77 | n2 | 10 | 15 | 25 (RBstart=0) | 5 | 9.1 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 20 | 15 | 100 (RBstart=0) | 20 | 6.7 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 1 | NOTE 4 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n5 | 20 | 15 | 20 (RBstart=0) | 20 | 4.3 | NOTE 5 | UL1/DL4 |
| n77 | n7 | 20 | 30 | 50 (RBstart=0) | 10 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n7 | 20 | 30 | 50 (RBstart=0) | 50 | 8.8 | NOTE 3 | UL2/DL3 |
| n77 | n8 | 10 | 15 | 25 (Rbstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n8 | 20 | 15 | 20 (Rbstart=0) | 20 | 4.3 | NOTE 5 | UL1/DL4 |
| n77 | n12 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n12 | 15 | 15 | 75 (RBstart=0) | 15 | 29.2 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 (RBstart=0) | 5 | 9.2 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 20 | 15 | 100 (RBstart=0) | 40 | 2.0 | NOTE 4 | UL1/DL2 |
| n77 | n28 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 30 | 15 | 160 (RBstart=0) | 30 | 11.7 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 (RBstart=0) | 5 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 25 (RBstart=0) | 10 | 10.6 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 4.4 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 10 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 100 | 8.8 | NOTE 3 | UL2/DL3 |
| n78 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 1 | NOTE 4 | UL1/DL2 |
| n78 | n5 | 10 | 15 | 25 (RBstart=0) | 5 | [8.1] | NOTE 5 | UL1/DL4 |
| n78 | n5 | 20 | 15 | 20 (RBstart=0) | 20 | [4.3] | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 20 | 15 | 20 (RBstart=0) | 20 | 4.3 | NOTE 5 | UL1/DL4 |
| n78 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 30 | 15 | 160 (RBstart=0) | 30 | 11.7 | NOTE 1 | UL1/DL5 |
| n79 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 28.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 6: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4b: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n2 | 10 | 15 | 25 (RBstart=0) | 5 | 11.8 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 20 | 15 | 100 (RBstart=0) | 20 | 9.2 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 11.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 4 | NOTE 4 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 (RBstart=0) | 5 | 10.7 | NOTE 5 | UL1/DL4 |
| n77 | n5 | 20 | 15 | 20 (RBstart=0) | 20 | 6.4 | NOTE 5 | UL1/DL4 |
| n77 | n7 | 20 | 30 | 50 (RBstart=0) | 10 | 16.1 | NOTE 3 | UL2/DL3 |
| n77 | n7 | 20 | 30 | 50 (RBstart=0) | 100 | 11.5 | NOTE 3 | UL2/DL3 |
| n77 | n12 | 10 | 15 | 25 (RBstart=0) | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n12 | 15 | 15 | 75 (RBstart=0) | 15 | 32.2 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 (RBstart=0) | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 50 (RBstart=0) | 10 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 (RBstart=0) | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 50 (RBstart=0) | 10 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 (RBstart=0) | 5 | 11.9 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 20 | 15 | 100 (RBstart=0) | 40 | 3.3 | NOTE 4 | UL1/DL2 |
| n77 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 30 | 15 | 160 (RBstart=0) | 30 | 14.7 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 (RBstart=0) | 5 | 37 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 50 (RBstart=0) | 10 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 (RBstart=0) | 5 | 16.1 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 25 (RBstart=0) | 10 | 13.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 10 | 16.1 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 100 | 11.5 | NOTE 3 | UL2/DL3 |
| n78 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 11.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 4 | NOTE 4 | UL1/DL2 |
| n78 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 30 | 15 | 160 (Rbstart=0) | 30 | 14.7 | NOTE 1 | UL1/DL5 |
| n79 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 31.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

### << End of changes >>