3GPP TSG-RAN WG4 Meeting #110    	R4-2403628

Athens, Greece, 26 February – 1 March 2023

Title:	WF on PC2 HPUE MSD

Agenda item:	7.19.2

Source:	Skyworks Solutions Inc., Huawei, Qualcomm Inc.

Document for:	Approval

# 1	Introduction

The reference sensitivity (REFSENS) power level requirements for intra-band CA are currently specified only for power class 3 (PC3). Requests for FDD PC2 operation require new REFSENS tables. This WF captures a framework on how to capture the PC2 FDD REFSENS following initial proposals in [1,2] for intra-band CA. This WF also captures agreements to simplify PC2 FDD 1UL MSD tables for inter-band CA.

- On parameter ΔRIBNC,  ΔRIBC

# 2.1 	Background

Intra-band CA REFSENS requirements are specified as a REFSENS relaxation using (Figure 1):

- ΔRIBC for intra-band contiguous CA,

- ΔRIBNC for intra-band non-contiguous CA.

Figure 1: ΔRIBNC, ΔRIBC definition – Clause 3.2

- These parameters were defined at a stage where FDD bands supported only PC3. With the introduction of PC2 and in anticipation of future HPUE power classes, it is necessary to clarify to which REFSENS level the relaxation ΔRIBNC, ΔRIBC refer to. This WF clarifies that the PC2 relaxation is specified relative to the PC3 REFSENS level.

## 2.2	Way Forward on ΔRIBNC,  ΔRIBC

WF1 on ΔRIBNC,  ΔRIBC: Adopt the changes highlighted in green below.

In Clause 3.2,

ΔRIBC 	Allowed relaxation to the power class 3 reference sensitivity level due to support for intra-band contiguous CA operation.

ΔRIBNC 	Allowed relaxation to the power class 3 reference sensitivity level due to support for intra-band non-contiguous CA operation.

In Clause 7.3A.2.1 (Reference sensitivity power level for Intra-band contiguous CA),

For aggregation of two or more downlink FDD carriers with two uplink carriers, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.1-1 and the power class 3 reference sensitivity power level increased by ΔRIBC.

In Clause 7.3A.2.2 (Reference sensitivity power level for Intra-band non-contiguous CA),

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, and Table 7.3A.2.2-1 with the power class 3 reference sensitivity power level increased by ΔRIBNC given in Table 7.3A.2.2-1 for the SCC(s).

and

For intra-band non-contiguous carrier aggregation with two uplink carriers and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, and Table 7.3A.2.2-2 with the power class 3 reference sensitivity power level increased by ΔRIBNC given in Table 7.3A.2.2-2 for the PCC and SCC(s).

- On REFSENS tables for PC2 Intra-Band REFSENS

# 3.1 	Background

New tables are needed to capture the PC2 intra-band CA REFSENS requirements, and three options are considered:

Option 1: For intra-band CA, capture REFSENS requirements using 3 tables and new suffix “a,b,c”:

- Use suffix “a” to capture PC3 REFSENS, e.g. Table 7.3A.2.1-1a for contiguous CA

- Use suffix “b” to capture 1Tx PC2 REFSENS, e.g. Table 7.3A.2.1-1b for contiguous CA

- Use suffix “c” to capture TxD PC2 REFSENS, e.g. Table 7.3A.2.1-1c for contiguous CA

Option 2: For intra-band CA, capture REFSENS requirements using 2 tables and new suffix “a,b”:

- Use suffix “a” to capture PC3 REFSENS, e.g. Table 7.3A.2.1-1a for contiguous CA

- Use suffix “b” to capture the worst-case PC2 REFSENS, e.g. Table 7.3A.2.1-1b for contiguous CA,

Option 3: For intra-band CA, capture the PC2 REFSENS requirements using only 1 table:

- Add suffix “a” to the PC3 table number to capture both the 1Tx and the 2Tx PC2 REFSENS, e.g. Table 7.3A.2.1-1a for contiguous CA. A footnote is used to distinguish the MSD for 1Tx vs 2Tx.

Each option has pro & cons:

- Option 1 captures the difference between 1Tx and 2Tx performance, but since many band combinations are introduced with PC2 FDD, the impact on TS complexity and TS maintenance is important.

- Option 2 minimizes the impact on TS complexity and maintenance, but it does not allow to distinguish 1Tx from 2Tx performance.

- Option 3: Captures both 1Tx and 2Tx performance, and minimizes the impact on the number of tables, but comes at the expense of footnotes.

As a compromise between option 1 and option 2, this WF proposes to adopt option 3.

## 3.2	Way Forward on Intra-Band REFSENS tables

WF2 on intra-band CA REFSENS tables:

For intra-band CA, capture the 1Tx and the 2Tx FDD PC2 REFSENS requirements into a new, single table by adding the suffix “a” to the PC3 Table number.

The 1Tx vs 2Tx MSD test points are distinguished using footnotes.

Table 1 below shows an example of table template for the example of intra-band non-contiguous CA.

Table 1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity for PC2 in FDD bands -Table 7.3A.2.2-1a

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNCX (dB) | ΔRIBNCY (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 20MHz + 5MHz | Wgap = 10.0 | 20 (RBstart = 0) | [FFS] | [FFS] | FDD |
| NOTE X: Applicable to UE supporting PC2 with single Tx. NOTE Y: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |

In TPs for TR, the PC2 intra-band CA REFSENS should be studied for both 1Tx and 2Tx architectures.

- On REFSENS tables for PC2 Inter-Band REFSENS

# 4.1 	Background

For inter-band CA, two tables have been introduced for PC2 MSD due to UL harmonics for CA_n3-n78:

- one table to capture the 1x26dBm PC2 REFSENS/MSD (Table 7.3A.4-2a)

- one table to capture the TxD 23+23dBm PC2 REFSENS/MSD (Table 7.3A.4-2b).

With the increasing number of inter-band combinations with PC2 FDD bands, it is necessary to ensure that the impact on the TS complexity is minimized. This WF proposes to port the WF2 on intra-band PC2 REFSENS to inter-band CA MSD tables.

It should be noted that for Rel-18, PC2 in an FDD band can only be supported as a single UL configuration for FDD-FDD or FDD-TDD inter-band CA combinations. This is due to 3Tx Rel-18 scope restricted to 2Tx operation on the TDD band.

Therefore, the scope of this WF is restricted to 1UL MSD tables, namely:

- MSD due to UL harmonics,

- MSD due to Rx harmonic mixing

- MSD due to Cross-band isolation.

## 4.2	Way Forward on Inter-Band REFSENS tables

WF3 on inter-band CA REFSENS tables:

For inter-band CA, capture the 1Tx and the 2Tx FDD PC2 REFSENS requirements into a single table for:

- MSD due to UL harmonics,

- MSD due to Rx harmonic mixing

- MSD due to cross-band isolation,

The PC2 FDD table number uses the PC3 Table number and suffix “a”.

The 1Tx vs 2Tx MSD test points are distinguished using footnotes.

For CA_n3-n78 MSD due to Harmonic interference UL2/DL1, merge Table 7.3A.4-2a and Table 7.3A.4-2b into Table 7.3A.4-2a and adopt footnotes “X” and “Y” to distinguish 1Tx vs 2Tx MSD.

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1. for UE not supporting Tx Diversity

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSDX | MSDY | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 27.1 | 32.3 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 16.6 | N/A | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. NOTE X: Applicable to UE supporting PC2 with single Tx. NOTE Y: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |  |  |

- On evaluating 2Tx MSD

# 5.1 	Background

Evaluation of the impact of 2Tx PA to PA reverse coupling is challenging to simulate or calculate. In many cases, measurements are needed to evaluate the impact of 2Tx architecture on MSD. TP for TRs are not complete without 2Tx MSD analysis. For 2Tx MSD analyses, MRC combining is severely impacted due to interference level being nearly equal on the primary and on the diversity antenna port. Hence, guidelines are needed to help proponents complete the MSD analyses for PC2 FDD bands.

## 5.2	Way Forward on evaluating 2Tx MSD

WF4: For PC2 FDD, interested companies are encouraged to bring guidelines to simplify 2Tx MSD analyses.

- References

- [1]		R4-2400365, PC2 CA_n71(2A) MSD, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions Inc.

- [2]		R4-2400366, PC2 CA_n71B BCS4-5 MSD, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions Inc.