3GPP TSG-RAN WG4 Meeting # 110	        R4-2403649

# Athens, Greece, Feb 26 – Mar 1, 2024

Agenda item:			8.18.9

Source:	ZTE Corporation

Title:	WF on NTN UE RF requirements

Document for:	Approval

- 1.Agreement reached in online discussion

Issue 2-1: The minimum peak EIRP and minimum output power for VSAT

Agreement:

- Define two sets of minimum peak EIRP requirements

  - Set 1 requirements: based on type 1/2 UE with GSO, which can cover type 4/5 UE with GSO

  - Set 2 requirements: for type 3 UE with LEO [600]km

    - For (type 3 UE) fixed VSAT supporting LEO only with electronical steering antenna, specify the minimum EIRP as [60, 62, or 67.6] dBm.

      - Further down-selection of the values of minimum EIRP

Agreement:

    - For (type 3 UE) fixed VSAT supporting LEO only with electronical steering antenna, specify the minimum EIRP as 61 dBm under the assumption of 600km orbit.

    - For  (type 1,2,4,5 UE),  specify the minimum EIRP as 70 dBm under the assumption of GSO.

- Open issues:

- Further discuss the minimum output power requirements

Issue 2-3: Transmitter spurious emission

Agreement:

- Further check if Prated,c,sys is based on TRP or EIRP.

  - If Prated,c,sys is based on TRP, go with proposal 3

  - If Prated,c,sys is based on EIRP, go with proposal 4

Agreement:

  - If Prated,c,sys is based on TRP and go with proposal 3:

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 4 kHz |

  - Further figure out the exact wording of Prated,c,sys for VSAT

Issue 2-6: the applicability of antenna type for GSO and LEO scenario

Agreement:

- Add the following definitions:

  - Co-polarized transmission: when the DUT transmission antenna polarization is aligned with test antenna polarization.

  - Cross-polarized transmission: when the DUT transmission antenna polarization is aligned with the tangent of the test antenna polarization.

Issue 2-7: feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40.NR_NTN_enh | 40-1 | VSAT UE type in NTN | Support of fixed or mobile VSAT (Very Small Aperture Terminal) UE type a) Type 1: a fixed VSAT, which can only be fixed. b) Type 2: a mobile VSAT, which is capable to move. A VSAT (Very Small Aperture Terminal) UE as defined in TS 38.101-5 shall indicate support of this capability with only one type. |  | Yes | N/A | The network doesn’t know the VSAT UE type. | Per UE | N/A | FR2 | N/A | Support receiving access control indication in system information The feature group is applied to FR2-NTN | Optional with capability signalling |
| 40.NR_NTN_enh | 40-2 | Beam steering | Support of beam steering capability - Type 1: Fully electronically-steered beam UEs - Type 2: Fully mechanically-steered beam UEs A VSAT (Very Small Aperture Terminal) UE as defined in TS 38.101-5 shall indicate support of this capability with only one type. |  | Yes | N/A | Beam steering is not supported. | Per UE | N/A | FR2 | N/A | The capability is not applicable for UE other than VSAT. The feature group is applied to FR2-NTN | Optional with capability signaling |

Issue 2-1: REFSENS requirements

Agreement:

- For minimum EIS requirement,Lower aperture size/low antenna gain compared with 60cm/39.7dBi simulation assumption could be considered for minimum EIS requirement.

  - FFS on the concrete values of requirements

Agreement:

- For the VSAT REFSENS requirement, single polarization shall be assumed as baseline.

- [If VSAT is capable with 2Rx diversity, the further delta_R [-2.5dB] with 2Rx could be added].

- Agreement:

- For the type 3, to specify the minimum EIS requirement as [-115.6dBm]for 50MHz and for the other channel bandwidth, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz under the assumption of 600km orbit.

- For the type 1,2,4,5, to specify the minimum EIS requirement as [-126.8dBm]for 50MHz and for the other channel bandwidth, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz under the assumption of GSO orbit.

Issue 2-2: Maximum input power for NTN VSAT

Agreement:

- Specify [-101]dBm as OTA maximum input level for (type 3 UE) fixed VSAT supporting LEO only with electronical steering antenna.

  - Take the proposal 2 method into consideration for the further evaluation to confirm the value above

  - Further study on the modulation order for maximum input level.

Agreement:

For type 3, for the maximum input power requirement, for Ka-band VSAT, only to define the requirement up to [16/64QAM].

- FFS on the coding rate when defining the FRC configuration for [16/64QAM] maximum input power.

- FFS on the value for maximum input power level.

- Note: for 64QAM modulation order, the lower orbits compared with 600KM might be considered .

For type 1/2 capable of connecting with both GSO and LEO, for the maximum input power requirement, for Ka-band VSAT, only to define the requirement up to [16/64QAM] under the LEO assumption as maximum input power

- FFS on the coding rate when defining the FRC configuration for [16/64QAM/] maximum input power.

- FFS on the value for maximum input power level.

For type 4/5 capable of connecting with GSO, for the maximum input power requirement, for Ka-band VSAT, only to define the requirement up to [16/64QAM/] under the GSO assumption as maximum input power

- FFS on the coding rate when defining the FRC configuration for [16/64QAM/] maximum input power.

- FFS on the value for maximum input power level.

Issue 2-7: Others

Agreement:

- Use ERC 74-01 as reference for band n512.