3GPP TSG-RAN4 Meeting #110  					    R4-2403658

Athens, GR, 26 Feb – 01 Mar, 2024

Source:	China Telecom

Title:	WF on MC UE RF requirements and feature list

Agenda item:			8.15.4

Document for:	Approval

Background

This WF capture the agreements regarding LS R4-2400022 and UE feature list.

The incoming LS R4-2400022 captures RAN2 agreement as shown below

For Rel-18 UL Tx switching, RAN2 achieved the following agreements in RAN2 #124 meeting: - (Agreement 1)RAN2 confirms that Rel-18 signalling can configure 2 bands UL Tx switching for a band pair that the UE supports according to the Rel-18 band pair list UE capability, in which case the network and UE assume the capability reported for R18 UL Tx switching is used. RAN2 respectfully asks RAN4 and RAN1 to take this into account, and feedback to RAN2 in case there is any concern. - (Agreement 2)RAN2 assumes that “if switching2T-Mode-r18 IE is configured for a band pair, then 2Tx-2Tx switching period of this band pair will be considered as the input for switching period calculation, for instance, when calculating “min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}” or “max(Tswitch_A-C,Tswitch_B-D ,Tswitch_A-D, Tswitch_B-C)” for switching across 4 bands. RAN2 respectfully asks RAN4 to take this into account, and feedback to RAN2 in case there is any concern.

Wayforward

Issue 1-1-1: R4-2400022 LS on UL Tx switching

Online Agreement:

- Extend the 3/4 band requirements to also cover 2 band fallback cases in Rel-18.

Issue 1-2-2: UE feature list

Online Agreement:

- Do not distinguish one TAG and two TAG

- Do not distinguish 3 band and 4 band cases

- Align the wording for 38-4 and 38-5 based on RAN4 agreements

Agreement:

The following feature groups are agreed to be captured in LS that to be sent to RAN2.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-1 | Switching period for dynamic UL Tx switching across up to 4 bands in case of inter-band CA, SUL up to two TAGs | UE to indicate support of dynamic UL Tx switching across up to 4 bands for inter-band UL CA, or SUL. switchingPeriodFor2T-r18 indicates the length of 2Tx-2Tx switching period. switchingPeriodFor1T-r18 indicates the length of 1Tx-2Tx switching and/or 1Tx-1Tx switching period, as specified in TS 38.101-1. n35us represents 35 us, n140us represents 140us, and n210us represents 210us, as specified in TS 38.101-1. |  | Yes |  | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination | Per BC, details are up to RAN2 | No need | Applicable only to FR1 |  |  | Optional with capability signaling |
| 38. NR_MC_enh | 38-2 | Application of DL interruptions due to dynamic UL Tx switching | 1. Capability to indicate that for the band where DL interruption is needed, the RRM interruption requirements defined in RAN4 shall be applied for duplex mode combinations except the combinations - SUL+TDD - TDD+TDD CA with the same UL-DL pattern | 38-1 | Yes | N/A | UL Tx switching where DL interruption is needed cannot be supported. | Per BC | No | FR1 only | Support mixture of FDD/TDD | Note: Field encoded as a bit map, where bit N is set to "1" if DL interruption on band N will occur during uplink Tx switching as specified in TS 38.133 [5]. The leading / leftmost bit (bit 0) corresponds to the first band of this band combination, the next bit corresponds to the second band of this band combination and so on. | Optional with capability signaling |
| 38. NR_MC_enh | 38-3 | Switching Period for unaffected Band for Dual UL | SwitchingPeriodUnaffectedBandDualUL indicate for a given band pair {band X and band Y}, whether/how the switching period is to be applied on band X, Y, Z, when a UL Tx switching is triggered from band pair {band X and band Z} to band pair {band Y and band Z}, as defined in 38.101-1. If absent for band Z, the UE is not required to transmit on any UL bands during the switching period reported for the band pair of band X and band Y, as defined in 38.101-1 - maintainedUL-Trans-r18 indicates that the UE is capable of uplink transmission on band Z and is not required to transmit on band X and Y during the switching period reported for the band pair of band X and band Y, as specified in 38.101-1. - periodOnULBands-r18 indicates the switching period to be applied on any UL bands as specified in 38.101-1. n35us represents 35 us, n140us represents 140us, and n210us represents 210us. - Band Z corresponds to the zth entry in the uplinkTxSwitchingPeriodUnaffectedBandDualUL-List-r18, which includes the UL band of this band combination excluding band X and band Y listed in the same order of the band combination. | 38-1 | Yes | N/A | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination. | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 3 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | 38-4 | [Additional switching Period for Dual UL] | [1. Indicate additionally the supported Tx switching period for switching between a band pair and another band pair or another band, when Rel-18 UL Tx switching is configured by uplinkTxSwitchingMoreBands-r18. If the capability is not reported, the switching period reported in switchingPeriodFor2T-r18 or switchingPeriodFor1T-r18 applies, as specified in TS 38.214 and TS 38.101-1.] | 38-1 | Yes | N/A | [UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination.] | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 1 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | 38-5 | [preferredBandPairs] | [1. Support the indication of UE’s preferred (switched-from, switched-to) band pairs for parallel UL transmission switching for a band combination consisting of four different bands.] | 38-1 | Yes | N/A | [Network can only assume the maximum switch period] | Per BC | No | FR1 only | Support mixture of FDD/TDD | [Note: Detailed information can refer to the LS to RAN2 in R4-2317609] | Optional with capability signalling |
| 38. NR_MC_enh | 38-6 | UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching among up to 4 bands | 1. Apply UL-MIMO coherence for the 2Tx-capable UL band(s). Rel-17 signalling on UL-MIMO coherence capability for 2Tx-2Tx switching is reused | 38-1, Rel-17 RAN4 UE FG 16-5, 16-6 | Yes | N/A | The existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable UL band(s) for Tx switching | Per BC | No | FR1 only | Support mixture of FDD/TDD | Note: Detailed information can refer to the LS to RAN2 in R4-2217741. | Optional with capability signalling |