3GPP TSG-RAN4 Meeting #110	R4-2403685

# Athens, Greek, 26th Feb. 2024 – 1st Mar. 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2068 | rev | 3 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR for 38.101-1 NonCol_IntraB_ENDC_NR_CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, Samsung, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To remove all [] around the temporary IE names based on RAN2’s agreements captured in R2-2313888. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To remove all [] around the temporaryIE names based on RAN2’s agreements captured in R2-2313888. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core part specification still keep all []. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.2, 7.10A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

5.5A.2	Configurations for intra-band non-contiguous CA

Table 5.5A.2-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink CA Configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  |  | See n1 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 55 | 4 and 5 |
| CA_n2(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  | - | See n2 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 40 | 4 and 5 |
| CA_n3(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20, 25, 30 |  |  | 60 | 1 |
|  |  | See n3 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 70 | 4 and 5 |
| CA_n5(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 25 | 0 |
| CA_n7(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  |  | See n7 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 65 | 4 and 5 |
| CA_n12(2A) | - | 5 | 5 |  |  | 10 | 0 |
| CA_n25(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 |  |  | 60 | 1 |
|  |  | See n25 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 60 | 4 and 5 |
| CA_n25(3A) | - | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 |  | 55 | 0 |
|  |  | See n25 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 55 | 4 and 5 |
| CA_n26(2A) | CA_n26(2A)7 | 5, 10, 15 | 5, 10, 15 |  |  | 30 | 0 |
| CA_n41(2A) | n413,4 CA_n41(2A) | 40, 50, 60, 80, 100 | 40, 50, 60, 80, 100 |  |  | 180 | 0 |
|  |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 190 | 1 |
|  |  | 10, 15, 20, 30, 40, 50, 60, 80, 90 | 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  | 190 | 2 |
|  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 190 | 3 |
|  |  | See n41 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 190 | 4 and 5 |
| CA_n41(3A) | n413,4 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 190 | 0 |
|  |  | See n41 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 190 | 4 and 5 |
| CA_n41(4A) | n413 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 190 | 0 |
|  |  | See n41 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 190 | 4 and 5 |
| CA_n46(2A) | - | 10, 20, 40, 60, 80 | 10, 20, 40, 60, 80 |  |  | 160 | 0 |
| CA_n48(2A) | - | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  | 1402 | 0 |
|  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 1402 | 1 |
|  |  | See n48 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 1402 | 4 and 5 |
| CA_n48(3A) | - | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 | 10, 15, 20, 40,50, 60, 80, 90, 100 |  | 1402 | 0 |
|  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1402 | 1 |
| CA_n48(4A) | - | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 1352 | 0 |
|  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1352 | 1 |
| CA_n66(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  |  | 60 | 0 |
|  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 |  |  | 80 | 1 |
|  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 |  |  | 80 | 2 |
|  |  | See n66 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 85 | 4 and 5 |
| CA_n66(3A) | - | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 |  | 80 | 0 |
| CA_n71(2A) | - | 5, 10, 15, 20 | 5,10,15, 20 |  |  | 30 | 0 |
|  |  | See n71 channel bandwidths in Table 5.3.5-1 for each carrier up to 25 MHz per carrier |  |  |  | 30 | 4 and 5 |
| CA_n77(2A)6 | n773,4 CA_n77(2A)3 | 20, 40, 80, 100 | 20, 40, 80, 100 |  |  | 200 | 0 |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 200 | 1 |
|  |  | See n77 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 200 | 4 and 5 |
| CA_n77(3A) | n773,4 CA_n77(2A)3 | 20, 40, 80, 100 | 20, 40, 80, 100 | 20, 40, 80, 100 |  | 300 | 0 |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 300 | 1 |
|  |  | See n77 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 300 | 4 and 5 |
| CA_n78(2A)6 | n783,4 CA_n78(2A)3 | 10, 20, 40, 50, 60, 80, 90, 100 | 10, 20, 40, 50, 60, 80, 90, 100 |  |  | 200 | 0 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  | 200 | 1 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 200 | 2 |
|  |  | See n78 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  | 200 | 4 and 5 |
| CA_n96(2A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 |  |  | 160 | 0 |
| CA_n96(3A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 |  | 240 | 0 |
| CA_n96(4A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 | 320 | 0 |
| CA_n102(2A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 |  |  | 160 | 0 |
| CA_n102(3A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 |  | 240 | 0 |
| CA_n102(4A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 | 320 | 0 |
| NOTE 1: Void. NOTE 2: Parameter value accounts for both, the maximum frequency range of band n48 (150 MHz), and the minimum frequency gaps in between NR non-contiguous component carriers. NOTE 3: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 4: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 5: Only single uplink carriers with power class other than PC3 are listed. NOTE 6: For a UE not indicating intraBandNR-CA-non-collocated-r18, the minimum requirements for intra-band non-contiguous CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided and UE is configured with maxMIMO-Layers with value less than or equal to 2, the power imbalance requirement defined in subclause 7.10A apply. For a UE indicating intraBandNR-CA-non-collocated-r18 and when nonCollocatedTypeNR-CA-r18 is provided, the minimum requirements for intra-band non-contiguous CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable intra-band non-contiguous NR CA configuration is a subset of a higher order NR CA configuration. NOTE 7: Unless otherwise stated, only RF requirements for dual PA architecture are applicable for UL CA_n26(2A) and UE shall indicate the dualPA-Architecture for UL CA_n26(2A). |  |  |  |  |  |  |  |

7.10A	Power imbalance for CA

7.10A.1	General

Power imbalance requirement is a measure of the receiver’s ability to receive a wanted signal in the presence of another signal with a power imbalance and a specific frequency offset from the wanted signal.

Power imbalance requirement in this subclause is only applicable for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided and UE is configured with maxMIMO-Layers with value less than or equal to 2.

7.10A.2	Minimum requirement

For the test parameters in table 7.10A.2-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1).

Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: The minimum requirements only apply for non simultaneous Rx/Tx between all carriers for TDD intra-band non-contiguous CA band combinations in Table 7.10A.2-2 |  |  |  |  |

For a UE indicating intraBandNR-CA-non-collocated-r18 for the following CA band combinations in Table 7.10A.2-2, the Rx requirements for two Rx ports are applicable for each component carrier if nonCollocatedTypeNR-CA-r18 is not provided and UE is configured with maxMIMO-Layers with value less than or equal to 2.

Table 7.10A.2-2: NR CA combinations

CA combination

CA_n77(2A)

CA_n78(2A)

<End of change>