# 3GPP TSG-RAN WG4 Meeting # 110                                                             R4-2403716

# Athens, Greece, Feb. 26th – Mar. 1st , 2024

# Source:	ZTE Corporation, Skyworks

# Title:	WF on CA_n40A-n41C

# Agenda Item:	7.1.1.1

# Document for:	Approval

# Introduction/background

For UL CA_n40A-n41C, it was already included in the specification TS38.101-1, which is:

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n41C | CA_n41C CA_n40A-n41A CA_n40A-n41C | n40 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |

Due to simultaneous Rx/Tx operation is supported between band n40 and band n41, the co-existence studied for the intra-band CA_n40C can be found in the following:

Table 5.x.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 0 | 190 | 2496 | 2690 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 2496 | 2506 | 2686 | 2690 | 2680 | 2500 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 10 | 190 | 5002 | 5370 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2306 | 2880 | 7498 | 8060 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 20 | 380 | 4802 | 5570 | 9994 | 10750 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2116 | 3070 | 7298 | 8260 | 12490 | 13440 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 30 | 570 | 4612 | 5760 | 9794 | 10950 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 1926 | 3260 | 7108 | 8450 | 12290 | 13640 |

It can be seen that 3rd order, 5th order and 7th order IMD caused by CA_n41C impact band n40 DL., which means the MSD values should be defined. But unfortunately, the MSD is missing in current specification. In this meeting, a TP[1] to TR38.718-02-01 is provided to analysis the MSD analysis for UL CA_n41C which would fall into band 40 DL Rx.

For the simultaneous Rx-Tx with PC3 CA_n40A-n41A, the cross band isolation MSD for victim band n40 is defined as:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 31.4 | ACLR2 |

In terms of the comments in the moderator ad hoc minute[2], the MSD caused by CA_n41C for PC3 UL CA_n40A-n41C should be further evaluation as companies think the MSD should higher than for n41A UL since for n41C the collision is within the ACLR1 range.

The RF parameters for the IMD MSD analysis refer to TR38.894. In addition, 0dB diplexer out of band isolation is assumed for the common antenna architecture according to [3].

In this WF, the RF parameters assumptions, IMD3 MSD test points, etc. are captured for PC3 UL CA_n40A-n41C for further MSD evaluation.

# Way forward

Using the following test points to estimate the MSD value:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2353.5 | FFS | TDD | IMD3 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |

Note: For the band n40 which is subject to IMD5 and IMD7 caused by CA_n41C, the MSD is not specified

# Reference

[1]  R4-2401272, TP for TR38.718-02-01_CA_n40A-n41C, ZTE Corporation

[2]  R4-24xxxxx, ad hoc minutes for [110][105] NR_Baskets_Part_1, Skyworks Solution Inc.

[3]  R4-2105276, TP for TR 37.717-00-00 for SUL_n41A-n97A, Huawei, HiSilicon, CMCC