3GPP TSG-RAN WG4 Meeting #110	R4-2403868

Athens, Greece, February 26 – March 01, 2024

Title: 	WF on NR sidelink UE RF requirements

Agenda Item:	8.22.5

Source: 	LG Electronics

Document for:	Approval

- [110][134] NR_SL_enh2_UERF_part1

Issue 1-1-1: UE feature list

<On-line Agreement>

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. # NR_SL_enh2 | 45-3 | Power class for sidelink unlicensed | This parameter indicates the supported power class for this band used for sidelink unlicensed. If the field is absent, the UE supports the default power class in TS 38.101-1 [2], Table 6.2E.1F-1. |  | Yes | Yes | UE cannot transmit in proper power class as specified in 38.101-1, e.g., power class 5 | Per Band | No | FR1 only | N/A |  | Optional with capability signalling |

Issue 1-2-1: Default power class

<On-line Agreement>

- Add Note 2: Power class 5 is default power class unless otherwise stated into the Table 6.2E.1F-1.

Issue 1-2-2: Configured transmitted power

<On-line Agreement>

- Agree on adding the subclause for SL-U configured transmitted power

Issue 2-1-1: Remaining NS values for SL-U

<On-line Agreement>

- If SL is agreed as Rel-19 RAN4-led package and the remaining NS values are included, specify them in Rel-19. If not, specify them in Rel-18 maintenance.

Issue 2-2-1: MPR results for PSFCH

<On-line Agreement>

- Capture OPPO’s MPR results(R4-2401810) in TR 387.786

- [110][136] NR_SL_enh2_UERF_part3

Issue 1: Channel bandwidth for Sidelink CA

<On-line Agreement>

Table 5.3E.1A-1 NR SL intra-band contiguous CA operating bands for SL CA in FR1

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47B | SL_n47B | 10 | 10, 20,30 |  |  | 70 | 0 |
|  |  | [20] | [20,30] |  |  |  |  |
|  |  | 30 | 30,40 |  |  |  |  |

Issue 2: PEMAX,CA for Sidelink CA

<On-line Agreement>

- Allow PEMAX,CA, defined by IE sl-maxTransPower-CA

- Issue 3: MPR for SL intra-band contiguous CA with non-contiguous RB allocation

<Recommended WF>

- If SL CA is agreed as Rel-19 RAN4-led package, specify the corresponding MPR in Rel-19. If not, specify it in Rel-18 maintenance.