3GPP TSG-RAN WG4 Meeting #110-bis	R4-2404129

Changsha, China, April 15-19, 2024

Source:	Nokia

Title:	Simulations for MIMO_evo BS demodulation

Agenda item:	6.19.4.2

Document for:	Information

Introduction

During the RAN4#109, the discussions on eDMRS performance for Rel-18 MIMO were agreed, in this contribution we present the results of our simulation campaign based on the parameters agreed within the WF [1].

Discussion

| Ant | Rank | BW | SCS | PRB | Mapping Type | Channel | MCS | Ideal Result SINR [dB] @70% TP | Impairment Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TypeA 2 eDMRS | TDLC300-100 Low | 16 | 7.8 | 9.3 |
|  |  | 10 |  | 24 |  |  | 21 | 12.0 | 13.5 |
|  |  |  |  |  | TypeB 2 eDMRS |  | 16 | 7.8 | 9.3 |
|  |  |  | 30 |  |  |  | 21 | 12.0 | 13.5 |
|  |  |  |  |  | TypeA 2 eDMRS |  | 16 | 7.8 | 9.3 |
|  |  | 100 |  | 272 |  |  | 21 | 12.7 | 14.2 |
|  |  |  |  |  | TypeB 2 eDMRS |  | 16 | 7.8 | 9.3 |
|  |  |  |  |  |  |  | 21 | 12.7 | 14.2 |
| 1Tx1Rx | Rank1 |  |  |  | TypeA 2 eDMRS |  | 16 | 7.7 | 9.2 |
|  |  | 5 |  | 24 |  |  | 21 | 11.7 | 13.2 |
|  |  |  |  |  | TypeB 2 eDMRS |  | 16 | 7.7 | 9.2 |
|  |  |  | 15 |  |  |  | 21 | 11.7 | 13.2 |
|  |  |  |  |  | TypeA 2 eDMRS |  | 16 | 7.9 | 9.4 |
|  |  | 50 |  | 270 |  |  | 21 | 11.8 | 13.3 |
|  |  |  |  |  | TypeB 2 eDMRS |  | 16 | 7.8 | 9.3 |
|  |  |  |  |  |  |  | 21 | 11.8 | 13.3 |
|  |  |  |  |  | TypeA 2 eDMRS |  | 16 | 16.0 | 17.5 |
|  |  | 10 |  | 24 |  |  | 21 | 23.5 | 25 |
|  |  |  |  |  | TypeB 2 eDMRS |  | 16 | 16.0 | 17.5 |
|  |  |  | 30 |  |  |  | 21 | 23.5 | 25 |
|  |  |  |  |  | TypeA 2 eDMRS |  | 16 | 16.7 | 18.2 |
|  |  | 100 |  | 272 |  |  | 21 | 29.2 | 30.7 |
|  |  |  |  |  | TypeB 2 eDMRS |  | 16 | 16.6 | 18.1 |
| 2Tx2Rx | Rank2 |  |  |  |  |  | 21 | 29.1 | 30.6 |
|  |  |  |  |  | TypeA 2 eDMRS |  | 16 | 15.2 | 16.7 |
|  |  | 5 |  | 24 |  |  | 21 | 19.6 | 21.1 |
|  |  |  |  |  | TypeB 2 eDMRS |  | 16 | 15.2 | 16.7 |
|  |  |  | 15 |  |  |  | 21 | 19.6 | 21.1 |
|  |  |  |  |  | TypeA 2 eDMRS |  | 16 | 15.5 | 17 |
|  |  | 50 |  | 270 |  |  | 21 | 20.1 | 21.6 |
|  |  |  |  |  | TypeB 2 eDMRS |  | 16 | 15.4 | 16.9 |
|  |  |  |  |  |  |  | 21 | 20.1 | 21.6 |

Conclusion

We have presented Nokia's results for eDMRS and BS demodulation performances with relation to MIMO evolution.

No observations or proposals were made.

- References

- R4-2321141 “WF on NR_MIMO_evo_DL_UL_demod”, RAN4#109, Chicago, November 2023.