3GPP TSG-RAN WG4 Meeting #110bis	R4-2404145

Changsha, China, 15th – 19th April, 2024

Agenda item:	6.1.3.1.1

Source:	MediaTek Inc.

Title:	Simulation results on PDSCH requirements for 8Rx UE

Document for:	Information

- Introduction

In RAN4#110 meeting, it was agreed to define two sets of requirements for 8Rx UE. In this contribution, we provide simulation results based on the WF [1].

Issue 1-1: UE feature definition - Agreement - Define two optional without UE capability signalling UE feature for the support of different 8Rx receiver types - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors Issue 1-2: Rank2 test cases - Agreement - Define two sets of performance requirements as per UE support of different 8Rx receiver type - Set#1 requirement: Baseline SU-MIMO 8Rx receiver - Set#2 requirement: Simplified SU-MIMO 8Rx receiver Issue 1-3: Rank4 test cases - Agreement - Define two sets of performance requirements as per UE support of different 8Rx receiver type - Set#1 requirement: Baseline SU-MIMO 8Rx receiver - Set#2 requirement: Simplified SU-MIMO 8Rx receiver Issue 2-1: How to define requirements for CC with Rank2 - Agreement - Define two set of performance requirements as per UE support of different 8Rx receiver type - Set#1 requirement: Baseline SU-MIMO 8Rx receiver - Set#2 requirement: Simplified SU-MIMO 8Rx receiver

- Simulation Results

      - 2.1 Simulation results for single carrier

We provide our simulation results for 8Rx UE receiver with Rank 2, 4 and 8 in Tables 1, 2 and 3.

Table 1. Simulation results for single carrier Rank 2 and 4 (baseline receiver)

| Rank | MCS | Propagation Condition | Channel Correlation | Alignment results TDD SNR (dB) | Impairment results TDD SNR (dB) | Alignment results FDD SNR (dB) | Impairment results FDD SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 19 (64 QAM, MCS Table 1) | TDLC 300-100 | ULA Medium B | 11.1 | 13.6 | 11.2 | 13.7 |
| 4 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 10.3 | 12.7 | 10.2 | 12.7 |

Table 2. Simulation results for single carrier Rank 2 and 4 (simplified receiver)

| Rank | MCS | Propagation Condition | Channel Correlation | Alignment results TDD SNR (dB) | Impairment results TDD SNR (dB) | Alignment results FDD SNR (dB) | Impairment results FDD SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 19 (64 QAM, MCS Table 1) | TDLC 300-100 | ULA Medium B | 12.2 | 14.7 | 12.0 | 14.5 |
| 4 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 12.5 | 15.0 | 12.7 | 15.2 |

Table 3. Simulation results for single carrier Rank 8

| Rank | MCS | Propagation Condition | Channel Correlation | Alignment results TDD SNR (dB) | Impairment results TDD SNR (dB) | Alignment results FDD SNR (dB) | Impairment results FDD SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 20.8 | 23.3 | 20.4 | 22.9 |

      - 2.2 Simulation results for carrier aggregation

Simulation results for 8Rx CA requirements are provided in the following tables.

Table 4. Simulation results for CA Rank 2 (TDD, baseline receiver)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD | 30 | 5 | 2 | 19 (Table 1) | TDLC300-100 | 2x8 ULA Medium B | 11.2 | 13.7 |
|  |  | 10 |  |  |  |  | 11.2 | 13.7 |
|  |  | 15 |  |  |  |  | 11.2 | 13.7 |
|  |  | 20 |  |  |  |  | 11.3 | 13.8 |
|  |  | 25 |  |  |  |  | 11.3 | 13.8 |
|  |  | 30 |  |  |  |  | 11.2 | 13.7 |
|  |  | 40 |  |  |  |  | 11.1 | 13.6 |
|  |  | 50 |  |  |  |  | 11.3 | 13.8 |
|  |  | 60 |  |  |  |  | 11.4 | 13.9 |
|  |  | 80 |  |  |  |  | 11.5 | 14.0 |
|  |  | 90 |  |  |  |  | 11.6 | 14.1 |
|  |  | 100 |  |  |  |  | 11.7 | 14.2 |

Table 5. Simulation results for CA Rank 2 (TDD, simplified receiver)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD | 30 | 5 | 2 | 19 (Table 1) | TDLC300-100 | 2x8 ULA Medium B | 12.1 | 14.6 |
|  |  | 10 |  |  |  |  | 12.0 | 14.5 |
|  |  | 15 |  |  |  |  | 12.2 | 14.7 |
|  |  | 20 |  |  |  |  | 12.2 | 14.7 |
|  |  | 25 |  |  |  |  | 12.3 | 14.8 |
|  |  | 30 |  |  |  |  | 12.3 | 14.8 |
|  |  | 40 |  |  |  |  | 12.2 | 14.7 |
|  |  | 50 |  |  |  |  | 12.4 | 14.9 |
|  |  | 60 |  |  |  |  | 12.3 | 14.8 |
|  |  | 80 |  |  |  |  | 12.4 | 14.9 |
|  |  | 90 |  |  |  |  | 12.5 | 15.0 |
|  |  | 100 |  |  |  |  | 12.6 | 15.1 |

Table 6. Simulation results for CA Rank 2 (FDD, baseline receiver)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 15 | 5 | 2 | 19 (Table 1) | TDLC300-100 | 2x8 ULA Medium B | 11.2 | 13.7 |
|  |  | 10 |  |  |  |  | 11.2 | 13.7 |
|  |  | 15 |  |  |  |  | 11.2 | 13.7 |
|  |  | 20 |  |  |  |  | 11.4 | 13.9 |
|  |  | 25 |  |  |  |  | 11.3 | 13.8 |
|  |  | 30 |  |  |  |  | 11.3 | 13.8 |
|  |  | 35 |  |  |  |  | 11.4 | 13.9 |
|  |  | 40 |  |  |  |  | 11.4 | 13.9 |
|  |  | 45 |  |  |  |  | 11.5 | 14.0 |
|  |  | 50 |  |  |  |  | 11.5 | 14.0 |

Table 7. Simulation results for Rank 2 (FDD, simplified receiver)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 15 | 5 | 2 | 19 (Table 1) | TDLC300-100 | 2x8 ULA Medium B | 12.1 | 14.6 |
|  |  | 10 |  |  |  |  | 12.0 | 14.5 |
|  |  | 15 |  |  |  |  | 12.2 | 14.7 |
|  |  | 20 |  |  |  |  | 12.2 | 14.7 |
|  |  | 25 |  |  |  |  | 12.3 | 14.8 |
|  |  | 30 |  |  |  |  | 12.3 | 14.8 |
|  |  | 35 |  |  |  |  | 12.3 | 14.8 |
|  |  | 40 |  |  |  |  | 12.3 | 14.8 |
|  |  | 45 |  |  |  |  | 12.4 | 14.9 |
|  |  | 50 |  |  |  |  | 12.4 | 14.9 |

Table 8. Simulation results for Rank8 (TDD)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD | 30 | 5 | 8 | 17 (Table 1) | TDLA30-10 | 8x8 ULA low | 20.8 | 23.3 |
|  |  | 10 |  |  |  |  | 20.6 | 23.1 |
|  |  | 15 |  |  |  |  | 20.7 | 23.2 |
|  |  | 20 |  |  |  |  | 20.6 | 23.1 |
|  |  | 25 |  |  |  |  | 20.7 | 23.2 |
|  |  | 30 |  |  |  |  | 20.9 | 23.4 |
|  |  | 40 |  |  |  |  | 20.8 | 23.3 |
|  |  | 50 |  |  |  |  | 20.7 | 23.2 |
|  |  | 60 |  |  |  |  | 20.8 | 23.3 |
|  |  | 80 |  |  |  |  | 20.7 | 23.2 |
|  |  | 90 |  |  |  |  | 20.7 | 23.2 |
|  |  | 100 |  |  |  |  | 20.8 | 23.3 |

Table 9. Simulation results for Rank8 (FDD)

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 15 | 5 | 8 | 17 (Table 1) | TDLA30-10 | 8x8 ULA low | 20.5 | 23.0 |
|  |  | 10 |  |  |  |  | 20.4 | 22.9 |
|  |  | 15 |  |  |  |  | 20.4 | 22.9 |
|  |  | 20 |  |  |  |  | 20.3 | 22.8 |
|  |  | 25 |  |  |  |  | 20.5 | 23.0 |
|  |  | 30 |  |  |  |  | 20.4 | 22.9 |
|  |  | 35 |  |  |  |  | 20.6 | 23.1 |
|  |  | 40 |  |  |  |  | 20.7 | 23.2 |
|  |  | 45 |  |  |  |  | 20.7 | 23.2 |
|  |  | 50 |  |  |  |  | 20.8 | 23.3 |

- Conclusion

In this contribution, we provide simulation results for 8Rx PDSCH single carrier and CA requirements.

- Reference

- R4-2402857, “WF for UE 8Rx performance requirements”, Huawei, HiSilicon