# 3GPP TSG-RAN WG4 Meeting #110-bis                                                         R4-2404155

# Changsha, China, 15th – 19th April 2024

# Title:	TP for TR38.898 HP-ENDC for FR1

# Source:	Softbank, LG Electronics

# Agenda Item:	5.15.2

# Document for:	Approval

- Introduction

This contribution is a text proposal for TR38.898[5] to include the following HP-ENDC combinations as requested in RAN4#110.

- DC_1A-11A_n77A

- DC_1A-11A_n79A

- DC_3A-8A_n79A

- DC_3A-11A_n77A

- DC_8A-11A_n77A

- DC_8A-11A_n79A

- DC_3A_n1A-n79A

- Reference

[3] 3GPP TS 38.101-3 "User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 18)", v18.5.1.

[5] 3GPP TR38.898 “High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL) (Release 18)”, V0.8.0

[6] 3GPP TR37.863-01-01 “E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of LTE 1 Down Link (DL) / 1 Up Link (UL) and 1 NR band (Release 15)”, V15.4.0

[7] 3GPP TR37.863-01-01 “E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of 1 band LTE (1 Down Link (DL) / 1 Up Link (UL)) and 1 NR band (1 DL / 1 UL)(1DL/1UL) (Release 16)”, V16.0.0

- Text Proposal

- -- Start of TP –

-- Unaffected parts omitted –

### 5.x	DC_1-11_n77

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-11A_n77A5,14 | DC_1A_n77A14 DC_11A_n77A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.x.2	Maximum output power for DC

Based on studies of PC2 DC_1_n77 and PC3 DC_11_n77, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n77 and DC_11_n77 captured in TR 37.863-01-01[6], own Rx impact of the 3rd band is the followings.

- 2nd order IMD generated by band 1 and band n77 may fall into own band 11.

Table 5.x.3-1: MSD test points for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-11A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | 11 | N/A | 5 | N/A | 1486 | 37.6 | IMD2 |
|  | n77 | 3441 | 10 | 50 | 3441 | N/A | N/A |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.x	DC_1-11_n79

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-11A_n79A5,14 | DC_1A_n79A14 DC_11A_n79A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.x.2	Maximum output power for DC

Based on studies of PC2 DC_1_n79 and PC3 DC_11_n79, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n79 and DC_11_n79 captured in TR 37.863-01-0 [6], own Rx impact of the 3rd band is the followings.

- 4th order IMD generated by band 1 and band n79 may fall into Rx frequencies of band 11.

Table 5.x.3-1: MSD test points for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-11A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 11 | N/A | 5 | N/A | 1483 | 22.2 | IMD4 |
|  | n79 | 4427 | 40 | 216 | 4427 | N/A | N/A |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.x	DC_3-8_n79

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-8A_n79A5,14 | DC_3A_n79A14 DC_8A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.x.2	Maximum output power for DC

Based on studies of PC2 DC_3_n79 and PC2 DC_8_n79, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n79 and DC_8_n79 captured in TR 37.863-01-01[6], own Rx impact of the 3rd band is the followings.

- 3rd order IMD generated by band 8 and band n79 may fall into Rx frequencies of band 8 and band n79. However, TDD band does not transmit and receive simultaneously in a single band, tuns the own band of n79 would not be interfered.

- 4th order IMD may also fall into Rx frequencies of band 3.

Table 5.x.3-1: MSD test points for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-8A_n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1850 | 21.2 | IMD4 |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.x	DC_3-11_n77

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-11A_n77A5,14 | DC_3A_n77A14 DC_11A_n77A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.x.2	Maximum output power for DC

Based on studies of PC2 DC_3_n77 and PC3 DC_11_n77, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n77 and DC_11_n77 captured in TR 37.863-01-01[6], own Rx impact of the 3rd band is the followings.

- 4th order IMD generated by band 3 and band n77 may fall into Rx frequencies of band 11.

Table 5.x.3-1: MSD test points for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-11A_n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 3675 | 10 | 50 | 3675 | N/A | N/A |
|  | 11 | N/A | 5 | N/A | 1491 | 20.2 | IMD4 |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.x	DC_8-11_n77

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A-11A_n77A5,14 | DC_8A_n77A14 DC_11A_n77A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.x.2	Maximum output power for DC

Based on studies of PC2 DC_8_n77 and PC3 DC_11_n77, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_8_n77 and DC_11_n77 captured in TR 37.863-01-01[6], own Rx impact of the 3rd band is the followings.

- 3rd order IMD generated by band 8 and band n77 may fall into Rx frequencies of band 11.

Table 5.x.3-1: MSD test points for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-11A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 11 | N/A | 5 | N/A | 1491 | 28.4 | IMD3 |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.x	DC_8-11_n79

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A-11A_n79A5,14 | DC_8A_n79A14 DC_11A_n79A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.x.2	Maximum output power for DC

Based on studies of PC2 DC_8_n79 and PC3 DC_11_n79, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_8_n79 and DC_11_n79 captured in TR 37.863-01-01[6], own Rx impact of the 3rd band is the followings.

- 5th order IMD generated by band 8 and band n79 may fall into Rx frequencies of band 11.

Table 5.x.3-1: MSD test points for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-11A_n77A | 8 | 882.5 | 5 | 25 | 927.5 | N/A | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 11 | N/A | 5 | N/A | 1478.4 | 16.2 | IMD5 |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.x	DC_3_n1-n79

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n1A-n79A5,14 | DC_3A_n1A DC_3A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.x.2	Maximum output power for DC

Based on studies of PC3 DC_3_n1 and PC2 DC_3_n79, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n1 captured in TR37.716-11-11[7] and DC_3_n79 captured in TR 37.863-01-01[6], own Rx impact of the 3rd band is the followings.

- 5th order IMD generated by band 3 and band n79 may fall into Rx frequencies of band n1.

Table 5.x.3-1: MSD test points for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n1A-n79A | n1 | N/A | 5 | N/A | 2140 | 18.7 | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

-- Unaffected parts omitted –

-- End of TP –