3GPP TSG RAN WG4 Meeting #110-bis		R4-2404194

Changsha, China, April 15 – 19, 2024

Agenda item:	9.1.3

Source:	Apple

Title:	On power domain enhancement for NR

WI/SI:	NR_ENDC_RF_Ph4

Release:	Rel-19

Document for:	Approval

# 1	Introduction

Power domain enhancement is a re-occurring topic and enters the 4th phase with Rel-19. The objectives are covered in the WID [1]. This contribution focuses on the high-level aspects of MPR reduction. The WID lists those goals as:

# 2  Discussion

Over the past releases RAN4 discussed various aspect allowing to increase the output power and consequentially improve UL coverage. In Rel-19 a new angle is approached by evaluating potential power domain improvements from relaxing emission requirements. Such requirements might be ACLR, SEM and spurious emissions. Since ACLR is typically associated with being the limiting factor for large allocations and SEM often for narrow allocations, high-throughput and cell-edge scenarios might see performance gains.

Challenges from lifting emission requirements such as increased interference to neighbouring channels and potential impact to UE-to-UE coexistence need to be carefully analysed. It is important to the check influence on the network as a whole and not only investigate the uplink power improvements itself. Furthermore, it needs to be taken care that under no circumstances regulatory requirements are violated by lifting those emission limits.

The WID description is highly ambitious with the goal to guarantee a useful MPR reduction scheme. The WID states that NR single carrier shall be investigated as well as UL CA. For both configurations several scenarios are assumed. Modulations are QPSK and 16QAM. Additionally, RedCap devices are included. This leads to a multitude of evaluations throughout the Rel-19 time frame. Due to the ambitious workload it seems to be advice-able that RAN4 tackles the individual aspects step-by-step. This approach would guarantee that every meeting enough results from different companies are present to allow meaningful decisions or/and agreements. For the initial stage RAN4 could evaluate the performance gains of single carrier and the interference on other devices. As second step UL CA could be analysed.

Observation:  The WID description is highly ambitious to guarantee a useful MPR reduction scheme. The WID states that NR single carrier shall be investigated as well as UL CA. For both configurations several scenarios are assumed. Modulations are QPSK and 16QAM. Additionally, RedCap devices are included. This leads to a multitude of evaluations throughout the Rel-19 time frame. Due to the ambitious workload it seems to be adviceable that RAN4 tackles the individual aspects step-by-step. This approach would guarantee that every meeting enough results from different companies are present to allow meaningful decisions or/and agreements.

Proposal: For the initial stage RAN4 could evaluate the performance gains of single carrier and the interference on other devices. As second step UL CA could be analysed.

# 4	Conclusions

This contribution provides a short high-level discussion on the MPR reduction aspect of the power domain enhancement WID. The following observation and proposal are made:

Observation:  The WID description is highly ambitious to guarantee a useful MPR reduction scheme. The WID states that NR single carrier shall be investigated as well as UL CA. For both configurations several scenarios are assumed. Modulations are QPSK and 16QAM. Additionally, RedCap devices are included. This leads to a multitude of evaluations throughout the Rel-19 time frame. Due to the ambitious workload it seems to be adviceable that RAN4 tackles the individual aspects step-by-step. This approach would guarantee that every meeting enough results from different companies are present to allow meaningful decisions or/and agreements.

Proposal: For the initial stage RAN4 could evaluate the performance gains of single carrier and the interference on other devices. As second step UL CA could be analysed.

# 4	References

- RP-240828, “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, 3GPP TSG RAN Meeting #103