3GPP TSG RAN WG4 Meeting #110-bis		R4-2404195

Changsha, China, April 15 – 19, 2024

Agenda item:	9.14.2

Source:	Apple

Title:	Initial views on LPWUS for NR

WI/SI:	NR_LPWUS-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

LP-WUS/WUR shall be introduced to NR in Rel-19. The objectives are covered in the WID [1]. This contribution focuses on the high-level aspects of RAN4 core requirements. The WID lists those goals as:

# 2  Discussion

Discussions have been held on the various architecture options with respect to their diverse impairments and current consumption. Especially, the latter is important for LP WUR as it significantly influences the lifetime of the device. Current consumption is highly dependent on the architecture and design choices. One of those design choices is the receiver sensitivity. It has been discussed that there exists a trade-off between sensitivity level and power consumption of the receiver. Stringent requirements translate into higher filter order and increased dynamic range for the LNA. While filter order increases complexity of the hardware the high dynamic range also means larger power consumption. In particular the ACS can be an important factor as LP WUR’s power consumption and adjacent channel rejection are not independent and need to be balanced for achieving optimum performance.

As RAN4 proceeds with the standardisation efforts on the key RF requirements (ACS, ASCS, REFSENS and others) with respect to power consumption, it is important to consider the diverse use cases and costs which allow different hardware choices, form factors allowing and different battery capacities. To achieve reasonable lifetimes for different LP WUR implementations and scenarios, RAN4 should consider more than one set of receiver requirements with respect to architecture and waveform.

Observation: It has been found that there exists a trade-off between sensitivity level and power consumption of the receiver. Stringent requirements translate into higher filter order and increased dynamic range for the LNA. While filter order increases complexity of the hardware the high dynamic range also means larger power consumption. In particular the ACS can be an important factor as LP WUR’s power consumption and adjacent channel rejection are not independent and need to be balanced for achieving optimum performance. As RAN4 proceeds with the standardisation efforts on the key RF requirements (ACS, ASCS, REFSENS and others) with respect to power consumption, it is important to consider the diverse use cases and costs which allow different hardware choices, form factors allowing and different battery capacities.

Proposal: To achieve reasonable power consumption for potential LP WUR hardware implementations, RAN4 should consider more than one set of receiver requirements with respect to architecture and waveform.

# 4	Conclusions

This contribution provides a short high-level discussion on LP WUR. The following observation and proposal are made:

Observation: It has been found that there exists a trade-off between sensitivity level and power consumption of the receiver. Stringent requirements translate into higher filter order and increased dynamic range for the LNA. While filter order increases complexity of the hardware the high dynamic range also means larger power consumption. In particular the ACS can be an important factor as LP WUR’s power consumption and adjacent channel rejection are not independent and need to be balanced for achieving optimum performance. As RAN4 proceeds with the standardisation efforts on the key RF requirements (ACS, ASCS, REFSENS and others) with respect to power consumption, it is important to consider the diverse use cases and costs which allow different hardware choices, form factors allowing and different battery capacities.

Proposal: To achieve reasonable power consumption for potential LP WUR hardware implementations, RAN4 should consider more than one set of receiver requirements with respect to architecture and waveform.

# 4	References

- RP-240801, “Low-power Wake-up Signal and Receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, 3GPP TSG RAN Meeting #103