3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404213

# Changsha, China, 15th – 19th April, 2024

Agenda item:	6.20.4

Source:	Qualcomm Incorporated

Title:	On NR SL enhancement demod requirement scope

Document for:	Discussion

# Introduction

In this contribution, we present our view on the scope of R18 SL enhancement.

# Discussion

Observation 1: Since the Doppler spread is less than 200Hz, the coherent time is around 5ms which is much larger than one slot. We also don’t observe any channel estimation degradation when using 2 DMRS compared to 3 DMRS. Therefore, we propose to have 2 DMRS regardless of PSFCH.

Proposal 1: Use 2 DMRS for SL-U PSSCH demod test.

Proposal 2: RAN4 to decide the proper TE emulation for the transmitted SL-UE behavior (including COT sharing, LBT model) and whether DUT UE PSFCH transmission failure can have impact on statistics.

# Conclusions

Proposal 1: Use 2 DMRS for SL-U PSSCH demod test.

Proposal 2: RAN4 to decide the proper TE emulation for the transmitted SL-UE behavior (including COT sharing, LBT model) and whether DUT UE PSFCH transmission failure can have impact on statistics.