3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404216

Changsha, China, 15th – 19th April, 2024

Agenda item:	9.11.4

Source: 	Qualcomm Incorporated

Title: 	On AI/ML RAN4 CSI Interoperability

Document for:	Approval

# Introduction

We present our view on the CSI use case in this contribution. We propose the model parameters for option 3 based on the table agreed in the previous meeting [1] and analyze the potential options for specifying test decoders for option 4 in this contribution.

# Discussion

## Option 3 fully specified decoder proposals

We reorganized the table for specifying option 3 agreed in [1] based on the following observations:

Observation 1: Some of the layer/model sizes are determined by the test configurations, and therefore we can parameterize the layer sizes as a function of test configuration parameters, add rows in the table for test configurations, to simplify the discussion. Therefore, the agreed table can be applicable to multiple test configurations if needed.

We suggest to further divide the ML model type into different stages based on their functionalities and relationship to the latent message since it has to have an agreed (fixed) format.

Based on the observations above, we illustrate our proposal of test decoder in the figure below, and propose the new table and the test decoder parameters as follows:

Proposal 1: We propose a set of test decoder model parameters in Table 1 and depict it in Figure 1. Besides the test decoder , we propose to add a new category: test configuration (rows are in orange). The rows in the test configuration list some parameters used in the layer size and model type to separate the test configuration discussion from the model design discussion.

Note: although the proposal is for test decoder, we include the paired encoder (Figure 1) to better represent the overall structure proposal.

Scalar quantizer

Scalar quantizer

Input

(nSBxnTx)

MLP

Flatten

Output layer

Latent message

Latent message

Input layer

Reshape

MLP

Output

(nSBxnTx)

Input

(nSBxnTx)

MLP

Flatten

Output layer

Latent message

Latent message

Input layer

Reshape

MLP

Output

(nSBxnTx)

Figure 1: Test decoder structure proposal and its paired encoder structure

Table 1: Test decoder parameter proposal

| Category | Parameter | Proposal |
| --- | --- | --- |
| Test configuration | Number of Tx antenna ports | nTx = |
|  | Propagation condition | TBD |
|  | Payload format and size (Encoder-decoder interface) | Vector of size Zdim=64 with 128 bits |
|  | Bandwidth/ Number of sub-bands | 20MHz/ nSB = 12 |
| Model architecture parameters (orange) Assumption: encoder model is the mirror of decoder model | Model type | MLP |
|  | Model depth | Three linear layers (with one activation function) |
|  | Layer type/size | MLP with expansion factor N = 4, and each layer/function is described in the following - 1st linear layer: input is latent message of size Zdim and output is a vector of size nSB x nTx - Reshape: convert the vector of size nSB x nTx to nSB vectors with size nTx - 2nd Linear layer: For each subband, the input is a vector of size nTx, and the output is a vector of size N x nTx. The same linear layer is applied to each of nSB subbands. - Activation function: GELU - 3rd Linear layer: the input is a vector of size N x nTx, and the output is with a vector of size nTx. The same linear layer is applied to each of nSB subbands. Note: 2nd linear layer, Activation function, 3rd linear layer are shown as “MLP” is Figure 1. |
|  | Fixed point representation | TBD |
|  | Format of input to encoder/output of decoder | TBD |
|  | Quantization method for the encoder output (encoder) | Scalar quantizer, with 2 bits per dimension |
| Training related parameters | Loss function | SGCS |
|  | Training dataset | Encoder input dataset should cover all the contributing companies’ encoder input data |

For the rest of the parameters in the table from [1], we have the following observations:

Observation 2: If model architecture parameters are agreed, different training procedures and hyperparameters are likely to lead to very similar decoders.

Propose 2:  Remove training procedure from “the parameter needed to be agreed”, RAN4 shouldn’t discuss training procedures, hyperparameters and cross-validation details before model architecture parameters are agreed.

## Option 4 partially specified decoder proposals

In TR 38.843, we require partially specified decoders to satisfy the principle of test repeatability/bounded variation across implementations:

Test repeatability should be ensured (variation among TE vendor implementations should be bound)

Test repeatability is guaranteed by the decoder output consistency/similarity given the same encoder output (latent message) among test decoders implemented by TE vendors. The simplest way to achieve testability is option 3, fully specified test decoder. However, when the agreed decoder will be captured in the spec as the standardized test decoder, reaching agreements in RAN4 may be more challenging. Therefore, we explore the options to derive the partially specified test decoder and correspondingly, what should be captured in specification. We start with the options which requires RAN4 to agree a pair of encoder and decoder (with full details, call them reference encoder/decoder to distinguish them from test decoder), but instead of capturing the decoder as test decoder on the specification, RAN4 captures the items close related to the test decoder in specification.

The first option we explore is after RAN4 agrees a pair of encoder and decoder with full details (from model structure down to every weights/connections on all layers) and an encoder input data generation procedure, RAN4 can use this encoder/decoder pair and the generation procedure to generate a set of decoder input and output data and capture this dataset in the specification. TE vendors are expected to train their test decoder based on the specified dataset. RAN4 also needs to specify a test decoder verification procedure based on the specified dataset to ensure TE vendors done the training correctly. We call this option the standardized dataset, option 4a-1.

The second option we explore is after RAN4 agrees a pair of encoder and decoder with full details (from model structure down to every agreed parameters in the test decoder parameter table in the previous meeting WF[1]) and an encoder input data generation procedure, RAN4 can capture the encoder as a reference encoder and the encoder input data generation procedure in the specification. TE vendors are expected to train their test decoder based on the specified reference encoder and encoder input data generation procedure. RAN4 also needs to specify a test decoder verification procedure based on the specified reference encoder and encoder input data generation procedure to ensure TE vendors done the training correctly. We call this option the reference encoder, option 4b.

Then we consider the options which doesn’t require RAN4 to agree a pair of encoder and decoder. RAN4 can first achieve some agreements (e.g., part of but not all the parameters in the test decoder parameter table in the previous meeting WF[1]) for the test decoder, and companies can design their own encoder/decoder pairs to contribute the (decoder input, decoder output) dataset to RAN4. RAN4 then aggregates all the datasets from all the contributing companies, and capture the aggregated dataset in the specification. We call this option the standardized aggregated dataset, option 4a-2.

The following table compares the above listed options, together with option 3 fully specified test decoder as reference:

Table 2: Partially specified decoder options comparison

|  | Option 3 fully specified decoder | Option 4a-1 standardized dataset | Option 4b reference encoder | Option 4a-2 standardized aggregated dataset |
| --- | --- | --- | --- | --- |
| Required RAN4 agreement (in the WF, not spec) | - Encoder input generation procedure - (Likely to have it as the by-product of test decoder) an encoder with full details | - An encoder/ decoder pair with full details - Encoder input generation procedure | - Encoder input generation procedure - (Likely to have it as the by-product of reference encoder) a decoder with full details | Part of the encoder/decoder model parameters |
| RAN4 specification | A test decoder with full details | - Dataset of (decoder input, decoder output) from the agreed encoder/ decoder pair - Decoder verification procedure | - A reference encoder with full details - Decoder verification procedure | - Aggregated dataset of (decoder input, decoder output) from each contributing companies’ encoder/ decoder pair - Decoder verification procedure |
| Note that “with full details” refers to agreed parameters from model structures, depth, size, quantization etc, i.e., every agreed parameters in the test decoder parameter table in the previous meeting WF[1]. |  |  |  |  |

Proposal 3: RAN4 consider the following options for deriving the partially specified test decoder specifications:

- Option 4a-1 standardized dataset

  - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure.

  - Step 2: RAN4 uses this encoder/decoder pair and the generation procedure to generate a set of decoder input and output data and captures this dataset in the specification.

  - Step 3: RAN4 specifies a test decoder verification procedure based on the specified dataset.

- Option 4a-2 standardized aggregated dataset

  - Step 1: RAN4 achieves some agreements (e.g., part of but not all the parameters in the test decoder parameter table in the previous meeting WF[1]) for the test decoder.

  - Step 2: Interested companies can design their own encoder/decoder pairs based on the agreements to contribute the (decoder input, decoder output) dataset to RAN4

  - Step 3: RAN4 aggregates the datasets from all the contributing companies, and capture the aggregated dataset in the specification

  - Step 4: RAN4 specifies a test decoder verification procedure based on the specified dataset.

- Option 4b reference encoder

  - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure

  - Step 2: RAN4 capture the encoder as a reference encoder and the encoder input data generation procedure in the specification.

  - Step 3: RAN4 specifies a test decoder verification procedure based on the reference encoder.

For option 4a-2, it’s not obvious that there exists a test decoder that can properly recover the encoder input from the encoder output, given that the dataset is from multiple encoder and decoder pairs.

For option 4a-1 and 4b, we provide mathematical analysis for the above proposed options from the perspective of test repeatability/decoder output consistency. We first define the following sets

- Encoder input sets:

VRAN4: the set of encoder input from RAN4 generation procedure

VUE: the set of encoder input from UE generation procedure

VRAN4_data: RAN4 specified encoder input dataset

- Latent message spaces/sets

Z: the set of all the possible latent messages

Zrefenc,RAN4: the set of the output from reference encoder,  when the input is from RAN4 generation procedure in set VRAN4

ZRAN4_data: RAN4 specified latent dataset

- Decoder output spaces

Vtest_dec: the set of output from speficifed test decoder from input set Z

VRAN4_data: RAN4 specified decoder output dataset

And a few additional notations

A\B: the set of A excluding the intersection of set A and B

The encoder output of an encoder f from input is x denoted by f(x), and the decoder output of a decoder g from input is y denoted by g(y). We denote UE encoder by fUE, reference encoder by fRAN4, TE encoder by gTE, and RAN4 fully specified test decoder by gtest.

Based on the above notations, we analyze the options in the following:

- Specify dataset: RAN4 specification ZRAN4_data→VRAN4_data

For any encoder input v∈VRAN4_data or v close to any element in VRAN4_data, UE can train its encoder to produce the corresponding latent message in ZRAN4_data based on the specified dataset, so that the decoder output gTEfUE(v), fUE(v)∈ZRAN4_data  is close to the encoder input v and the decoder output consistency can be achieved for those encoder input.

Input data generation procedure

(RAN4 assumption)

Decoder

Channel generation
(RAN4 assumption)

RAN4 agreement covers the first row of blocks, but only capture dataset in spec

Input data generation’ (UE)

Encoder’

(UE)

???

Channel generation

(TE)

Different test decoders may recover the encoder output of v’ very differently, consistency is not guaranteed

Decoder

v

v’

The dataset cannot tell what the right output should be for the new input v’ that is far from v in the dataset.

Encoder

z

Dataset {(v,z)}

Input data generation procedure

(RAN4 assumption)

Decoder

Channel generation
(RAN4 assumption)

RAN4 agreement covers the first row of blocks, but only capture dataset in spec

Input data generation’ (UE)

Encoder’

(UE)

???

Channel generation

(TE)

Different test decoders may recover the encoder output of v’ very differently, consistency is not guaranteed

Decoder

v

v’

The dataset cannot tell what the right output should be for the new input v’ that is far from v in the dataset.

Encoder

z

Dataset {(v,z)}

However, for other input which can be possibly generated by UE pre-processing, i.e., encoder input v' far from any elements in VRAN4_data, UE vendor has no idea how to build training dataset for these inputs, and when they are mapped to the latent messages outside of ZRAN4_data, test decoder implemented by different TE vendors can generate very different output, and therefore the test repeatability can’t be guaranteed. Therefore, the option may not be feasible due to the limited coverage of VRAN4_data which may not be able to accommodate different VUE’s from different UE pre-processing procedures (channel estimation, whitening and precoder derivation etc).

- Specify reference encoder: RAN4 specifies fRAN4 and the procedure to generate VRAN4

Input data generation procedure

Encoder

Decoder

Channel generation
(RAN4 assumption)

RAN4 specified reference encoder and generation procedure (green box): need to ensure latent space coverage so that UE vendors can construct {(v’,z’)} from {(v,z)} to train its encoder

Input data generation’ (UE)

Encoder’

(UE)

Channel generation

(TE)

Decoder

v

v’

z’

z

Input data generation procedure

Encoder

Decoder

Channel generation
(RAN4 assumption)

RAN4 specified reference encoder and generation procedure (green box): need to ensure latent space coverage so that UE vendors can construct {(v’,z’)} from {(v,z)} to train its encoder

Input data generation’ (UE)

Encoder’

(UE)

Channel generation

(TE)

Decoder

v

v’

z’

z

The decoder verification procedure ensures that gTE(fRAN4(v)) is close to v if v∈VRAN4 for any TE decoder gTE. Therefore, UE vendors can use the reference decoder and the RAN4 encoder input generation procedure to generate v,fRAN4(v)| v∈VRAN4 to train its encoder, if the UE vendor doesn’t want to implement the reference encoder. When the set of reference encoder output fRAN4(v) from RAN4 encoder input generation procedure, Zrefenc,RAN4, is large enough so that Zrefenc,RAN4~Z, test repeatability can be guaranteed. However, when the UE encoder output is z∈Z/Zrefenc,RAN4, the test decoders implemented by different TE vendors may vary significantly and the test repeatability is not achievable in these cases. Note that generating v,fRAN4(v)| v∈VRAN4 and training the encoder accordingly is more complicated than jointly training the encoder with the specified test decoder.

- Specify test decoder: RAN4 specifies gTE

Test repeatability is guaranteed in this case, and we have further analysis of precoder recovery performance below. UE vendors can train its encoder by connecting the specified test decoder, or based on the dataset generated by implementing the fully specified test decoder z,gtest(z)| z∈Z. Therefore, as long as Vtest_dec, which is the set of gtest(z) for z∈Z, has a good coverage on the encoder input set of a UE, VUE, based on the UE’s encoder input generation procedure, we expect gtest(fUE(v)) is close to v for any v∈VUE. We can ensure Vtest_dec’s coverage on VUE’s by the suggestion made in proposal 1: Encoder input dataset should cover all the contributing companies’ encoder input data.

We summarize the above analysis in the below proposal

Proposal 4: Feasibility and concerns of the partially specified test decoder option should be analysed from the perspective of test repeatability (decoder output consistency/similarity given the same encoder output (latent message) among test decoders implemented by TE vendors). The following feasibility issues and concerns for options 4a-1, 4a-2 and 4b (as described in proposal 3) need to be resolved:

- Option 4a-1 (standardized dataset from one enc/dec pair):  whether it is feasible to design a standardized dataset sufficiently representing the propagation channel condition and possible UE procedures (channel estimation, whitening and desired precoder derivation). If the dataset doesn’t cover all the possible UE procedures, the encoder input not captured in the standardized dataset can produce very different decoder output, which violates the consistency requirement in option 4.

- Option 4a-2 (standardized dataset from multiple enc/dec pairs):   whether it is feasible to design a test decoder that can properly recover the encoder input from the encoder output, given that the dataset is from multiple encoder and decoder pairs.

- Option 4b (reference encoder): required to define a proper encoder input data generation procedure and a reference encoder with a good coverage of latent space to guarantee decoder output consistency by the verification procedure with the reference encoder. Another concern is that this option implies a more complicated procedure to train UE encoder, compared to option 3 fully specified decoder.

The AI/ML model we proposed in proposal 1 is applicable to the derivation of the standardized dataset in option 4a-1 and the reference encoder in option 4b.

Observation 3: RAN4 can consider Table 1 and Figure 1 for derivation of the highlighted entries in the below table (copy of Table 2) if option 4a-1 or 4b is chosen.

|  | Option 3 fully specified decoder | Option 4a-1 standardized dataset | Option 4b reference encoder | Option 4a-2 standardized aggregated dataset |
| --- | --- | --- | --- | --- |
| Required RAN4 agreement (in the WF, not spec) | - Encoder input generation procedure - (Likely to have it as the by-product of test decoder) an encoder with full details | - An encoder/ decoder pair with full details - Encoder input generation procedure | - Encoder input generation procedure - (Likely to have it as the by-product of reference encoder) a decoder with full details | Part of the encoder/decoder model parameters |
| RAN4 specification | A test decoder with full details | - Dataset of (decoder input, decoder output) from the agreed encoder/ decoder pair - Decoder verification procedure | - A reference encoder with full details - Decoder verification procedure | - Aggregated dataset of (decoder input, decoder output) from each contributing companies’ encoder/ decoder pair - Decoder verification procedure |
| Note that “with full details” refers to agreed parameters from model structures, depth, size, quantization etc, i.e., every agreed parameters in the test decoder parameter table in the previous meeting WF[1]. |  |  |  |  |

Although we find issues and concerns for options visible to us until RAN4#110 meeting for option 4 partially specified decoder, due to the simplicity of RAN4 discussion and specification procedure with option 4, we would like to encourage further RAN4 study to find a feasible (from test repeatability perspective) and implementation/specification friendly solution to option 4.

Proposal 5: RAN4 continues to study a feasible (from test repeatability perspective) and implementation/specification friendly solution to option 4.

# Conclusion

Observation 1: Some of the layer/model sizes are determined by the test configurations, and therefore we can parameterize the layer sizes as a function of test configuration parameters, add rows in the table for test configurations, to simplify the discussion. Therefore, the agreed table can be applicable to multiple test configurations if needed.

Proposal 1: We propose a set of test decoder model parameters in Table 1 and depict it in Figure 1. Besides the test decoder , we propose to add a new category: test configuration (rows are in orange). The rows in the test configuration list some parameters used in the layer size and model type to separate the test configuration discussion from the model design discussion.

Observation 2: If model architecture parameters are agreed, different training procedures and hyperparameters are likely to lead to very similar decoders.

Propose 2:  Remove training procedure from “the parameter needed to be agreed”, RAN4 shouldn’t discuss training procedures, hyperparameters and cross-validation details before model architecture parameters are agreed.

Proposal 3: RAN4 consider the following options for deriving the partially specified test decoder specifications:

- Option 4a-1 standardized dataset

  - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure.

  - Step 2: RAN4 uses this encoder/decoder pair and the generation procedure to generate a set of decoder input and output data and captures this dataset in the specification.

  - Step 3: RAN4 specifies a test decoder verification procedure based on the specified dataset.

- Option 4a-2 standardized aggregated dataset

  - Step 1: RAN4 achieves some agreements (e.g., part of but not all the parameters in the test decoder parameter table in the previous meeting WF[1]) for the test decoder.

  - Step 2: Interested companies can design their own encoder/decoder pairs based on the agreements to contribute the (decoder input, decoder output) dataset to RAN4

  - Step 3: RAN4 aggregates the datasets from all the contributing companies, and capture the aggregated dataset in the specification

  - Step 4: RAN4 specifies a test decoder verification procedure based on the specified dataset.

- Option 4b reference encoder

  - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure

  - Step 2: RAN4 capture the encoder as a reference encoder and the encoder input data generation procedure in the specification.

  - Step 3: RAN4 specifies a test decoder verification procedure based on the reference encoder.

Proposal 4: Feasibility and concerns of the partially specified test decoder option should be analysed from the perspective of test repeatability (decoder output consistency/similarity given the same encoder output (latent message) among test decoders implemented by TE vendors). The following feasibility issues and concerns for options 4a-1, 4a-2 and 4b (as described in proposal 3) need to be resolved:

- Option 4a-1 (standardized dataset from one enc/dec pair):  whether it is feasible to design a standardized dataset sufficiently representing the propagation channel condition and possible UE procedures (channel estimation, whitening and desired precoder derivation). If the dataset doesn’t cover all the possible UE procedures, the encoder input not captured in the standardized dataset can produce very different decoder output, which violates the consistency requirement in option 4.

- Option 4a-2 (standardized dataset from multiple enc/dec pairs):   whether it is feasible to design a test decoder that can properly recover the encoder input from the encoder output, given that the dataset is from multiple encoder and decoder pairs.

- Option 4b (reference encoder): required to define a proper encoder input data generation procedure and a reference encoder with a good coverage of latent space to guarantee decoder output consistency by the verification procedure with the reference encoder. Another concern is that this option implies a more complicated procedure to train UE encoder, compared to option 3 fully specified decoder.

Observation 3: RAN4 can consider Table 1 and Figure 1 for derivation of the highlighted entries in the below table (copy of Table 2) if option 4a-1 or 4b is chosen.

|  | Option 3 fully specified decoder | Option 4a-1 standardized dataset | Option 4b reference encoder | Option 4a-2 standardized aggregated dataset |
| --- | --- | --- | --- | --- |
| Required RAN4 agreement (in the WF, not spec) | - Encoder input generation procedure - (Likely to have it as the by-product of test decoder) an encoder with full details | - An encoder/ decoder pair with full details - Encoder input generation procedure | - Encoder input generation procedure - (Likely to have it as the by-product of reference encoder) a decoder with full details | Part of the encoder/decoder model parameters |
| RAN4 specification | A test decoder with full details | - Dataset of (decoder input, decoder output) from the agreed encoder/ decoder pair - Decoder verification procedure | - A reference encoder with full details - Decoder verification procedure | - Aggregated dataset of (decoder input, decoder output) from each contributing companies’ encoder/ decoder pair - Decoder verification procedure |
| Note that “with full details” refers to agreed parameters from model structures, depth, size, quantization etc, i.e., every agreed parameters in the test decoder parameter table in the previous meeting WF[1]. |  |  |  |  |

Proposal 5: RAN4 continues to study a feasible (from test repeatability perspective) and implementation/specification friendly solution to option 4.

# Reference

[1] R4-2403712