3GPP TSG-RAN WG4 Meeting #110-bis	                               R4-2404222

Changsha, China, April 15 – April 19, 2024

Source: 	Murata Manufacturing Co Ltd.

Title: 	MSD for CA_n39A-n40A-n41A supporting simultaneous Rx/Tx

Agenda item:	5. 21. 2

Document for:	Approval

Introduction

In the last RAN4#109 meeting, the WF [1] was approved to provide the missing MSD requirements for higher order combinations whose fallbacks support simultaneous Rx/Tx operation, which have not been specified.

In this contribution	we evaluate IMD calculation and resulting MSD for higher order combinations of CA_n40A-n41A/CA_n39A_41A, in other words CA_n39A-n40A-n41A.

Discussion

IMD and MSD calculations

Below IMD relations can be observed for CA_n39A-n40A-n41A whose fallback combinations support simultaneous Rx/Tx operation.

  - IMD3 between n39 UL and n40 UL is falling into n41 DL

  - IMD3 between n40 UL and n41 UL is falling into n39 DL

  - IMD5 between n40 UL and n41 UL is falling into n39 DL

Reference architecture and assumptions

WF [2] agreed that 3GPP specifies RF front-end and antenna structure agnostic MSD for simultaneous Rx-Tx requirements for higher order band combinations. It should be noted that this band combination is MB-HB-HB combination so different from other higher order combinations whose fallbacks support simultaneous Rx/Tx operation in terms of RF front end architecture. The band combinations in [3] are MB-HB or HB-HB + LB or UHB, so we have two options of RF front end architectures – shared antenna and separate antenna. However, CA_n39A-n40A-n41A should be supported by using only shared antenna structure since separating antenna for MB and HB leads to increase of UE antenna especially considering MHB MIMO operation.

Observation 1: CA_n39A-n40A-n41A should be supported by using only shared antenna structure since separating antenna for MB and HB leads to increase of UE antenna especially considering MHB MIMO operation.

Figure 1 shows two different options using shared antenna structure. Figure 1(a) separates n41 PA from n39 PA and n40 PA and Figure 1(b) separates n39 PA from n40 PA and n41 PA. There might be LB-MHB diplexer just before the antenna but there would be no impact on the MSD because there is no attenuation between MB and HB.

(a)

(b)

Figure 1. Two reference architecture options using shared antenna, (a) and (b)

Also, it should be noted that the insertion loss of the triplexer is increased compared to single filter to have sufficient attenuation level between bands. So the ΔTIB,c and ΔRIB,c due to CA_n39-n40-n41 are updated as in Table 1 and Table 2 respectively.

Table 1 ΔTIB,c due to CA_n39-n40-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n39-n40-n41 | 0.3 | 0.6 | 0.6 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 2 ΔRIB,c due to CA_n39-n40-n41

| Inter-band CA combination | ΔRIB,c for NR bands (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| CA_n39-n40-n41 | 0.3 | 0.6 | 0.6 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

Proposal 1: The ΔTIB,c and ΔRIB,c due to CA_n39-n40-n41 are updated as in Table 1 and Table 2 respectively since triplexer insertion loss is increased compared to single filter.

We performed IMD calculation based on the below linearity parameters (IP3), filter attenuation, diplexer isolation and PCB and antenna isolation which are presented in Table 3 and Table 4.

Table 3 Intercept point parameters

|  | IP3 |
| --- | --- |
| PA forward | 30 |
| PA reverse | 29 |
| Switch | 68 |
| Filter (Triplexer) | 74 |
| LNA | -5 |

Table 4 Filter(triplexer) attenuation/isolation, PCB and antenna isolation

| n39 filter attenuation at n40 | 40 |
| --- | --- |
| n39 filter attenuation at n41 | 40 |
| n40 filter attenuation at n39 | 35 |
| n40 filter attenuation at n41 | 40 |
| n41 filter attenuation at n39 | 35 |
| n41 filter attenuation at n40 | 35 |
| Isolation btw n39 and n40 at n39 | 45 |
| Isolation btw n39 and n40 at n40 | 40 |
| Isolation btw n39 and n41 at n39 | 45 |
| Isolation btw n39 and n41 at n41 | 40 |
| Isolation btw n40 and n41 at n40 | 40 |
| Isolation btw n40 and n41 at n41 | 40 |
| Antenna isolation | 15 |
| PCB isolation | 60 |

IMD and MSD calculation

IMD and corresponding MSD performance was calculated after correlated MRC for each band combinations. The correction factors were included in the resulting IMD calculation to account for the differences between CW based IMD calculation and modulation IMD calculation. The correction factor is highly dependent on the two ULs bandwidth (or UL configuration), DL BW and IMD order because the partial portion of the full IMD product is failing in the victim DL bandwidth.

Table 5 and Table 6 summarizes the resulting IMD and MSD of option (a) and option (b)

Table 5 Results for IMD calculation and MSD for CA_n39A-n40A-n41A – option (a)

Table 6 Results for IMD calculation and MSD for CA_n39A-n40A-n41A – option (b)

Therefore, we propose to use below table for RF requirement of MSD for CA_n39A-n40A-41A supporting simultaneous Rx/Tx by averaging MSD values in mW (linear scale) from two options.

Table 7 3DL/2UL MSDs for CA_n39A-n40A-n41A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39-n40-n41 | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | 31.6 | TDD | IMD3 |
|  | n39 | N/A | 5 | N/A | 1917.5 | 31.6 | TDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Proposal 2: Use Table 7 for the RF requirements of MSD of higher order combinations of CA_n39A-40A-41A supporting simultaneous Rx/Tx.

Conclusion

In this contribution, we presented the IMD relations for CA_n39A-40A-n41A supporting simultaneous Rx/Tx determining the IMD downlink overlaps and resulting MSD, which has not been specified yet. With the introduction of the result, missing RF requirement can be identified by making the following proposals.

Observation 1: CA_n39A-n40A-n41A should be supported by using only shared antenna structure since separating antenna for MB and HB leads to increase of UE antenna especially considering MHB MIMO operation.

Proposal 1: The ΔTIB,c and ΔRIB,c due to CA_n39-n40-n41 are updated as in Table 1 and Table 2 respectively since triplexer insertion loss is increased compared to single filter.

Proposal 2: Use Table 7 for the RF requirements of MSD of higher order combinations of CA_n39A-40A-41A supporting simultaneous Rx/Tx.

References

[1]	R4-2321992, “WF on simultaneous Rx-Tx requirements”, Apple, 3GPP TSG RAN WG4 Meeting #109, Chicago, United States, Nov 13th – 17th, 2023

[2] R4-2403606, “WF on simultaneous Rx-Tx issues”, Huawei, 3GPP TSG RAN WG4 Meeting #110, Athens, Greece, Feb 26th – Mar 1st, 2024

[3] R4-2400583, “MSD for higher order combinations of CA_n40A-41A and CA_n39A-41A supporting simultaneous Rx/Tx”, Murata Manufacturing Co Ltd., 3GPP TSG RAN WG4 Meeting #110, Athens, Greece, Feb 26th – Mar 1st, 2024