3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404227

Changsha, China, 15th - 19th April 2024

Title:	TP for TR 36.718-02-01 Addition of CA_1-8-11

Source:	SoftBank

Agenda item:	7.1.3.2

Document for:	Approval

1	Introduction

This contribution is a text proposal for TR 36.718-02-01[1] to include CA_1-8-11 as requested in RAN4#109.

2. Reference

[1] TR36.718-02-01, LTE-A intra-band/inter-band Carrier Aggregation for x (x<=6) bands DL with y bands (y=1, 2) UL (Release 18) V0.0.4

Text Proposal

-- Start of TP –

### 5.4.x	CA_1-8-11

#### 5.4.x.1	Channel bandwidths per operating band for CA

Table 5.4.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |

Table 5.4.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-8A-11A | CA_1A-8A CA_1A-11A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |

#### 5.4.x.2	Co-existence studies

Coexistence for CA_1-8 and CA_1-11 has already been specified in TS 36.101.

#### 5.4.x.3	∆TIB and ∆RIB values

Already included in TS 36.101.

#### 5.4.x.4	REFSENS Requirements

Table 5.4.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_1A-8A-11A | CA_8A-11A | 8 | 890 | 5 | 25 | 905 | 5 | N/A | FDD | N/A |
|  |  |  |  | 11 |  | 1440 |  | 5 |  | 25 |  | 1488 |  | 5 |  | N/A |  |  |  | N/A |  |
|  |  |  |  | 1 |  | 1975 |  | 5 |  | 25 |  | 2120 |  | 5 |  | 3.2 |  |  |  | IMD5 |  |
|  |  | CA_1A-11A |  | 1 |  | 1975 |  | 5 |  | 25 |  | 2120 |  | 5 |  | N/A |  | FDD |  | N/A |  |
|  |  |  |  | 11 |  | 1440 |  | 5 |  | 25 |  | 1488 |  | 5 |  | N/A |  |  |  | N/A |  |
|  |  |  |  | 8 |  | 890 |  | 5 |  | 25 |  | 905 |  | 5 |  | 16.2 |  |  |  | IMD3 |  |

-- End of TP --