3GPP TSG-RAN WG4 Meeting 110-bis	R4-2404232

Changsha, China 15th – 19th April 2024

Source: 	Nokia, Nokia Shanghai Bell

Title: 					FWA PC1 NS_03, NS_03U and NS_100 NR A-MPR

Agenda Item: 			5.14.2

Document for:	Approval

# 1	Introduction

We study the A-MPR for power class PC1 for NR bands n25 and n66. Both of these bands have NS-values NS_03, NS_03U and NS_100.

# 2	Simulation assumptions and scenario

## 2.1	Simulation parameters

- Power class PC1

- CBWs from 5 MHz to 45 MHz

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only additional emission requirements from NS_03, NS_03U and NS_100 are simulated. Other gating factors (EVM, NR ACLR, etc.) are excluded.

## 2.2	Additional mission requirements

NS_03 signals an additional SEM and NS_100 signals UTRA ACLR requirement. NS_03U has both NS_03 SEM and UTRA ACLR requirement, and hence the NS_03U A-MPR can be defined as the maximum of the NS_03 and NS_100 A-MPRs.

### 2.2.1	NS_03 additional SEM

Table 6.5.2.3.3-1: Additional requirements for "NS_03" and “NS_03U”

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 35, 40, 45 |  |
| 0-1 | -13 | -13 | 1 % of channel BW |
| 1-6 | -13 |  | 1 MHz |
| 6-10 | -25 |  |  |
| 1-BWChannel |  | -13 |  |
| BWChannel-(BWChannel+5) |  | -25 |  |

### 2.2.2	UTRA ACLR

The NR technical specification [1] currently defines the UTRA ACLR limits for PC3 and PC2, see table 6.5.2.4.2-1 below. We assume the same limits are also applicable for PC1 and hence apply the 33 dB and 36 dB limits in this contribution.

Table 6.5.2.4.2-1: UTRA ACLR requirement

|  | Power class 2 | Power class 3 |
| --- | --- | --- |
| UTRAACLR1 | 33 dB | 33 dB |
| UTRAACLR2 | 36 dB | 36 dB |

# 3	Simulation results and discussion

According to our simulations the MPR is enough to satisfy the UTRA ACLR limits. For NS_03 the MPR is slightly exceeded for 10 MHz CBW in a few points, other CBWs showing backoff below or equal MPR. In Appendix we present the difference of simulated backoff and MPR for NS_03 10 MHz CBW. The MPR is exceeded only by maximum of 0.2 dB, and we regard this as negligible. Hence, we propose no A-MPR also for NS_03 and NS_03U.

Proposal 1: The RAN4 shall consider defining no A-MPR for PC1 NS_03, NS_03U, and NS_100.

We note that our proposal is in line with our previous simulations and proposals for PC3 [2,3].

# 4	Conclusions

We have simulated the A-MPR for power class PC1 for NS_03, NS_03U and NS_100. Based on our simulation results we have made the following proposal.

Proposal 1: The RAN4 shall consider defining no A-MPR for PC1 NS_03, NS_03U, and NS_100.

# 5	References

[1] TS 38.101-1 (V18.4.0), 3GPP

[2] R4-1812049, NS_03 A-MPR for NR, Nokia

[3] R4-1800066, UTRA A-MPR, Nokia

# 6	Appendix