3GPP TSG-RAN WG4 Meeting # 110								         		R4-2404243

Athens, Greece, 26 Feb – 01 Mar, 2024

Title: 	Improved harmonic related MSDs template

Source: 	Skyworks Solutions Inc., Nokia

Agenda Item:	11.3	RAN4 basket WI work plan (according to WF R4-2403721)

Document for:	Approval

# 1	Background

This contribution is a re-submission based on [2] that was not treated as there was no AI for treating such “pre-R19” topics. Still, it was possible to discuss the paper [1] presenting our plans to improve the R19 block approval process and technical scope, which resulted in a way forward [4].

In this contribution, we propose a new table template covering UL harmonics and harmonic mixing, with the latter updated for proposals made in [5] for valid harmonic mixing orders that accounts for input in [2, 3].

# Discussion

## 2.1 Improved tables combining UL harmonic and harmonic mixing cases

The matrix format is helpful to visualize which harmonic mixing cases to consider. Combined with the UL harmonic table, it provides a compact view of both UL harmonic and harmonic mixing issues, that are specified in the same section of 38.101-1, but in different tables.

With this, we would like to propose a matrix format for the TP template for 2DL/1UL TR, in the scope of the Release 19 block approval process improvement agreed upon in a way forward [4] last meeting.

Proposal on a new table format for UL harmonic and harmonic mixing: The below table is used in the two DL one UL with one CC section of the block approval TPs.

- When a collision is detected with an overlap of ULX range with DLY range >0Hz, the ULX/DLY cell is marked “D” for direct hit.

        - For an odd DL harmonic order, a UL+DL order of up to 7 for harmonic mixing and up to UL5 for UL harmonics shall be considered

        - For an even DL harmonic order, a UL+DL order of up to 5 shall be considered

- When the ULX range is less than X*Minimum UL channel bandwidth away from DLY range (the gap is between 0Hz and X*MinULCBW), the ULX/DLY cell is marked “N” for Near miss and is only specified if there is no direct hit case for the same UL and DL orders. For harmonic mixing, only UL1 and odd DL orders are to be considered

- Note that when both UL and DL guard bands are accounted for, when designing the test point, there may no longer be an applicable Direct hit and/or Near miss case.

- A specific row is added to capture conclusions on the UL harmonic and harmonic mixing analysis.

- Notes are provided for guidance and orders to be considered.

Table 1: UL/DL harmonics collision table

| UL/DL harmonics |  | nX | UL13 | UL2 | UL32 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | fULlow | 2*fULlow | 3*fULlow | 4*fULlow | 5*fULlow |  |
| nY | fLow | fHigh | fULhigh | 2*fULhigh | 3*fULhigh | 4*fULhigh | 5*fULhigh |  |
| DL1 | fDLlow | fDLhigh | N/A |  |  |  |  | UL harmonic |
| DL22 | 2*fDLlow | 2*fDLhigh |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL33 | 3*fDLlow | 3*fDLhigh |  |  | N/A |  | N/A |  |
| DL4 | 4*fDLlow | 4*fDLhigh |  | N/A | N/A | N/A | N/A |  |
| DL53 | 5*fDLlow | 5*fDLhigh |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| UL/DL harmonics |  | nY | UL13 | UL2 | UL32 | UL4 | UL5 | MSD type |
|  |  | fLow | fULlow | 2*fULlow | 3*fULlow | 4*fULlow | 5*fULlow |  |
| nX | fLow | fHigh | fULhigh | 2*fULhigh | 3*fULhigh | 4*fULhigh | 5*fULhigh |  |
| DL1 | fDLlow | fDLhigh | N/A |  |  |  |  | UL harmonic |
| DL22 | 2*fDLlow | 2*fDLhigh |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL33 | 3*fDLlow | 3*fDLhigh |  |  | N/A |  | N/A |  |
| DL4 | 4*fDLlow | 4*fDLhigh |  | N/A | N/A | N/A | N/A |  |
| DL53 | 5*fDLlow | 5*fDLhigh |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| Note 1: When a collision is detected with an overlap >0Hz between the UL(X) with DL(Y) frequency ranges, the UL(X)/DL(Y) cell is marked “D” for direct hit. When the gap between UL(X) and DL(Y) frequency range is from 0Hz to X*MinULCBW, the UL(X)/DL(Y) cell is marked “N” for Near miss. Note 2: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 3: For harmonic mixing, near-miss cases only apply for UL1 and odd DL orders. |  |  |  |  |  |  |  |  |

## 2.2 CA_n1-n102 example

Table 2, show an example of the use of the table for CA_n1-n012 that present a direct hit UL1/DL3 harmonic mixing and a near-miss UL3/UL1 UL harmonic cases.

Table 2: UL/DL harmonics collision table for CA_n1-n102

| UL/DL harmonics |  | n102 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 5945 | 11890 | 17835 | 23780 | 29725 |  |
| n1 | fLow | fHigh | 6425 | 12850 | 19275 | 25700 | 32125 |  |
| DL1 | 2110 | 2170 | N/A | - | - | - | - | UL harmonic |
| DL2 | 4220 | 4340 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL3 | 6330 | 6510 | D | - | N/A | - | N/A |  |
| DL4 | 8440 | 8680 | - | N/A | N/A | N/A | N/A |  |
| DL5 | 10550 | 10850 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Direct hit n102 UL1 / n1 DL3 |  |  |  |  |  |
| UL/DL harmonics |  | n1 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
|  |  | fLow | 1920 | 3840 | 5760 | 7680 | 9600 |  |
| n102 | fLow | fHigh | 1980 | 3960 | 5940 | 7920 | 9900 |  |
| DL1 | 5945 | 6425 | N/A | - | N | - | - | UL harmonic |
| DL2 | 11890 | 12850 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL3 | 17835 | 19275 | - | - | N/A | - | N/A |  |
| DL4 | 23780 | 25700 | - | N/A | N/A | N/A | N/A |  |
| DL5 | 29725 | 32125 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Near miss n1 UL3 / n102 DL1 (5MHz) |  |  |  |  |  |

# Conclusion

In this contribution, we adapted the table format for better detection of UL harmonic and harmonic mixing issues, and added criteria on the valid harmonic mixing orders and guidance on direct-hit and Near-miss cases.

Proposal on a new table format for UL harmonic and harmonic mixing: The below table is used in the two DL one UL with one CC section of the block approval TPs.

- When a collision is detected with an overlap of ULX range with DLY range >0Hz, the ULX/DLY cell is marked “D” for direct hit.

        - For an odd DL harmonic order, a UL+DL order of up to 7 for harmonic mixing and up to UL5 for UL harmonics shall be considered

        - For an even DL harmonic order, a UL+DL order of up to 5 shall be considered

- When the ULX range is less than X*Minimum UL channel bandwidth away from DLY range (the gap is between 0Hz and X*MinULCBW), the ULX/DLY cell is marked “N” for Near miss and is only specified if there is no direct hit case for the same UL and DL orders. For harmonic mixing, only UL1 and odd DL orders are to be considered

- Note that when both UL and DL guard bands are accounted for, when designing the test point, there may no longer be an applicable Direct hit and/or Near miss case.

- A specific row is added to capture conclusions on the UL harmonic and harmonic mixing analysis.

- Notes are provided for guidance and orders to be considered.

Table 1: UL/DL harmonics collision table

| UL/DL harmonics |  | nX | UL13 | UL2 | UL32 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | fULlow | 2*fULlow | 3*fULlow | 4*fULlow | 5*fULlow |  |
| nY | fLow | fHigh | fULhigh | 2*fULhigh | 3*fULhigh | 4*fULhigh | 5*fULhigh |  |
| DL1 | fDLlow | fDLhigh | N/A |  |  |  |  | UL harmonic |
| DL22 | 2*fDLlow | 2*fDLhigh |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL33 | 3*fDLlow | 3*fDLhigh |  |  | N/A |  | N/A |  |
| DL4 | 4*fDLlow | 4*fDLhigh |  | N/A | N/A | N/A | N/A |  |
| DL53 | 5*fDLlow | 5*fDLhigh |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| UL/DL harmonics |  | nY | UL13 | UL2 | UL32 | UL4 | UL5 | MSD type |
|  |  | fLow | fULlow | 2*fULlow | 3*fULlow | 4*fULlow | 5*fULlow |  |
| nX | fLow | fHigh | fULhigh | 2*fULhigh | 3*fULhigh | 4*fULhigh | 5*fULhigh |  |
| DL1 | fDLlow | fDLhigh | N/A |  |  |  |  | UL harmonic |
| DL22 | 2*fDLlow | 2*fDLhigh |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL33 | 3*fDLlow | 3*fDLhigh |  |  | N/A |  | N/A |  |
| DL4 | 4*fDLlow | 4*fDLhigh |  | N/A | N/A | N/A | N/A |  |
| DL53 | 5*fDLlow | 5*fDLhigh |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| Note 1: When a collision is detected with an overlap >0Hz between the UL(X) with DL(Y) frequency ranges, the UL(X)/DL(Y) cell is marked “D” for direct hit. When the gap between UL(X) and DL(Y) frequency range is from 0Hz to X*MinULCBW, the UL(X)/DL(Y) cell is marked “N” for Near miss. Note 2: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 3: For harmonic mixing, near-miss cases only apply for UL1 and odd DL orders. |  |  |  |  |  |  |  |  |

# References

[1] R4-2400257 Further improvements to the block approval process in R19, Skyworks Solutions Inc., Nokia, RAN4#110

[2] R4-2400259 On harmonic mixing orders and analysis, Skyworks Solutions Inc., Nokia, RAN4#110

[3] R4-2400645 RX Mixing evaluations, Qualcomm France, RAN4#110

[4] R4-2403721 WF on Rel-19 Band Combination Work, Nokia, Skyworks, ZTE, Samsung, Ericsson, AT&T, Qualcomm, CHTTL, Murata, T-Mobile USA, Apple, MediaTek, Huawei, Verizon, RAN4#110

[5] R4-2404244 On valid harmonic mixing order, Skyworks Solutions Inc., Nokia, Qualcomm, RAN4#110bis