# 3GPP TSG-RAN WG4 Meeting # 110bis                                                     R4-2404271

# Changsha, China, 15-19 April , 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | - | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for TS 38.101-3 DC_R18_xBLTE_2BNR_yDL2UL without FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, KT, KT SAT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R18_xBLTE_2BNR_yDL2UL-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce the below mentioned band combinations: DC_3(n)AA_n77A DC_3(n)AA_n77(2A) DC_n3A-n77A_1A DC_n3A-n77(2A)_1A DC_n3A-n77A_8A DC_n3A-n77(2A)_8A DC_3(n)AA_n8A_1A DC_3(n)AA_n77A_1A DC_3(n)AA_n77(2A)_1A DC_3(n)AA_n77A_8A DC_3(n)AA_n77(2A)_8A DC_n8A-n77A_1A-3A DC_n77A-n3A_1A-8A DC_3(n)AA_n77A_1A-8A DC_3(n)AA_n77(2A)_1A-8A The fallbacks are proposed in the same meeting via R4-2404268 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the below mentioned band combinations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New configurations are not included in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4a.2, 5.5B.4a.3, 5.5B.4a.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

#### 5.5B.4a.2	Inter-band NE-DC configurations within FR1 (three bands)

Table 5.5B.4a.2-1: Inter-band NE-DC configurations within FR1 (three bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |  |
| --- | --- | --- |
| DC_n3A_1A-8A | DC_n3A_1A DC_n3A-8A |  |
| DC_3(n)AA_n77A | DC_3(n)AA2 DC_n77A _3A |  |
| DC_3(n)AA_n77(2A) | DC_3(n)AA2 DC_n77A _3A |  |
| DC_n3A-n8A_1A | DC_n3A_1A DC_n8A_1A |  |
| DC_n3A-n77A_1A | DC_n3A_1A DC_n77A_1A |  |
| DC_n3A-n77(2A)_1A | DC_n3A_1A DC_n77A_1A |  |
| DC_n3A-n77A_8A | DC_n3A_8A DC_n77A_8A |  |
| DC_n3A-n77(2A)_8A | DC_n3A_8A DC_n77A_8A |  |
| DC_n8A_1A-3A | DC_n8A_1A DC_n8A_3A |
| DC_n8A-n77A_1A | DC_n8A_1A DC_n77A_1A |  |
| DC_n8A-n77(2A)_1A | DC_n8A_1A DC_n77A_1A |  |
| DC_n8A-n77A_3A | DC_n8A_3A DC_n77A_3A |  |
| DC_n8A-n77(2A)_3A | DC_n8A_3A DC_n77A_3A |  |
| DC_3(n)AA_n8A | DC_3(n)AA1 DC_3A_n8A |  |
| DC_3(n)AA_n77A | DC_3(n)AA1 DC_3A_n77A |  |
| DC_n77A_1A-3A | DC_n77A_1A DC_n77A_3A |  |
| DC_n77(2A)_1A-3A | DC_n77A_1A DC_n77A_3A |  |
| DC_n77A_1A-8A | DC_n77A_1A DC_n77A_8A |  |
| DC_n77(2A)_1A-8A | DC_n77A_1A DC_n77A_8A |  |
| DC_n77A_3A-8A | DC_n77A_3A DC_n77A_8A |  |
| DC_n77(2A)_3A-8A | DC_n77A_3A DC_n77A_8A |  |
| DC_n78A_1A-3A DC_n78A_1A-3C | DC_n78A_1A DC_n78A_3A |  |
| DC_n78A_1A-5A | DC_n78A_1A DC_n78A_5A |  |
| DC_n78A_1A-7A | DC_n78A_1A DC_n78A_7A |  |
| DC_n78A_1A-7A-7A | DC_n78A_1A DC_n78A_7A |  |
| DC_n78A_1A-8A | DC_n78A_1A DC_n78A_8A |  |
| DC_n78A_3A-5A | DC_n78A_3A DC_n78A_5A |  |
| DC_n78A_3A-7A | DC_n78A_3A DC_n78A_7A |  |
| DC_n78A_3A-7A-7A | DC_n78A_3A DC_n78A_7A |  |
| DC_n78A_3A-8A DC_n78A_3C-8A | DC_n78A_3A DC_n78A_8A |  |
| DC_n78A_5A-7A | DC_n78A_5A DC_n78A_7A |  |
| DC_n78A_5A-7A-7A | DC_n78A_5A DC_n78A_7A |  |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported. |  |  |

#### 5.5B.4a.3	Inter-band NE-DC configurations within FR1 (four bands)

Table 5.5B.4a.3-1: Inter-band NE-DC configurations within FR1 (four bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n3A-n77A_1A-8A | DC_n3A_1A DC_n3A_8A DC_n77A_1A DC_n77A_8A |
| DC_3(n)AA_n8A_1A | DC_n3A_1A DC_3(n)AA2 DC_n8A_1A DC_n8A_3A |
| DC_3(n)AA_n77A_1A | DC_n3A_1A DC_3(n)AA2 DC_n77A_1A DC_n77A_3A |
| DC_3(n)AA_n77(2A)_1A | DC_n3A_1A DC_3(n)AA2 DC_n77A_1A DC_n77A_3A |
| DC_3(n)AA_n77A_8A | DC_3(n)AA2 DC_n3A_8A DC_n77A_3A DC_n77A_8A |
| DC_3(n)AA_n77(2A)_8A | DC_3(n)AA2 DC_n3A_8A DC_n77A_3A DC_n77A_8A |
| DC_n8A-n77A_1A-3A | DC_n8A_1A DC_n77A_1A DC_n8A_3A DC_n77A_3A |
| DC_n8A-n77(2A)_1A-3A | DC_n8A_1A DC_n77A_1A DC_n8A_3A DC_n77A_3A |
| DC_n77A_1A-3A-8A DC_n77(2A)_1A-3A-8A | DC_n77A_1A DC_n77A_3A DC_n77A_8A |
| DC_n77A-n3A_1A-8A | DC_n3A_1A DC_n3A_8A DC_n77A_1A DC_n77A_8A |
| DC_n78A_1A-3A-5A | DC_n78A_1A DC_n78A_3A DC_n78A_5A |
| DC_n78A_1A-3A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_7A |
| DC_n78A_1A-3A-7A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_7A |
| DC_n78A_1A-3A-8A DC_n78A_1A-3C-8A | DC_n78A_1A DC_n78A_3A DC_n78A_8A |
| DC_n78A_1A-5A-7A | DC_n78A_1A DC_n78A_5A DC_n78A_7A |
| DC_n78A_1A-5A-7A-7A | DC_n78A_1A DC_n78A_5A DC_n78A_7A |
| DC_n78A_3A-5A-7A | DC_n78A_3A DC_n78A_5A DC_n78A_7A |
| DC_n78A_3A-5A-7A-7A | DC_n78A_3A DC_n78A_5A DC_n78A_7A |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported. |  |

#### 5.5B.4a.4	Inter-band NE-DC configurations within FR1 (five bands)

Table 5.5B.4a.4-1: Inter-band NE-DC configurations within FR1 (five bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_3(n)AA_n77A_1A-8A | DC_n3A_1A DC_3(n)AA2 DC_n3A_8A DC_n77A_1A DC_n77A_3A DC_n77A_8A |
| DC_3(n)AA_n77(2A)_1A-8A | DC_n3A_1A DC_3(n)AA2 DC_n3A-8A DC_n77A_1A DC_n77A_3A DC_n77A_8A |
| DC_n78A_1A-3A-5A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_5A DC_n78A_7A |
| DC_n78A_1A-3A-5A-7A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_5A DC_n78A_7A |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported |  |