3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404288

Changsha, China, 15 – 19 April 2024

Agenda Item:	9.10.2

Source: 	Nokia

Title: 	UE RF specification impact for inter-band NR CA/DC with 3MHz CBW

Document for:	Approval

# 1.	Introduction

A new WI for NR channel BW less than 5MHz for FR1 Phase 2 was approved at TSG RAN#103 [1]. One of the objectives of this WI is to:

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band

  - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101

This contribution provides the necessary changes of the UE RF requirements in TS 38.101-1 [2] to support inter-band NR CA/DC with 3MHz CBW in band n100 and 5MHz or 10MHz CBW in band n101.

# 2.	Discussion

Table 1 lists all UE Tx and Rx requirements one by one and indicates which requirements need modification to support inter-band NR CA/DC with 3MHz CBW in band n100 and 5MHz or 10MHz CBW in band n101.

Table 1: Specification impact analysis

| NR UE Tx/Rx requirement | Required change for CA | Required change for NR-DC |
| --- | --- | --- |
| 6.2.1 UE maximum output power | Add a new row for CA_n100A-n101A in Table 6.2A.1.3-1. | Add a new row for DC_n100A-n101A in Table 6.2B.1.3-1. |
| 6.2.2 MPR | No specification impact | No specification impact |
| 6.2.3 A-MPR | No specification impact | No specification impact |
| 6.2.4 Configured transmitted power | Add a new row for CA_n100-n101 in Table 6.2A.4.2.3-1. | No specification impact |
| 6.3.1 Minimum output power | No specification impact | No specification impact |
| 6.3.2 Transmit OFF power | No specification impact | No specification impact |
| 6.3.3 Transmit ON/OFF time mask | No specification impact | No specification impact |
| 6.3.4 Power control | No specification impact | No specification impact |
| 6.4.1 Frequency error | No specification impact | No specification impact |
| 6.4.2.1 EVM | No specification impact | No specification impact |
| 6.4.2.2 Carrier leakage | No specification impact | No specification impact |
| 6.4.2.3 In-band emissions | No specification impact | No specification impact |
| 6.4.2.4 EVM equalizer spectrum flatness | No specification impact | No specification impact |
| 6.5.1 Occupied bandwidth | No specification impact | No specification impact |
| 6.5.2.2 Spectrum emission mask | No specification impact | No specification impact |
| 6.5.2.3 Additional Spectrum emission mask | No specification impact | No specification impact |
| 6.5.2.4.1 NR ACLR | No specification impact | No specification impact |
| 6.5.2.4.2 UTRA ACLR | No specification impact | No specification impact |
| 6.5.3.1 General spurious emissions | No specification impact | No specification impact |
| 6.5.3.2 Spurious UEtoUE co-ex | Add a new row for CA_n100-n101 in Table 6.5A.3.2.3-1 | No specification impact |
| 6.5.3.3 Additional spurious emissions | No specification impact | No specification impact |
| 6.5.4 Transmit intermodulation | No specification impact | No specification impact |
| 7.3.2 Reference sensitivity | Add a new row for CA_n100-n101 in Table 7.3A.3.2.1-1. | No specification impact |
| 7.4 Max input level | No specification impact | No specification impact |
| 7.5 Adjacent channel selectivity | No specification impact | No specification impact |
| 7.6.2 In-band blocking | No specification impact | No specification impact |
| 7.6.3 Out of band blocking | No specification impact | No specification impact |
| 7.6.4 Narrow band blocking | No specification impact | No specification impact |
| 7.7 Spurious response | No specification impact | No specification impact |
| 7.8 Intermodulation characteristics | No specification impact | No specification impact |

To summarise, a new row should be added in five tables (four for CA and one for DC) to support inter-band NR CA/DC with 3MHz CBW in band n100 and 5MHz or 10MHz CBW in band n101. The actual values in each of the five new rows (four for CA and one for DC) should be further discussed in the WI.

# 3.	Conclusion

This contribution has provided the necessary changes of the UE RF requirements in TS 38.101-1 [2] to support inter-band NR CA/DC with 3MHz CBW in band n100 and 5MHz or 10MHz CBW in band n101.

Proposal 1: To approve Table 1 for the required changes of the UE RF requirements in TS 38.101-1 to support inter-band NR CA/DC with 3MHz CBW in band n100 and 5MHz or 10MHz CBW in band n101.

Table 1: Specification impact analysis

| NR UE Tx/Rx requirement | Required change for CA | Required change for NR-DC |
| --- | --- | --- |
| 6.2.1 UE maximum output power | Add a new row for CA_n100A-n101A in Table 6.2A.1.3-1. | Add a new row for DC_n100A-n101A in Table 6.2B.1.3-1. |
| 6.2.2 MPR | No specification impact | No specification impact |
| 6.2.3 A-MPR | No specification impact | No specification impact |
| 6.2.4 Configured transmitted power | Add a new row for CA_n100-n101 in Table 6.2A.4.2.3-1. | No specification impact |
| 6.3.1 Minimum output power | No specification impact | No specification impact |
| 6.3.2 Transmit OFF power | No specification impact | No specification impact |
| 6.3.3 Transmit ON/OFF time mask | No specification impact | No specification impact |
| 6.3.4 Power control | No specification impact | No specification impact |
| 6.4.1 Frequency error | No specification impact | No specification impact |
| 6.4.2.1 EVM | No specification impact | No specification impact |
| 6.4.2.2 Carrier leakage | No specification impact | No specification impact |
| 6.4.2.3 In-band emissions | No specification impact | No specification impact |
| 6.4.2.4 EVM equalizer spectrum flatness | No specification impact | No specification impact |
| 6.5.1 Occupied bandwidth | No specification impact | No specification impact |
| 6.5.2.2 Spectrum emission mask | No specification impact | No specification impact |
| 6.5.2.3 Additional Spectrum emission mask | No specification impact | No specification impact |
| 6.5.2.4.1 NR ACLR | No specification impact | No specification impact |
| 6.5.2.4.2 UTRA ACLR | No specification impact | No specification impact |
| 6.5.3.1 General spurious emissions | No specification impact | No specification impact |
| 6.5.3.2 Spurious UEtoUE co-ex | Add a new row for CA_n100-n101 in Table 6.5A.3.2.3-1 | No specification impact |
| 6.5.3.3 Additional spurious emissions | No specification impact | No specification impact |
| 6.5.4 Transmit intermodulation | No specification impact | No specification impact |
| 7.3.2 Reference sensitivity | Add a new row for CA_n100-n101 in Table 7.3A.3.2.1-1. | No specification impact |
| 7.4 Max input level | No specification impact | No specification impact |
| 7.5 Adjacent channel selectivity | No specification impact | No specification impact |
| 7.6.2 In-band blocking | No specification impact | No specification impact |
| 7.6.3 Out of band blocking | No specification impact | No specification impact |
| 7.6.4 Narrow band blocking | No specification impact | No specification impact |
| 7.7 Spurious response | No specification impact | No specification impact |
| 7.8 Intermodulation characteristics | No specification impact | No specification impact |

# References

[1]	RP-240832, “New WID: WI resulting from discussion on NR channel BW less than 5MHz for TN”, Moderator (vivo).

[2]	3GPP TS 38.101-1 v18.5.0: “NR; User Equipment (UE) radio transmission and reception”.