3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2404304

Changsha, China, Apr 15-19, 2024

Agenda item:	6.19.4.1

Source:	Apple

Title:	On UE demod and CSI requirements for NR MIMO evolution

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#110 demodulation impact and requirements scope for MIMO evo were discussed and way forward [1] was agreed.  In this contribution we present our views on UE demodulation and CSI reporting requirements for MIMO evo.

# 2. Discussion

## Type II Doppler Codebook

In RAN4#110 the following agreements we made for PMI reporting requirements for Type II Doppler codebook:

Issue 1-1-1: clarify if introduce PMI reporting requirements for ‘typeII-Doppler-r18’ codebook Agreement: - Introduce PMI reporting requirements for ‘typeII-Doppler-r18’ codebook. Issue 1-1-3: clarify criteria of feasibility for ‘typeII-Doppler-r18’ codebook Agreement: - Use the mechanism agreement which has already agreed in RAN4#109 and CSI-RS setup in section 2.2.1. Issue 1-1-4: clarify test metric for PMI reporting requirements with ‘typeII-Doppler-r18’ codebook Agreement: - Test metric defined as γ1=ttypeII-dopplertrnd, where ttypeII-doppler is X % of the maximum throughput obtained at SNRtypeII-doppler using the typeII-Doppler-r18 precoder configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-doppler with random precoding based on Type I Single Panel codebook Issue 2-1-1: Propagation channel Agreement: - TDLA30-30 as baseline. Interested companies can also provide results with 20 and 40 Hz in addition to 30 Hz. Issue 2-1-3: MCS Agreement: - MCS13 (16QAM, 0.48) for PMI reporting requirements of typeII-Doppler-r18 codebook Issue 2-1-7: Beam steering modelling for TypeII-Doppler-r18 PMI reporting requirements Agreement: - Dual cluster beam steering for TypeII-Doppler-r18 PMI reporting requirements setup.

Issue 2-1-2: N4 and K configuration

Way forward:

- Option 1: N4=4 and K=4

- Option 2: N4=1 and K=4

- Other options are not precluded.

There are two different features with Type II Doppler codebook – CSI prediction (with N4=1) and CSI compression (with N4> 1). For R18 Type II Doppler codebook the baseline capability is with N4=1. N4=4 is additional UE capability. As we have seen from performance evaluation results from last meeting, there is not significant gain compared to R16 Type II codebook across most Doppler. With N4=4 it is more challenging to find the optimal Doppler that would give us the benefit of prediction over extended slots compared to N4=1 since we need to factor in the channel aging between the CSI measurement at UE and application at gNB.

- N4=1 is baseline UE capability for R18 Type II Doppler.

- With N4=4 it is more challenging to find a suitable Doppler that gives good performance

We propose to define requirements with N4=1.

Define requirements with N4=1.

Issue 2-1-4: X% of the maximum throughput in Test metric

Way forward:

- Option 1: 60%

- Option 2: 90%

- Other options are not precluded.

The test metric is based on random PMI with Type I codebook, so the performance gain observed with respect to R16 Type II codebook is no longer the criteria to define requirements. We prefer to keep the test metric the same as other PMI reporting requirements as 90% of max TP.

Define requirements for PMI reporting with Type II Doppler codebook with test metric at 90% of max TP.

## Type II CJT Codebook

In RAN4#110 the following agreements we made for PMI reporting requirements for Type II CJT codebook:

Issue 1-1-2: clarify if introduce PMI reporting requirements for ‘typeII-CJT-r18’ codebook Agreement: - Introduce PMI reporting requirements for ‘typeII-CJT-r18’ codebook. Issue 1-1-5: clarify test metric for PMI reporting requirements with ‘typeII-CJT-r18’ codebook Agreement: - For the PMI reporting requirements with typeII-CJT-r18, define the test metric as γcjt=ttypeII-CJTtrnd, where ttypeII-CJT is [90] % of the maximum throughput obtained at SNRtypeII-CJT using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-CJT with random precoding based on type I Single Panel codebook per TRP. Issue 1-1-6: clarify how to approach random PMI precoding in mTRP case for Test metric defined as γ3=ttypeII-CJTtrnd Agreement: - For each TRP, use random type I SP PMI precoding independently. Issue 2-2-1: Propagation channel and correlation configuration Agreement: - Use TDLA30-10 with XP high as the propagation channel and correlation configuration for Rel-18 TypeII for CJT test. Issue 2-2-3: paramCombination-CJT-L-r18 Agreement: - Set paramCombination-CJT-L-r18 as 7 ({4, 4}) Issue 2-2-4: paramCombination-CJT-r18 Agreement: - Set paramCombination-CJT-r18 as 4 (pν=14and18, β=12)

Issue 2-2-2: N1, N2, O1, O2 and the number of CSI-RS ports

Way forward:

- Option 1: Set PCSI-RS=8 CSI-RS ports per TRP with (N1, N2) = (4, 1), (O1, O2) = (4, 1) for Rel-18 TypeII for CJT PMI test.

- Other options are not precluded.

We propose to confirm option 1 for the Tx configuration.

Set PCSI-RS=8 CSI-RS ports per TRP with (N1, N2) = (4, 1), (O1, O2) = (4, 1) for Rel-18 TypeII for CJT PMI test

Issue 2-2-5: RI restriction (typeII-CJT-RI‑Restriction-r18)

Way forward:

- Option 1: Set RI restriction as 0001 for Rel-18 TypeII for CJT PMI test.

- Option 2: Set RI restriction as 0010 for Rel-18 TypeII for CJT PMI test.

With 8 ports per TRP, we think it would be reasonable to define dual layer PMI reporting requirements for Type II CJT codebook. With dual layer the RI restriction should be 0010.

Define requirements with 2 layers for Rel-18 TypeII for CJT PMI

Set RI restriction as 0010 for Rel-18 TypeII for CJT PMI test

Issue 2-2-10: TRS configuration in CJT

Way forward:

- Option 1: One TRS for both TRPs

- Option 2: separate TRS for each TRP

CJT multi-TRP transmission is not an enhanced multi-TRP transmission scheme. The UE is receiving from 2 TRPs, and unless configured with Type II CJT codebook it is transparent to the UE.  We expect the same signals and data to be transmitted from both the TRPs. Hence the same TRS should be transmitted as well from both TRPs.

Configure same TRS for both TRP for Rel-18 TypeII for CJT PMI test

Issue 2-2-11: Beam steering modelling for TypeII-CJT-r18 PMI reporting requirements

Way forward:

- Option 1: principle beam direction specified in Annex B.2.3.2.3

- Option 2: dual cluster beams defined in Annex B.2.3.2.3A

For Type II codebook we use dual cluster beam steering, hence we should use the same for R18 Type II CJT MI reporting requirements.

Use dual cluster beam steering for Rel-18 TypeII for CJT PMI test

## DMRS Enhancements

In RAN4#110 the following agreements we made for UE demod/ CSI requirements for DMRS enhancements:

Issue 2-3-1: DMRS ports Agreement: - {1008, 1009} for Rank 2 test; {1000, 1001, 1008, 1009} for Rank 4 test Issue 2-3-2: Duplex mode for tests need to be defined for Rel-18 DMRS Agreement: - both FDD and TDD Issue 2-3-3: Number of Rx for tests need to be defined for Rel-18 DMRS Agreement: - both 2Rx and 4Rx Issue 2-3-4: Cases need to be defined for FR1 Rel-18 DMRS Agreement: - For Rank 2 with 2Rx, Test 2-1 in Chapter 5.2.2.1.1, 5.2.2.2.1 - For Rank 4 with 4Rx, Test 4-1 in Chapter 5.2.3.1.1, 5.2.3.2.1 Issue 2-3-5: Cases need to be defined for FR2-1 Rel-18 DMRS Agreement: - No need to define cases for FR2-1 Rel-18 DMRS.

One open issue to further discuss is the requirements for with R19 eDMRS -

Issue 2-3-6: Minimum requirements for tests need to be defined for Rel-18 DMRS

Way forward:

- Option 1: reuse legacy value

- Option 2: new value according simulation results

- Other options are not precluded

We acknowledge that the performance can be the same between R15 DMRS and R18 eDMRS for the parameters considered for the test case. Since we are defining requirements with new DMRS configuration in R18, we prefer to define requirements based on new simulation results.

Define new requirements based on new simulation results for PDSCH demod with R18 eDMRS.

Applicability Rule

Since the requirements are based on legacy test cases, we can introduce an applicability rule that states that if UE supports enhanced DMRS, it can skip the corresponding tests with legacy DMRS configuration. The test configuration is the same for the rank 2 tests between R18 eDMRS and R15 DMRS. But for rank 4 test with eDMRS the test configuration is different from the R15 test case with rank 4. Hence the applicability rule can only apply to Rank 2 tests.

Introduce applicability rule that UE supporting enhanced DMRS needs to only be tested for new requirements for rank 2 with enhanced DMRS and can skip the corresponding tests for rank 2 with legacy DMRS configuration.

Simulation Results

For the agreed simulation parameters in [1] we provide simulation results for enhanced DMRS in the table below.

Table 1: Simulation Results with enhanced DMRS

| Duplex | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | SNR @ 70% Max TP (dB) |
| --- | --- | --- | --- | --- | --- |
| FDD | 10MHz / 15kHz | 64QAM, 0.50 Rank 2 | TDLA30-10 | 2x2, ULA Low | 17.1 |
| TDD | 40MHz / 30kHz | 64QAM, 0.50 Rank 2 | TDLA30-10 | 2x2, ULA Low | 17.4 |
| FDD | 10MHz / 15kHz | 16QAM, 0.48 Rank 4 | TDLA30-10 | 4x4, ULA Low | 13.2 |
| TDD | 40MHz / 30kHz | 16QAM, 0.48 Rank 4 | TDLA30-10 | 4x4, ULA Low | 13.5 |

# 3. Conclusion

In this paper, we provide our views on open issues on the scope for UE demodulation ad CSI reporting requirements for R18 MIMO evolution. Our observations and proposals are captured below:

Type II Doppler Codebook

- N4=1 is baseline UE capability for R18 Type II Doppler.

- With N4=4 it is more challenging to find a suitable Doppler that gives good performance

- Define requirements with N4=1.

Define requirements for PMI reporting with Type II Doppler codebook with test metric at 90% of max TP.

Type II CJT Codebook

Set PCSI-RS=8 CSI-RS ports per TRP with (N1, N2) = (4, 1), (O1, O2) = (4, 1) for Rel-18 TypeII for CJT PMI test

Define requirements with 2 layers for Rel-18 TypeII for CJT PMI

Set RI restriction as 0010 for Rel-18 TypeII for CJT PMI test

Configure same TRS for both TRP for Rel-18 TypeII for CJT PMI test

Use dual cluster beam steering for Rel-18 TypeII for CJT PMI test

DMRS Enhancements

Define new requirements based on new simulation results for PDSCH demod with R18 eDMRS.

Introduce applicability rule that UE supporting enhanced DMRS needs to only be tested for new requirements for rank 2 with enhanced DMRS and can skip the corresponding tests for rank 2 with legacy DMRS configuration.

# Reference

[1]. R4-2402868, “Way forward on [110][323] NR_MIMO_evo_DL_UL_demod”, Samsung