3GPP TSG-RAN WG4 Meeting #110bis 					                     R4-2404314

Changsha, China, April 15 – 19, 2024

Agenda item:	6.6.1

Source:	Apple

Title:	Discussion on RRM core requirements for bandwidth part operation without restriction

Document for:	Discussion

# Introduction

RRM requirements of BWP without restriction were extensively discussed in previous RAN4 meetings. The latest agreements can be found in [1]. However, there are still some open issues left. In this contribution, we continue discussing the open issues.

# Discussion

#### Issue 1-5: Configuration of SSB resources within CSI-ResourceConfig settings for option B-1-1

- LS RAN2 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1.

It is our understanding that SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1, since we couldn’t find any restriction in RAN1/2 spec that this configuration is not allowed. Since CSI-ResourceConfig is configured per BWP, UE only needs to measure target SSB which is associated to the active BWP, although it is outside active BWP. However, current wordings in RAN4 spec are misleading (yellow highlighted below). ‘for the serving cell’ may have the implication that UE needs to measure CSI-ResourceConfig associated with other inactive BWP, since they also belong to the serving cell. On the other hand, it’s better to clarify that is only for SSB, since current wording may imply CSI-RS is also supported.

### 9.5.1 Introduction When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements within the active BWP. For UE supporting bwpOperationMeasWithoutInterrupt-r18, the measurements shall also be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements outside the active BWP. For UE supporting ncd-SSB-BWP-Wor-r18, the SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified. If SSB in the active DL BWP of serving cell i is NCD-SSB, for serving cell i the requirements in clause 8.1 apply provided that serving cell i is PCell. The UE shall be able to measure all CSI-RS resources and/or SSB resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet within the CSI-ResourceConfig settings configured for L1-RSRP for the active BWP, or for the serving cell if the UE supports bwpOperationMeasWithoutInterrupt-r18, provided that the number of resources, including the number of SSB resources of the cell with PCI different from serving cell configured for L1-RSRP measurements in 9.13, does not exceed the UE capability indicated by beamManagementSSB-CSI-RS. The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the reportConfigType according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP, or for the serving cell if the UE supports bwpOperationMeasWithoutInterrupt-r18. The measurement reporting delay can be longer for the measurement reporting requirements in this clause when IDC autonomous denial is configured.

Proposal 1: update FG53-1 related clarification in L1 RSRP measurement requirements.

#### Issue 1-1: Requirements/UE behaviour for UE supporting both option B-1-1 and A

- FFS following options

  - Option 1a:

    - For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour.

  - Option 1b:

    - For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

  - Option 2:

    - For UE supporting both option B-1-1 and A, if CSI-RS is configured within the active BWP for L1 measurements, then UE is NOT expected to perform RLM/BFD/BM based on CD-SSB outside active BWP.

    - It is clarified in the SSB based L1 measurement requirements that if UE supports both option B-1-1 and option A then the UE is not required to measure the SSB if it is outside active BWP.

We believe it is unnecessary for UE to measure both CSI-RS and SSB for RLM/BM/BFD simultaneously even if UE supports both option A and option B-1-1. On the other hand, network does not need to configure both option A and B-1-1 simultaneously. For instance, if network wants UE to perform L1 operations based CD-SSB (B-1-1), network doesn’t need to configure CSI-RS based L1 operation. This is doable since anyway it is per-UE configuration. If network wants UE to rely on CSI-RS for L1 operation, network can configure CSI-RS for each candidate BWP and not configure CD-SSB as RLM-RS. Thus, this issue can be avoided.

Proposal 2: For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour.

#### Issue 1-3: Requirements/UE behaviour for UE supporting both option C and B-1-1

- FFS following options

  - Option 1:

    - For UE supporting both option B-1-1 and C, if both CD-SSB and NCD-SSB are configured outside active BWP, then UE is expected to perform RLM/BFD/BM following NW configuration based on TS 38.331. No additional requirements will be defined for such scenario.

  - Option 2:

    - For UE supporting both option B-1-1 and C, if the active BWP that does not contain CD- or NCD-SSB, UE should perform L1 measurement on CD-SSB. No additional requirements will be defined for such scenario.

We see two candidate solutions. One is to assume UE should perform L1 measurement on CD-SSB (option 2). This is simpler. However, this doesn’t always provide the most accurate result.

For instance, in above example if BWP4 is the active BWP, NCD-SSB can provide more accurate result than CD-SSB since it is closer. Alternatively, we can let network to decide which one to use for UE supporting both option B-1-1 and option C.

Proposal 3: For UE supporting both option B-1-1 and C, if the active BWP that does not contain CD- or NCD-SSB

- Option 1: NW should indicate whether CD- or NCD-SSB should be used for UE supporting both option B-1-1 and option C when active BWP contains neither CD- nor NCD-SSB

- Option 2: UE should perform L1 measurement on CD-SSB.

# Conclusion

In this contribution, we provide discussion on general apsect of BWP without restriction. After discussion, the following conclusions are provided:

Proposal 1: update FG53-1 related clarification in L1 RSRP measurement requirements.

Proposal 2: For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour.

Proposal 3: For UE supporting both option B-1-1 and C, if the active BWP that does not contain CD- or NCD-SSB

- Option 1: NW should indicate whether CD- or NCD-SSB should be used for UE supporting both option B-1-1 and option C when active BWP contains neither CD- nor NCD-SSB

- Option 2: UE should perform L1 measurement on CD-SSB.

# References

- R4-2403544, WF on core maintenance and performance part for BWP without restriction, vivo, Vodafone

- R4-2321607, WF on BWP without restriction, vivo, Vodafone

- R4-2317337, WF on support for bandwidth part operation without restriction, vivo

- R4-2314339, WF on support for bandwidth part operation without restriction, vivo

- RP-232677, Further revised WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, vivo