3GPP TSG-RAN WG4 Meeting #110bis 					                     R4-2404320

Changsha, China, April 15 – 19, 2024

Agenda item:	6.14.1.2

Source:	Apple

Title:	Discussion on core part maintenance of improvement on SCG/Scell setup/resume delay

Document for:	Discussion

# Introduction

Core part maintenance of FR2 SCell/SCG setup delay have been discussed widely in the previous RAN4 meeting. The latest agreement can be found in the approved WF [1]. In this contribution, we will continue discussing the remaining issues.

# Discussion

Issue 1-1: UE feature list of R18 Further NR mobility enhancement part 2

Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-x1 | Measurement validation based on EMR measurement during connection setup/resume | Indicate UE supporting measurement validation based on EMR measurement during IDLE/INACTIVE state and reporting for fast CA/DC setup | idleInactiveNR-MeasReport-r16 | Yes | N/A | UE does not support EMR measurement validation during connection setup/resume and reporting for fast CA/DC | Per-UE | No | FFS | N/A |  | Optional with capability signaling |
|  | 39-x2 | Measurement validation based on non-EMR measurement during connection setup/resume | Indicate UE supporting measurement validation based on non-EMR measurement during IDLE/INACTIVE state and reporting for fast CA/DC setup |  | Yes | N/A | UE does not support non-EMR measurement validation during connection setup/resume and reporting for fast CA/DC | Per-UE | No | FFS | N/A |  | Optional with capability signaling |

As can be seen in the agreed UE feature list above, it is FFS in the column of Need of FR1/FR2 differentiation. The reason is that originally this objective targeted at FR2. Some company proposed to extend this to cover FR1 as well. In previous RAN4 meeting, RAN4 agreed the following scope in RAN4#106:

- Agreement

  - RAN4 shall focus on inter-band target cell in FR2. If final solution to be agreed can cover intra-band and FR1 without extra standardization effort, it is unnecessary to exclude these two scenarios.

Actually, the agreed solution (validity check based on configured new timer) can apply to both FR1 and FR2 without any additional change. We also went through new requirements introduced in this objective in TS38.133 clause 4.7 and 5.8 and found that the requirements are quite general and can cover target cell in both FR1 and FR2. Therefore, we propose to allow differentiation between FR1/FR2 in the new UE capabilities.

Proposal 1: allow differentiation between FR1 and FR2 for the new UE capabilities 39-x1 and 39-x2.

# Conclusion

In this contribution, we provide further discussion on improvement on FR2 SCell/SCG setup delay. After discussion, the following conclusions are provided:

Proposal 1: allow differentiation between FR1 and FR2 for the new UE capabilities 39-x1 and 39-x2.

# References

- R4-2403548, WF on R18 Further NR mobility enhancement – part 2, Apple

- R4-2321641, Big CR to TS 38.133 on Further NR mobility enhancement, MediaTek Inc., Apple

- RP-233969, Rel-18 Work Item Exception for Further NR Mobility Enhancements, Rapporteur (MediaTek Inc., Apple)

- R4-2317328, WF on R18 Further NR mobility enhancement – Improvement on SCell/SCG setup delay and Enhanced CHO, Apple

- R4-2314332, WF on NR Mobility Enhancements (Part 2), Apple

- R4-2310064, WF on NR Mobility Enhancements RRM requirements (part 2), Apple

- R4-2306395, WF on NR Mobility Enhancements RRM requirements (part 2), Apple

- R4-2220415, WF on improvement on FR2 SCell/SCG setup/resume, Apple, MediaTek Inc.