3GPP TSG-RAN WG4 Meeting #110bis	R4-2404385

# Changsha, China, April 15-19, 2024

Source:	T-Mobile USA

Title:	TP for TR38.850: FDD PC2 for n71 with DL CA_n71B

Agenda item:	5.18.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 n71 for DL CA_n71B.

There is no MSD for CA_n71B in 38.101-1 because the max UL was 20 MHz so the Tx impairments don’t hit the SCC DL. However, since 25, 30 and 35 MHz channel bandwidths were added for n71, and BCS4 and 5 would allow for 25 and 30 MHz PCells, it is necessary to consider 25 and 30 MHz UL n71 for DL CA_n71B.

In other PC2 and PC1.5 MSD analysis, T-Mobile USA has been using calculations that increase the Interference by 3dB, but not the noise. This contribution proposes new MSD values based on this approach for single Tx.

- Reference

[1]

- Text Proposal

<Start of text proposal>

## 5.x	CA_ n71B

### 5.x.1	UE maximum output power

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n71B | n713 | 5 | 20 |  |  |  | 25 | 0 |
|  |  | 10 | 15 |  |  |  |  |  |
|  |  | 10 | 20 |  |  |  | 35 | 1 |
|  |  | 15 | 15, 20 |  |  |  |  |  |
|  |  | 5, 10, 15 | 15, 20 |  |  |  | 35 | 2 |
|  |  | See n71 channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  | 35 | 4 and 5 |

### 5.x.2	Reference sensitivity requirements

#### 5.x.2.0	General

For PC3, CA_n71B does not have self-interference for UL n71. This section will examine the existing PC3 MSD and propose MSD for PC2 FDD.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n71 without TxD

For PC3 CA_n71B, there is currently no MSD defined. However, there is a proposal for MSD for MSD for PCC channel bandwidths >20 MHz. This is the proposed configuration and MSD for UL n71 with PC3.

Table 7.3A.2.2-1: Intra-band contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) |  | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71B |  |  |  |  |  |  | FDD |

For PC3 MSD we have N+IPC3. For PC2 MSD we have N+IPC2. So, for PC2 compared to PC3, I increases by 3 dB. For our other PC2 and PC1.5 MSD analysis we have been using the following approach:

MSD due to interference power I is given by:

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have:

If I is increased by X dB, then MSD(X) is given by

Using that approach, the following is proposed as a the PC2 MSD which would require a new table in 38.101-1:

Table 7.3A.2.2-1a: Intra-band contiguous CA with one PC2 uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71B |  |  |  |  |  | FDD |

#### 5.x.2.2	Reference sensitivity requirements with PC2 on n71 with TxD

[TBD]