3GPP TSG-RAN WG4 Meeting #110bis				                                        R4-2404398

Changsha, China, 15th – 19th April, 2024

Title:	Discussion on performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG

Source:	CATT

Agenda Item:	6.5.3

Document for:	Discussion

1 Introduction

In last meeting, the test cases for pre-configured MGs, concurrent MGs and NCSG were discussed and the conclusions were captured in the approved WF [1]. In this paper, we further discuss the test cases including list and test configurations, and present our views.

2 Discussion

In last meeting, the following agreements were made for the test cases:

Issue 4-1-1: Which general configuration shall be defined for the test cases? < Agreement from online session > - support the following principles for case 1 and case 2: - Only define test case in NR SA in both FR1 and FR2: - Do not introduce the test for L1 impact: - Define a minimum set of test cases for SSB-based measurement: - Only define test case under non-DRX: - Define test case without SBI reporting: - On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations: - Do not define test cases with simultaneously FR1 and FR2 gaps configured: - Test cases are limited to single serving carrier: - Only use mandatory gap patterns to define test cases: - Verify gap dropping behaviour without introducing additional test cases: - For case 1 and case 2, Define test for partial overlap, and fully non-overlap is used if no core requirements for partial overlap in some cases. ### Sub-topic 4-2: Test cases for Case 1 Issue 4-2-1: [Case 1] Which scenarios shall RAN4 RRM define test cases for concurrent MG with pre-MG Agreement: - The following scenarios be considered in the test cases: - Pre-MG + Type-2 MG - Pre-MG + Pre-MG - Under the following conditions: - Simultaneous multiple activation, FFS non-simultaneous multiple activation - Without dynamic collision - Dynamic collision with Pre-MG + Type-2 MG - FFS other scenarios, options for further discussion: - Pre-MG + Type-1 MG Issue 4-3-1: [Case 2] Which scenarios shall RAN4 RRM define test cases for NCSG and concurrent MG (Case 2) <Tentative agreement> The following scenarios be considered in the test cases: - NCSG + Type-2 MG - NCSG + NCSG - FFS other scenarios, options for further discussion: - NCSG + Type-1 MG

Based on the agreements above, the existing test cases for concurrent MG with partially overlapping can be used as baseline. To simplify the test cases, inter-frequency measurement can be used which is same as legacy test cases.

For the combination of concurrent MG and NCSG, there is no much difference for NCSG + Type-2 MG and NCSG + NCSG, so there is no need to further define test case for NCSG + NCSG. For the combination of Pre-MG/NCSG with Type-1 MG, there are no requirements when colliding and when the two gaps are fully non-overlapped, the case is same as the combination of Pre-MG/NCSG with Type-2 MG which is verified in existing test cases. So it is unnecessary to define test cases for NCSG + Type-1 MG and Pre-MG + Type-1 MG.

Proposal 1: Define test cases for inter-frequency measurement.

Proposal 2: Do not define test cases for NCSG + NCSG.

Proposal 3: Do not define test case for NCSG + Type-1 MG and Pre-MG + Type-1 MG.

In summary, the test case list is defined as below:

| No. # | Item of core requirements | Test purpose | MG configuration |
| --- | --- | --- | --- |
| Con-Pre-MG TC1 | inter-frequency in FR1 with concurrent gap and autonomous activation/deactivation of Pre-MG + Type-2 | - Measurement delay with Pre-MG and Type-2 MG in FR1 - Autonomous (de)activation delay - Collision handling | Type-2 MG: pattern#0 with offset 39 Pre-MG: pattern#1 with offset 79 |
| Con-Pre-MG TC2 | inter-frequency in FR2 with concurrent gap and network-controlled activation/deactivation of Pre-MG + Type-2 | - Measurement delay with Pre-MG and Type-2 MG in FR2 - Network-controlled (de)activation delay - Collision handling | Type-2 MG: pattern#12 with offset 39 Pre-MG: pattern#13 with offset 79 |
| Con-Pre-MG TC3 | inter-frequency in FR2 with concurrent gap with Pre-MG and autonomous activation/deactivation of two Pre-MG for FR2 | - Measurement delay with two Pre-MG in FR2 - Autonomous (de)activation delay including simultaneous and non-simultaneous | Pre-MG1: pattern#12 with offset 39 Pre-MG2: pattern#13 with offset 79 |
| Con-Pre-MG TC4 | inter-frequency in FR1 with concurrent gap with Pre-MG and network-controlled activation/deactivation of two Pre-MG for FR1 | - Measurement delay with two Pre-MG in FR1 - Network-controlled (de)activation delay including simultaneous and non-simultaneous | Pre-MG1: pattern#0 with offset 39 Pre-MG2: pattern#1 with offset 79 |
| Con-NCSG TC1 | inter-frequency in FR1 with NCSG + Type-2 | - Measurement delay with NCSG and Type-2 MG in FR1 - Collision handling | Type-2 MG: pattern#0 with offset 39 NCSG: pattern#1 with offset 79 |
| Con-NCSG TC2 | inter-frequency in FR2 with concurrent gap and network-controlled activation/deactivation of NCSG + Type-2 | - Measurement delay with NCSG and Type-2 MG in FR2 - Collision handling | Type-2 MG: pattern#12 with offset 39 NCSG: pattern#13 with offset 79 |

3 Summary

In this paper, we have some further discussions on the performance requirements for pre-configured MGs, concurrent MGs and NCSG, and the following proposals are given:

Proposal 1: Define test cases for inter-frequency measurement.

Proposal 2: Do not define test cases for NCSG + NCSG.

Proposal 3: Do not define test case for NCSG + Type-1 MG and Pre-MG + Type-1 MG.

In summary, the test case list is defined as below:

| No. # | Item of core requirements | Test purpose | MG configuration |
| --- | --- | --- | --- |
| Con-Pre-MG TC1 | inter-frequency in FR1 with concurrent gap and autonomous activation/deactivation of Pre-MG + Type-2 | - Measurement delay with Pre-MG and Type-2 MG in FR1 - Autonomous (de)activation delay - Collision handling | Type-2 MG: pattern#0 with offset 39 Pre-MG: pattern#1 with offset 79 |
| Con-Pre-MG TC2 | inter-frequency in FR2 with concurrent gap and network-controlled activation/deactivation of Pre-MG + Type-2 | - Measurement delay with Pre-MG and Type-2 MG in FR2 - Network-controlled (de)activation delay - Collision handling | Type-2 MG: pattern#12 with offset 39 Pre-MG: pattern#13 with offset 79 |
| Con-Pre-MG TC3 | inter-frequency in FR2 with concurrent gap with Pre-MG and autonomous activation/deactivation of two Pre-MG for FR2 | - Measurement delay with two Pre-MG in FR2 - Autonomous (de)activation delay including simultaneous and non-simultaneous | Pre-MG1: pattern#12 with offset 39 Pre-MG2: pattern#13 with offset 79 |
| Con-Pre-MG TC4 | inter-frequency in FR1 with concurrent gap with Pre-MG and network-controlled activation/deactivation of two Pre-MG for FR1 | - Measurement delay with two Pre-MG in FR1 - Network-controlled (de)activation delay including simultaneous and non-simultaneous | Pre-MG1: pattern#0 with offset 39 Pre-MG2: pattern#1 with offset 79 |
| Con-NCSG TC1 | inter-frequency in FR1 with NCSG + Type-2 | - Measurement delay with NCSG and Type-2 MG in FR1 - Collision handling | Type-2 MG: pattern#0 with offset 39 NCSG: pattern#1 with offset 79 |
| Con-NCSG TC2 | inter-frequency in FR2 with concurrent gap and network-controlled activation/deactivation of NCSG + Type-2 | - Measurement delay with NCSG and Type-2 MG in FR2 - Collision handling | Type-2 MG: pattern#12 with offset 39 NCSG: pattern#13 with offset 79 |

4 References

- R4-2403542, WF on NR_MG_enh2_part1, MediaTek Inc, RAN4#110