3GPP TSG-RAN WG4 Meeting #110bis	                                                       R4-2404403

Changsha, China, 15 April – 19 April, 2024

Title:	Discussion on performance requirements for sidelink positioning

Source:	CATT

Agenda Item:	6.12.3.1

Document for:	Discussion

1 Introduction

In last meeting, the performance requirements for sidelink positioning were discussed and the conclusions were captured in [1]. In this paper, we further discuss the performance requirements for sidelink positioning and give our proposals.

2 Discussion

In last meeting, the number of samples for sidelink positioning was updated as below:

Agreements: - Update the definition of Nsample as the following: - Nsample = 1 for SL-PRS BW > 48 PRBs, - Nsample = 4 for SL-PRS BW ≤ 48 PRBs - FFS whether for 48 PRBs SL-PRS BW a lower number of samples can be used based on performance results.

Based on our simulation results in [2], we see a little improvement when the SINR increases. But the results at -6dB is already acceptable. With the above agreement, for the smaller bandwidth, 4 samples are assumed which further compensates the drawback of bandwidth. We think the side condition for SL RSTD can be defined as [-3, -6]dB which is the same as the legacy requirements for reduced samples.

For the other measurements (SL PRS based RSRP/RSRPP/UE Rx-Tx), single set of requirements is enough and the side condition can be defined as -6dB.

Proposal 1: The side condition for SL RSTD is defined as [-3, -6]dB for reference cell and neighbor cell.

Proposal 2: The side condition for SL PRS RSRP/RSRPP and SL UE Rx-Tx is defined as -6dB.

Similar as the legacy measurements, based on the simulation results summary in [3], the accuracy requirements can be defined based on different groups of SL PRS bandwidth. But in legacy requirements, we have too many groups for the accuracy requirements and the groups between different measurements or different samples are different. It turns out such detailed group makes the test case design very complicated and hard to maintain once problems are identified in RAN5. Taking the experience in Rel-16 and Rel-17, we suggest reducing the number of sets of accuracy requirements and aligning the groups for different measurements as much as possible.

Proposal 3: When defining accuracy requirements, try to reduce the number of sets and align the grouping for different measurements as much as possible.

Based on the principle above and the simulation results, for SLPRS based measurements, we suggest the following structure to define the accuracy requirements:

- Table 1: SL RSTD/RSRP/RSRPP/UE Rx-Tx accuracy in FR1

| Accuracy | SL PRS Ês/Iot | SL PRS SCS | SL PRS bandwidth | Nsample |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | RB |  |
| [TBD] | For RSTD: (Ês/Iot)ref ≥-3dB (Ês/Iot)i ≥-6dB For other measurements: (Ês/Iot)i ≥-6dB | 15 | ≤ 48 | 4 |
|  |  |  | > 48 | 1 |
| [TBD] |  | 30 | ≤ 48 | 4 |
|  |  |  | > 48 | 1 |
| [TBD] |  | 60 | ≤ 48 | 4 |
|  |  |  | > 48 | 1 |

Proposal 4: Take table 1 as the structure for SL PRS based measurement accuracy requirements.

With the structure above, one set of requirements applies to all the comb configurations including different comb size and including fully staggered and partially staggered (half comb) SL PRS.

Proposal 5: One set of requirements is defined for all the comb configurations including both fully staggered and partially staggered (half comb).

Since the SL PRS based measurement is defined for single carrier, no need to consider RF calibration margin but the clock drift margin is possible when defining accuracy requirements. The accuracy requirements can be defined based on the simulation results and a small implementation margin.

Proposal 6: The accuracy requirements can be defined based on the simulation results and a small implementation margin (clock drift).

In last meeting, it was agreed to define accuracy requirements for SL-PRS based RSTD, SL-PRS based UE Rx-Tx time difference, SL-PRS based RSRP and SL-PRS based RSRPP. And do not define accuracy requirements for SL PRS based RTOA and AOA/ZOA measurement in Rel-18.

Based on the defined core requirements and accuracy requirements, and considering SL RSRP and RSRPP measurement is not reported independently, the following test cases are needed for sidelink positioning:

Table 2: Test case list for sidelink positioning

| Set | SL PRS measurement test case scenarios | Impacted section in TS 38.133 |
| --- | --- | --- |
| SL PRS measurement delay tests in RRC_CONNECTED |  |  |
| 1-1 | SL RSTD together with SL RSRP reporting delay test case in FR1 |  |
| 1-2 | SL UE Rx-Tx reporting delay test case in FR1 |  |
| 1-3 | SL RTOA reporting delay test case in FR1 |  |
| 1-4 | SL AOA reporting delay test case in FR1 |  |
| SL PRS measurement accuracy tests in RRC_CONNECTED |  |  |
| 2-1 | SL RSTD together with SL RSRP accuracy test case in FR1 |  |
| 2-2 | SL UE Rx-Tx accuracy test case in FR1 |  |

Proposal 7: Define test cases as listed in table 2.

For the test configurations, we need to decide the SL PRS pattern. For other configurations, the existing V2X test cases can be used as baseline. Based on the suggested accuracy requirements structure, the following SL PRS configurations can be used for test:

Table 3: SL PRS configurations for test

| PRS Parameters | Values |
| --- | --- |
| SL PRS transmission for different SL UE | TDMed in different slots |
| SCS | 15kHz, 30kHz |
| PRS comb size | 4, 2 |
| Number of PRS symbol | 4 |
| SL PRS resource time gap (slot) | 1 |
| RB numbers containing PRS within channel BW | 48，96 |
| Resource pool | dedicated |
| Note 1: Unless otherwise specified in the test case |  |

Proposal 8: Define SL PRS configurations for test as listed in table 3 and the other configurations can use the existing V2X test as baseline.

3 Summary

In this paper, we discuss the performance requirements on sidelink positioning and the following proposals are given:

Proposal 1: The side condition for SL RSTD is defined as [-3, -6]dB for reference cell and neighbor cell.

Proposal 2: The side condition for SL PRS RSRP/RSRPP and SL UE Rx-Tx is defined as -6dB.

Proposal 3: When defining accuracy requirements, try to reduce the number of sets and align the grouping for different measurements as much as possible.

Proposal 4: Take table 1 as the structure for SL PRS based measurement accuracy requirements.

- Table 1: SL RSTD/RSRP/RSRPP/UE Rx-Tx accuracy in FR1

| Accuracy | SL PRS Ês/Iot | SL PRS SCS | SL PRS bandwidth | Nsample |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | RB |  |
| [TBD] | For RSTD: (Ês/Iot)ref ≥-3dB (Ês/Iot)i ≥-6dB For other measurements: (Ês/Iot)i ≥-6dB | 15 | ≤ 48 | 4 |
|  |  |  | > 48 | 1 |
| [TBD] |  | 30 | ≤ 48 | 4 |
|  |  |  | > 48 | 1 |
| [TBD] |  | 60 | ≤ 48 | 4 |
|  |  |  | > 48 | 1 |

Proposal 5: One set of requirements is defined for all the comb configurations including both fully staggered and partially staggered (half comb).

Proposal 6: The accuracy requirements can be defined based on the simulation results and a small implementation margin (clock drift).

Proposal 7: Define test cases as listed in table 2.

Table 2: Test case list for sidelink positioning

| Set | SL PRS measurement test case scenarios | Impacted section in TS 38.133 |
| --- | --- | --- |
| SL PRS measurement delay tests in RRC_CONNECTED |  |  |
| 1-1 | SL RSTD together with SL RSRP reporting delay test case in FR1 |  |
| 1-2 | SL UE Rx-Tx reporting delay test case in FR1 |  |
| 1-3 | SL RTOA reporting delay test case in FR1 |  |
| 1-4 | SL AOA reporting delay test case in FR1 |  |
| SL PRS measurement accuracy tests in RRC_CONNECTED |  |  |
| 2-1 | SL RSTD together with SL RSRP accuracy test case in FR1 |  |
| 2-2 | SL UE Rx-Tx accuracy test case in FR1 |  |

Proposal 8: Define SL PRS configurations for test as listed in table 3 and the other configurations can use the existing V2X test as baseline.

Table 3: SL PRS configurations for test

| PRS Parameters | Values |
| --- | --- |
| SL PRS transmission for different SL UE | TDMed in different slots |
| SCS | 15kHz, 30kHz |
| PRS comb size | 4, 2 |
| Number of PRS symbol | 4 |
| SL PRS resource time gap (slot) | 1 |
| RB numbers containing PRS within channel BW | 48，96 |
| Resource pool | dedicated |
| Note 1: Unless otherwise specified in the test case |  |

4 References

- R4-2403368 WF on R18 NR positioning - SL positioning and Carrier Phase Positioning, CATT, RAN4#110

- R4-2318280 Simulation results for sidelink positioning, CATT, RAN4#109

- R4-2321610 Updated summary of the simulation results for sidelink positioning measurement, CATT, RAN4#109