3GPP TSG-RAN WG4 Meeting #110bis	                                                       R4-2404407

Changsha, China, 15 April – 19 April, 2024

Title:	Discussion on performance requirements for NTN-TN and NTN-NTN mobility and service continuity enhancements

Source:	CATT

Agenda Item:	6.16.7.3

Document for:	Discussion

1 Introduction

In last meeting, the performance requirements for NTN were not discussed but some baseline for the test cases was given in the approved WF [1]. In this paper, we further discuss the performance requirements for NTN-TN and NTN-NTN mobility and give our proposals.

2 Discussion

In last meeting, the following agreements were derived for the test case of NTN-TN and NTN-NTN mobility.

### Issue 6-3: NTN bands below 10 GHz Agreement: - For FR1-NTN test cases, the below are baseline. - Network verified UE location - NTN-TN cell reselection - NTN to NTN time-based measurement initiation for cell reselection in earth-moving cell, only for satellite switch - NTN to NTN location-based measurement initiation for cell reselection in earth-moving cell, for cell switch - NTN to NTN RACH-less (C)HO - NTN to NTN Satellite switching without PCI change - Hard switch - RACH based - RACH-less - soft switch - RACH based - RACH-less - NTN to NTN time-based trigger CHO enhancements - NTN to NTN location- based trigger CHO enhancements

The baseline well covered all the defined requirements. But to reduce the number of test cases, some down-selection can be considered. From our perspective, for NTN to NTN Satellite switching without PCI change, we think there is no need to define RACH-less switch since the RACH-less procedure can be verified in (C)HO test cases. Also for the time/location-based measurement initiation for cell reselection, since the test cases in existing specification do not differentiate fixed or moving cell, there is no need to introduce new test cases.

Proposal 1: For NTN to NTN Satellite switching without PCI change, only define RACH-based test cases including both hard switch and soft switch.

Proposal 2: Do not define test case for NTN to NTN time/location-based measurement initiation for cell reselection in earth-moving cell.

3 Summary

In this paper, we have some further discussions on the performance requirements for NTN-TN and NTN-NTN mobility, and the following proposals are given:

Proposal 1: For NTN to NTN Satellite switching without PCI change, only define RACH-based test cases including both hard switch and soft switch.

Proposal 2: Do not define test case for NTN to NTN time/location-based measurement initiation for cell reselection in earth-moving cell.

4 References

- R4-2403492 WF on RRM requirements for NR_NTN_enh, Qualcomm, RAN4#110