3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2404415

# Changsha, CN, 15th Apr – 19st Apr, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test cases for intra-f RACH based PCell switch from FR1 to FR1 for LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test cases for intra-f RACH based PCell switch without early TCI state activation from FR1 to FR1 for LTM shall be introduced into TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following test cases in TS 38.133 for LTM requirements: - A.6.3.x.1 Intra-frequency RACH based LTM PCell switch without early TCI state activation from FR1 to FR1; known target cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for intra-f RACH based PCell switch without early TCI state activation from FR1 to FR1 for LTM would be missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.6.3.x.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.3.x.1 Intra-frequency RACH based LTM PCell switch without early TCI state activation from FR1 to FR1; known target cell

A.6.3.x.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency LTM PCell switch without early TCI state activation requirements specified in clause 6.3.1.

A.6.3.x.1.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Supported test configurations are shown in table A.6.3.x.1.2-1. Both LTM PCell switch delay and interruption length are tested by using the parameters in table A.6.3.x.1.2-2, and A.6.3.x.1.2-3.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. No gap patterns are configured in the test case.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	The UE has performed L3 measurement and SSB based L1-RSRP measurement on cell 2.

T1 starts from UE transmitting a valid L1 report on cell 2. After receiving the first L1 report on cell 2 during T1.

The start of T2 is the instant when the last TTI containing LTM cell switch command MAC CE is sent to the UE. Contention-free Random Access Resources are indicated. The field of Timing Advance Command is set to FFF.

Table A.6.3.x.1.2-1: Intra-frequency LTM PCell switch from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.x.1.2-2: General test parameters Intra-frequency LTM PCell switch from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | DRX is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 2 s | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |
| T1 |  | s | 0.3 |  |
| T2 |  | s | 5 |  |

Table A.6.3.x.1.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency LTM PCell switch test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |
| CP length |  |  |  | Normal |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |
|  |  |  | dB | -0.64 | -0.64 | -0.64 | -0.64 |
|  |  |  | dB | 8 | 8 | 8 | 8 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 | -90 | -90 |
|  | Config 3 |  | dBm/SCS | -87 | -87 | -87 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -61.41 | -61.41 | -61.41 |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -55.31 | -55.31 | -55.31 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

A.6.3.x.1.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 in no later than Tcmd + TLTM-interrupt from the beginning of time period T2.

The rate of correct LTM PCell switchs observed during repeated tests shall be at least 90%.

NOTE:	The LTM PCell switch delay can be expressed as: Tcmd + TLTM-interrupt, where:

Tcmd = THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement as specified in TS 38.213 [3].

TLTM-interrupt is defined in clause 6.3.1.3 as TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. In the test, Tfirst-RS + TRS-proc=0, and

-	TLTM-interrupt = 50 ms if UE supports neither [Early processing of an LTM candidate cell RRC configuration] nor ltm-FastUE-Processing-R18 with TLTM-RRC-processing =10ms, TLTM-processing =20ms and TLTM-IU =20ms.

-	TLTM-interrupt = 40 ms if UE supports [Early processing of an LTM candidate cell RRC configuration] and does not support ltm-FastUE-Processing-R18 with TLTM-RRC-processing =0ms, TLTM-processing =20ms and TLTM-IU =20ms.

-	TLTM-interrupt = 35 ms if UE supports [Early processing of an LTM candidate cell RRC configuration] and reports 15ms for FR1-to-FR1 cell switch in the capability ltm-FastUE-Processing-R18, with TLTM-RRC-processing = 0ms, TLTM-processing =15ms and TLTM-IU =20ms.

-	TLTM-interrupt = 30 ms if UE supports [Early processing of an LTM candidate cell RRC configuration] and reports 10ms for FR1-to-FR1 cell switch in the capability ltm-FastUE-Processing-R18, with TLTM-RRC-processing = 0ms, TLTM-processing =10ms and TLTM-IU =20ms.

-	TLTM-interrupt = 45 ms if UE does not support [Early processing of an LTM candidate cell RRC configuration] and reports 15ms for FR1-to-FR1 cell switch in the capability ltm-FastUE-Processing-R18, with with TLTM-RRC-processing =10ms, TLTM-processing =15ms and TLTM-IU =20ms.

-	TLTM-interrupt = 40 ms if UE does not support [Early processing of an LTM candidate cell RRC configuration] and reports 10ms for FR1-to-FR1 cell switch in the capability ltm-FastUE-Processing-R18, with TLTM-RRC-processing =10ms, TLTM-processing =10ms and TLTM-IU =20ms.

<End of Change 1>