3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2404435

# Changsha, CN,  Apr.15 - Apr.19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-1 | CR | draftCR | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.115-1, Introduction of general and operating band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce the general and channel bandwidth part for NCR |  |  |  |  |  |  |  |  |
| Summary of change: |  | New NCR related description in Clause 4.8, 4.10, 4.11 and Clause 5 New Clause 4.9A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Applicability of requirement, and RF channel and test models for NCR would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8, New4.9A, 4.10, 4.11, 5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 4.8	Applicability of requirements

### 4.8.1	General

### 4.8.2	Applicability of test configurations for single-band operation

The applicable test configurations are specified in the tables below for each the supported RF configuration, which shall be declared according to clause 4.6. The generation and power allocation for each test configuration is defined in clause 4.7. This clause contains the test configurations for repeater or NCR-Fwd capable of single passband, and/or multi-passband operation in both contiguous and non-contiguous spectrum in single band, and the test configurations for a NCR-MT capable of single carrier operation in both contiguous and non-contiguous spectrum in single band

For a repeater declared to support a single passband within a single band (D.2), the test configurations in the second column of table 4.8.2-1 shall be used for testing.

For a NCR declared to support a single passband within a single band (D.2), the test configurations in the second column of table 4.8.2-2 shall be used for testing.

For a repeater declared to support more than one passband within a single band (D.2) and where the parameters in the manufacture's declaration according to clause 4.6 are identical for all passbands, the test configurations in the third column of table 4.8.2-1 shall be used for testing.

For a NCR declared to support more than one passband within a single band (D.2) and where the parameters in the manufacture's declaration according to clause 4.6 are identical for all passbands, the test configurations in the third column of table 4.8.2-2 shall be used for testing.

For a repeater declared to support more than one passband within a single band (D.2) and where the parameters in the manufacture's declaration according to clause 4.6 are not identical for all passbands, the test configurations in the fourth column of table 4.8.2-1 shall be used for testing.

For a NCR declared to support more than one passband within a single band (D.2) and where the parameters in the manufacture's declaration according to clause 4.6 are not identical for all passbands, the test configurations in the fourth column of table 4.8.2-2 shall be used for testing.

Table 4.8.2-1: Test configurations for a repeater capable of single or multiple passbands in a single band

| Test case | Single passband repeater | Multiple passband capable repeater with identical parameters per passband | Multiple passband capable repeater with different parameters per passband |
| --- | --- | --- | --- |
| Repeater output power | RTC1 | RTC1 | RTC1, RTC2 |
| Frequency stability | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Out of band gain | N/A | N/A | N/A |
| Transmit ON/OFF power (only applied for NR TDD repeater) | RTC1 | RTC1 | RTC1, RTC2 |
| Error Vector Magnitude | RTC1 | RTC1 | RTC1, RTC2 |
| Adjacent Channel Leakage power Ratio (ACLR) | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Cumulative ACLR requirement in non-contiguous spectrum | - | RTC2 | RTC2 |
| Operating band unwanted emissions | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Transmitter spurious emissions | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Output intermodulation | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Input intermodulation | N/A | N/A | N/A |
| Adjacent Channel Rejection Ratio (ACRR) | RTC1 | RTC2 | RTC1, RTC2 |
| Receiver spurious emissions | RTC1 | RTC1, RTC2 | RTC1, RTC2 |

Table 4.8.2-2: Test configurations for a NCR capable of single or multiple passbands in a single band

| Test case | Single passband NCR | Multiple passband capable NCR with identical parameters per passband | Multiple passband capable NCR with different parameters per passband |
| --- | --- | --- | --- |
| NCR output power | NCRTC1 | NCRTC1 | NCRTC1, NCRTC2 |
| Frequency stability | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Out of band gain | N/A | N/A | N/A |
| Transmit ON/OFF power (only applied for NR TDD NCR) | NCRTC1 | NCRTC1 | NCRTC1, NCRTC2 |
| Error Vector Magnitude | NCRTC1 | NCRTC1 | NCRTC1, NCRTC2 |
| Adjacent Channel Leakage power Ratio (ACLR) | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, RTC2 |
| Cumulative ACLR requirement in non-contiguous spectrum | - | NCRTC2 | NCRTC2 |
| Operating band unwanted emissions | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 |
| Transmitter spurious emissions | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 |
| Output intermodulation | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 |
| Input intermodulation | N/A | N/A | N/A |
| Adjacent Channel Rejection Ratio (ACRR) | NCRTC1 | NCRTC2 | NCRTC1, NCRTC2 |
| Output power dynamics (Note 1) | SC | SC | SC |
| Transmitter signal quality (Note 1) | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Transmit intermodulation (Note 1) | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 |
| Reference sensitivity (Note 1) | SC | SC | SC |
| Adjacent channel selectivity (Note 1) | NCRTC1 | NCRTC2 | NCRTC1, INCRTC2 |
| Reveiver Blocking characteristics (Note 1) | NCRTC1 | NCRTC2 | NCRTC1, NCRTC2 |
| Reveiver Intermodulation (Note 1) | NCRTC1 | NCRTC2 | NCRTC1, NCRTC2 |
| Receiver spurious emissions (Note 1) | NCRTC1 | NCRTC1, NCRTC2 | NCRTC1, NCRTC2 |
| Note 1: The test configuration is only applied to WA NCR-MT. For LA NCR-MT, the test configuration will refer to the corresponding test configuration in TS 38.521 |  |  |  |

### 4.8.3	Requirement set applicability

In table 4.8.3-1, the requirement applicability for each requirement set of RF repeater, NCR-Fwd and NCR-MT is defined. For each requirement, the applicable requirement clause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 4.8.3-1: Requirement set applicability for RF repeater, NCR-Fwds and NCR-MTs

| Requirement | RF repeater Requirement set | NCR -Fwd Requirement set | NCR -MT Requirement set |
| --- | --- | --- | --- |
| Output power | 6.2 | 6.2 | 6.2 |
| Frequency stability | 6.3 | 6.3 | NA |
| Out of band gain | 6.4 | 6.4 | NA |
| Unwanted emission | 6.5 | 6.5 | 6.5 |
| Error Vector Magnitude | 6.6 | 6.6 | NA |
| Input intermodulation | 6.7 | 6.7 | NA |
| Output intermodulation | 6.8 | 6.8 | NA |
| Adjacent Channel Rejection Ratio | 6.9 | 6.9 | NA |
| Transmit ON/OFF power | 6.10 | 6.10 | NA |
| Output power dynamic | NA | NA | 6.11 |
| Transmit signal quality | NA | NA | 6.12 |
| Transmit intermodulation | NA | NA | 6.13 |
| Reference sensitivity | NA | NA | 6.16 |
| Maximum input level | NA | NA | 6.17 (Note 1) |
| Adjacent channel selectivity | NA | NA | 6.18 |
| Reveiver Blocking characteristics | NA | NA | 6.19 |
| Reveiver Spurious response | NA | NA | 6.20 (Note 2) |
| Reveiver Intermodulation | NA | NA | 6.21 |
| Reveiver spurious emissions | NA | NA | 6.22 |
| Note 1: Maximum input level is only applied to LA NCR-MT Note 2: Reveiver Spurious response is only applied to LA NCR-MT |  |  |  |

### 4.8.4	Applicability of test configurations for multi-band operation

For a repeater declared to be capable of multi-band operation, the test configuration in table 4.8.4-1 and/or table 4.8.3-1 shall be used for testing. In the case where multiple bands are mapped on common multi-band connector, the test configuration in the second column of table 4.8.4-1 shall be used. In the case where multiple bands are mapped on common single-band connector, the test configuration in table 4.8.3-1 shall be used. In the case where multiple bands are mapped on separate single-band connector or multi-band connector, the test configuration in the third column of table 4.8.4-1 shall be used.

For a NCR declared to be capable of multi-band operation, the test configuration in table 4.8.4-2 and/or table 4.8.3-2 shall be used for testing. In the case where multiple bands are mapped on common multi-band connector, the test configuration in the second column of table 4.8.4-2 shall be used. In the case where multiple bands are mapped on common single-band connector, the test configuration in table 4.8.4-2 shall be used. In the case where multiple bands are mapped on separate single-band connector or multi-band connector, the test configuration in the third column of table 4.8.4-2 shall be used.

Table 4.8.4-1: Test configuration for a BS capable of multi-band operation

| BS test case | Test configuration |  |
| --- | --- | --- |
|  | Common connector | Separate connectors |
| Repeater output power | RTC1/2 (Note 1), RTC3 | RTC1/2 (Note 1), RTC3 |
| Frequency stability | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Out of band gain | N/A | N/A |
| Transmit ON/OFF power (only applied for NR TDD BS) | RTC3 | RTC3 |
| Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Error Vector Magnitude (Note 8) | RTC1/2 (Note 1), RTC3 | RTC1/2 (Note 1), RTC3 |
| Adjacent Channel Leakage power Ratio (ACLR) | RTC1/2 (Note 1), RTC4 (Note 2) | RTC1/2 (Note 1, 3), RTC4 (Note 2, 3) |
| Cumulative ACLR requirement in non-contiguous spectrum | RTC2 (Note 1), RTC4 (Note 2) | RTC2 (Note 1, 3) |
| Operating band unwanted emissions | RTC1/2 (Note 1), RTC4 | RTC1/2 (Note 1, 3), RTC4 (Note 3) |
| Transmitter spurious emissions | RTC1/2 (Note 1), RTC4 | RTC1/2 (Note 1, 3), RTC4 (Note 3) |
| Output intermodulation | RTC1/2 (Note 1) | RTC1/2 (Note 1, 3) |
| Input Intermodulation | N/A | N/A |
| Adjacent Channel Rejection Ratio | RTC1/2 (Note 1), RTC4 (Note 2) | RTC1/2 (Note 1, 3), RTC4 (Note 2, 3) |
| Receiver spurious emissions | RTC1/2 (Note 1), RTC4 | RTC1/2 (Note 1, 3), RTC4 (Note 3) |
| Note 1: RTC1 and/or RTC2 shall be applied in each supported operating band. Note 2: RTC4 may be applied for Inter RF Bandwidth gap only. Note 3: For single-band operation test, other antenna connector(s) is (are) terminated. |  |  |

Table 4.8.4-2: Test configuration for a NCR capable of multi-band operation

| BS test case | Test configuration |  |
| --- | --- | --- |
|  | Common connector | Separate connectors |
| NCR output power | NCRTC1/2 (Note 1), NCRTC3 | NCRTC1/2 (Note 1), NCRTC3 |
| Frequency stability | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Out of band gain | N/A | N/A |
| Transmit ON/OFF power (only applied for NR TDD NCR) | NCRTC3 | NCRTC3 |
| Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Error Vector Magnitude | NCRTC1/2 (Note 1), NCRTC3 | NCRTC1/2 (Note 1), NCRTC3 |
| Adjacent Channel Leakage power Ratio (ACLR) | NCRTC1/2 (Note 1), NCRTC4 (Note 2) | NCRTC1/2 (Note 1, 3), NCRTC4 (Note 2, 3) |
| Cumulative ACLR requirement in non-contiguous spectrum | NCRTC2 (Note 1), NCRTC4 (Note 2) | NCRTC2 (Note 1, 3) |
| Operating band unwanted emissions | NCRTC1/2 (Note 1), NCRTC4 | NCRTC1/2 (Note 1, 3), NCRTC4 (Note 3) |
| Transmitter spurious emissions | NCRTC1/2 (Note 1), NCRTC4 | NCRTC1/2 (Note 1, 3), NCRTC4 (Note 3) |
| Output intermodulation | NCRTC1/2 (Note 1) | NCRTC1/2 (Note 1, 3) |
| Input Intermodulation | N/A | N/A |
| Adjacent Channel Rejection Ratio | NCRTC1/2 (Note 1), NCRTC4 (Note 2) | NCRTC1/2 (Note 1, 3), NCRTC4 (Note 2, 3) |
| Receiver spurious emissions | NCRTC1/2 (Note 1), NCRTC4 | NCRTC1/2 (Note 1, 3), NCRTC4 (Note 3) |
| Output power dynamics (Note 4) | SC | SC |
| Transmitter signal quality (Note 4) | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Transmit intermodulation (Note 4) | NCRTC1 | NCRTC1, NCRTC2 |
| Reference sensitivity (Note 4) | SC | SC |
| Adjacent channel selectivity (Note 4) | NCRTC1 | NCRTC2 |
| Reveiver Blocking characteristics (Note 4) | NCRTC1 | NCRTC2 |
| Reveiver Intermodulation (Note 4) | NCRTC1 | NCRTC2 |
| Receiver spurious emissions (Note 4) | NCRTC1 | NCRTC1, NCRTC2 |
| Note 1: NCRTC1 and/or NCRTC2 shall be applied in each supported operating band. Note 2: NCRTC4 may be applied for Inter RF Bandwidth gap only. Note 3: For single-band operation test, other antenna connector(s) is (are) terminated. Note 4: The test configuration is only applied to WA NCR-MT. For LA NCR-MT, the test configuration will refer to the corresponding test configuration in TS 38.521 |  |  |

<End of Change 1>

<Start of Change 2>

4.9A	RF channels and test models for NCR

4.9A.1	RF channels

For the single passband or carrier testing many tests in this TS are performed with appropriate frequencies in the bottom, middle and top channels of the supported frequency range of the NCR. These are denoted as RF channels B (bottom), M (middle) and T (top).

Unless otherwise stated, the test shall be performed with a single passband or carrier at each of the RF channels B, M and T.

Many tests in this TS are performed with the maximum NCR RF Bandwidth located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BRFBW (bottom), MRFBW (middle) and TRFBW (top).

Unless otherwise stated, the test shall be performed at BRFBW, MRFBW and TRFBW defined as following:

-	BRFBW: maximum NCR RF Bandwidth located at the bottom of the supported frequency range in the operating band.

-	MRFBW: maximum NCR RF Bandwidth located in the middle of the supported frequency range in the operating band.

-	TRFBW: maximum NCR RF Bandwidth located at the top of the supported frequency range in the operating band.

For NCR capable of multi-band operation, unless otherwise stated, the test shall be performed at BRFBW_T'RFBW and B'RFBW_TRFBW defined as following:

-	BRFBW_T'RFBW: the NCR RF Bandwidths located at the bottom of the supported frequency range in the lowest operating band and at the highest possible simultaneous frequency position, within the Maximum Radio Bandwidth, in the highest operating band.

-	B'RFBW_TRFBW: the NCR RF Bandwidths located at the top of the supported frequency range in the highest operating band and at the lowest possible simultaneous frequency position, within the Maximum Radio Bandwidth, in the lowest operating band.

NOTE:	BRFBW_T'RFBW = B'RFBW_TRFBW = BRFBW_TRFBW when the declared Maximum Radio Bandwidth spans all operating bands. BRFBW_TRFBW means the NCR RF Bandwidths are located at the bottom of the supported frequency range in the lower operating band and at the top of the supported frequency range in the upper operating band.

When a test is performed by a test laboratory, the position of B, M and T for single passband or carrier, BRFBW, MRFBW and TRFBW for single band operation, the position of BRFBW_T'RFBW and B'RFBW_TRFBW in the supported operating band combinations shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

4.9A.2	Test models

4.9A.2.1	General

The following clauses will describe the FR1 test models needed for NCR type 1-C and NCR type 1-H.

4.9A.2.2	FR1 test models for NCR for DL

FR1 test model in clause 4.9.2.2 in TS 38.141-1[7] applies to NCR type 1-C and NCR type 1-H as below:

-	NR-FR1-TM1.1 applies to NCRDL-FR1-TM1.1

-	NR-FR1-TM1.2 applies to NCRDL-FR1-TM1.2

-	NR-FR1-TM2 applies to NCRDL-FR1-TM2

-	NR-FR1-TM2a applies to NCRDL-FR1-TM2a

-	NR-FR1-TM3.1 applies to NCRDL-FR1-TM3.1

-	NR-FR1-TM3.1a applies to NCRDL-FR1-TM3.1a

-	NR-FR1-TM3.2 applies to NCRDL-FR1-TM3.2

-	NR-FR1-TM3.3 applies to NCRDL-FR1-TM3.3

Testing models applying to NB-IoT operation in clause 4.9.2.2 in TS 38.141-1[7] are not applicable to NCR.

4.9A.2.3		FR1 test models for NCR for UL

##### 4.9A.2.3.1	General

The set-up of physical channels for transmitter tests shall be according to one of the FR1 test models (NCRUL- FR1-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NCR test models:

-	Duration is 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing

-	NRB is the maximum transmission bandwidth configuration seen in clause 5.3.2 in TS 38.106[2].

-	Normal CP

-	Virtual resource blocks of localized type

NCR test models are derived based on the uplink/downlink configuration as shown in the table 4.9A.2.3.1-1 using information element TDD-UL-DL-ConfigCommon as defined in TS 38.331 [10].

Table 4.9A.2.3.1-1: Configurations of TDD for NCR type 1-C and NCR type 1-H test models

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

Common physical channel parameters for all NCR FR1 test models are specified in table 4.9A.2.3.1-2 and table 4.9A.2.3.1-3 for PUSCH. Specific physical channel parameters for NCR FR1 test models are described in clauses 4.9A.2.3.2 to 4.9A.2.3.5.

Table 4.9A.2.3.1-2: Common physical channel parameters for PUSCH for NCR type 1-C and type 1-H test models

| Parameter | Value |
| --- | --- |
| Mapping type | PUSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | pos2 |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | Pos1 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PUSCH EPRE to DM-RS EPRE | 0 dB |

Table 4.9A.2.3.1-3: Common physical channel parameters for PUSCH by RNTI for NCR type 1-C and type 1-H test models

| Parameter | Value |
| --- | --- |
| PUSCH |  |

##### 4.9A.2.3.2	FR1 test model 1.1 (NCRUL-FR1-TM1.1)

This model shall be used for tests on:

-	NCR output power

-	Out of band gain

-	Transmit ON/OFF power

-	Unwanted emissions

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

-	Receiver spurious emissions

-	Transmitter intermodulation

-	Input intermodulation

-	ACRR

Common physical channel parameters are defined in clause 4.9A.2.3.1. Specific physical channel parameters for NCRUL-FR1-TM1.1 are defined in table 4.9A.2.3.2-1.

Table 4.9A.2.3.2-1: Specific physical channel parameters of NCRUL-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PUSCH | NRB |
| Modulation PUSCH | QPSK |

##### 4.9A.2.3.3	FR1 test model 2 (NCRUL-FR1-TM2)

This model shall be used for tests on:

-	Transmitted signal quality

-	EVM of single 64QAM PRB allocation (at lower PSD TX power limit at min power)

-	Frequency stability (at min power)

-	Frequency error (at min power)

Common physical channel parameters are defined in clause 4.9A.2.3.1. Specific physical channel parameters for NCRUL-FR1-TM2 are defined in table 4.9A.2.3.3-1.

Table 4.9A.2.3.3-1: Specific physical channel parameters of NCRUL-FR1-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PUSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0. 3n+1. 3n+2. |
| # of PUSCH PRBs which are not allocated |  |

##### 4.9A.2.3.3a	FR1 test model 2a (NCRUL-FR1-TM2a)

This model shall be used for tests on:

-	EVM of single 256QAM PRB allocation (at min power)

-	Frequency stability (at min power)

-	Frequency error (at min power)

Common physical channel parameters are defined in clause 4.9A.2.3.3. Physical channel parameters and numbers of the allocated PRB are defined in table 4.9A.2.3.3-1 with all 64QAM PUSCH PRBs replaced by 256QAM PUSCH PRBs.

##### 4.9A.2.3.4	FR1 test model 3.1 (NCRUL-FR1-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper TX PSD power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error (at max power)

-	Frequency stability (at min power)

-	EVM for modulation (at max power)

Common physical channel parameters are defined in clause 4.9A.2.3.1. Physical channel parameters are defined in table 4.9A.2.3.2-1 with all QPSK PUSCH PRBs replaced with selected modulation order PUSCH PRBs according to the corresponding test procedure.

##### 4.9A.2.3.5	FR1 test model 3.1a (NCRUL-FR1-TM3.1a)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper TX PSD power limit at max power with all 256QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency stability (at max power)

-	Frequency error (at max power)

-	EVM for 256QAM modulation (at max power)

Common physical channel parameters are defined in clause 4.9A.2.3.1. Physical channel parameters are defined in table 4.9A.2.3.2-1 with all QPSK PUSCH PRBs replaced by 256QAM PUSCH PRBs.

#### 4.9A.2.4	Data content of Physical channels and Signals for NCRUL-FR1-TM

##### 4.9A.2.4.1	General

Randomisation of the data content is obtained by utilizing a PN sequence generator and the length-31 Gold sequence scrambling of TS 38.211 [9], clause 5.2.1 which is invoked by all physical channels prior to modulation and mapping to the RE grid.

Initialization of the scrambler and RE-mappers as defined in TS 38.211 [9] use the following additional parameters:

-	 , Physical layer cell identity	= 1 is used as the default physical layer cell identity

-	q = 0 (single code word)

-	Rank 1, single layer

##### 4.9A.2.4.2	PUSCH

-	Generate the required amount of bits from the output of the PN23 sequence generator [15]. The PN sequence generator is initialized with a starting seed of "all ones" in the first allocated slot of each frame. The PN sequence is continuous over the slot boundaries.

-	Perform user specific scrambling according to TS 38.211 [9], clause 6.3.1.1.

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS 38.211 [9], clause 6.3.1.2

-

-	Perform mapping of the complex-valued symbols to layer according to TS 38.211 [9], clause 6.3.1.3.

-	Perform PUSCH mapping according to TS 38.211 [9] using parameters listed in table 4.9.2.3-2.

-	DM-RS sequence generation according to TS 38.211 [9], clause 6.4.1.1.1 where l is the OFDM symbol number within the slot with the symbols indicated by table 4.9.2.2-3.

-

-

-	DM-RS mapping according to TS 38.211 [9], clause 6.4.1.1.3 using parameters listed in table 4.9.2.2-3.

<End of Change 2>

<Start of Change 3>

4.10	Requirements for contiguous and non-contiguous spectrum

A spectrum allocation where a repeater or NCR operates can either be contiguous or non-contiguous. Unless otherwise stated, the requirements in the present specification apply for repeater or NCR configured for both contiguous spectrum operation and non-contiguous spectrum operation.

For repeater or NCR operation in non-contiguous spectrum, some requirements apply both at the repeater or NCR passband edges and inside the sub-block gaps. For each such requirement, it is stated how the limits apply relative to the repeater or NCR passband edges and the sub-block edges respectively.

<End of Change 3>

<Start of Change 4>

4.11	Requirements for repeater capable of multi-band operation

For multi-band connector the conducted test requirements in clause 6 apply separately to each supported operating band unless otherwise stated. For some conducted test requirements, it is explicitly stated that specific additions or exclusions to the requirement apply at multi-band connector(s) as detailed in the requirement clause. For repeater or NCR capable of multi-band operation, various structures in terms of combinations of different transceivers in downlink and transceivers in uplink implementations (multi-band or single band) with mapping of transceivers to one or more antenna connectors for repeater or NCR type 1-C or one or more TAB connectors for NCR type 1-Hin different ways are possible. For multi-band connector(s) the exclusions or provisions for multi-band apply. For single-band connector(s), the following applies:

-	Single-band transmitter spurious emissions, operating band unwanted emissions, ACLR, output intermodulation, ACRR and receiver spurious emissions requirements apply to this connector that is mapped to single-band.

-	If the repeater or NCR is configured for single-band operation, single-band requirements shall apply to this connector configured for single-band operation and no exclusions or provisions for multi-band capable repeater or NCR are applicable. Single-band requirements are tested separately at the connector configured for single-band operation, with all other connectors terminated.

For multi-band connectors supporting the bands for TDD, the RF requirements in the present specification assume no simultaneous uplink and downlink occur between the bands.

NOTE 1: The conducted test requirements for multi-band connectors supporting bands for both FDD and TDD are not covered by the present release of this specification.

<End of Change 4>

<Start of Change 5>

5	Operating bands and channel arrangement

For the NR repeater operation in NR operating bands specification, their channel bandwidth configurations, channel spacing and raster, as well as synchronization raster specification, refer to TS 38.106 [2], clause 5 and its relevant clauses.

<End of Change 5>