3GPP TSG-RAN WG4 Meeting #110bis	R4-2404508

Changsha, China, April 15 – 19, 2024

Source:	Huawei, Hisilicon

Title:	Discussion on the remaining issues for the feature list of Rel-18 Tx switching

Agenda item:			6.13.1

Document for:	Approval

# 1. Introduction

In RAN4#109, the LS [1] was sent to RAN2 including RAN4’s conclusion on the ambiguous switching period for the fallback band combinations.

- UE could additionally optionally indicate in a parent BC whether to support the fallback low order BC with the same Tx switching period capability. The network determines switching period for band pair among the applicable ones and signals to the UE with RRC signalling.

Considering it is RAN4 who identifies the issue and triggers the introduction of the capability, the feature should be included in the feature lists from RAN4.

The paper is to discuss the remaining issue for the feature list of Rel-18 Tx switching.

# 2. Discussion

To capture the RAN2 #125 agreement in March related to MC enhancements:

- For a given BC supporting UL Tx switching across up to 4 bands: - When the optional capability is not reported, it means all the fallback BCs are supported by the UE as legacy with the same switching capabilities reported in the parent BC including switching period as legacy. - When the optional capability is reported, it means all the fallback BCs are supported by the UE with the largest switching period value, i.e. 210us. - No matter the optional capability is reported or not, the UE can advertise fallback band combinations with different/same switching period by separate BandCombination entries as legacy in case of different fallback.

The design of UE capability and RRC signalling would be slightly different from RAN4’s conclusion, but it still solve the problem of ambiguous switching period for the fallback band combinations. Therefore, we propose to capture the following feature group related to switchingPeriodRestriction in UE feature list for Rel-18 multi-carrier enhancements WI.

Proposal 1: Capture the feature group of ‘Switching period restriction for fallback band combination’ in the following table in RAN4 feature list for Rel-18 multi-carrier enhancements WI.

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-7 | Switching period restriction for fallback band combination | Indicates the largest value, i.e. 210us is supported for each band pair in all fallback band combinations for a given band combination supporting UL Tx switching across up to 4 bands. | 38-1 | Yes | N/A | The same switching period reported for each band pair in this band combination is supported for the same band pair in all the fallback band combinations. | Per BC | No | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |

# 3. Conclusions

This contribution presents the following proposal:

Proposal 1: Capture the feature group of ‘Switching period restriction for fallback band combination’ in the following table in RAN4 feature list for Rel-18 multi-carrier enhancements WI.

# 4. Reference

[1] R4-2321986, LS on Rel-18 Tx switching enhancement, RAN4#109