3GPP TSG-RAN WG4 Meeting # 110	R4-2404509

# Changsha, China, April 15 – 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | <Rev#> | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_MC_enh-Core) draftCR for 38.101-1: Correction on time mask for Rel-18 Tx switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To capture the RAN2 #125 agreement related to MC cenh: - To refine the RAN4 agreed RRC configuration as below: For each band pair, a RRC parameter is introduced to configure switching period value between value 35 us and 140 us. When the RRC parameter is absent, 210us is applied. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To introduce the RRC signaling switchingPeriodConfigForBandPair according to the updated RAN2 agreement and spec to determine the switching period for a band pair. And the switching period per band pair is not indicated by UE capability any more in Rel-18. - To refine the Rel-18 multicarrier part in sub-clause 5.5C with RAN2 parameters according to the agreed CR R4-2400851. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE and the network may have different understanding on switching period. RAN4 and RAN2 specifications are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5C, 6.3A.3.3.6, 6.3C.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

5.5C	Configurations for SUL

The configuration tables for SUL describe Bandwidth Combination Sets. Bandwidth Combination Set 4 and 5 contains all possible defined channel bandwidths for each band in the combination. The fact that BCS4 and BCS5 contains all channel bandwidths for each band does not alter if a bandwidth is mandatory or optional for a given band. Bandwidths that are identified as optional in Table 5.3.5-1 for a given release are still optional for UEs that support BCS4 or BCS5. , where the bandwidths the UE supports for each band, the maximum bandwidth and/or minimum bandwidth for the band in the band combination are indicated in the UE capabilities. The minimum bandwidth per CC and aggregated FDD, TDD and total bandwidth per band combination may be indicated only for BCS5 as described in 38.306 [15] and BCS5 shall not be indicated together with BCS4 for a SUL configuration. For SUL band combinations including FR1 intra-band CA and with BCS4 or BCS5, the Bandwidth Combination Sets for the FR1 intra-band CA are BCS4 or BCS5.

For the NR SUL band configurations with inter-band CA in sub-clause 5.5C, when the capability supportedBandPairListNR-r18 is present, three or four bands can be configured in the uplink with simultaneous uplink transmission on up to two bands, and the corresponding requirements for SUL band configurations with inter-band CA and with uplink assigned to one or two bands shall apply. For each uplink band pair in the NR SUL band configurations with inter-band CA, according to the capability uplinkTxSwitchingOptionForBandPair,

–	if switchedUL is supported, uplink transmission on any one band of the band pair in the band combination shall be supported according to the scheduling commands, and the corresponding requirements for SUL band configuration with inter-band CA and with uplink assigned to one band on band X or band Y apply;

–	if dualUL is supported, simultaneous uplink transmission on the two NR UL bands from the band pair for which dualUL is declared in the band combination shall be supported according to the scheduling commands, and the corresponding requirements for SUL band configuration with inter-band CA and with uplink CA between the two uplink bands apply.

For SUL band configuration with inter-band CA, band pair(s) of two non-SUL bands with switchedUL or dualUL by the parameter uplinkTxSwitchingOption is supported, and any other band pair(s) including SUL with switchedUL is supported, in release 18.

<… Unchanged part is omitted….>

<<Next Change>>

6.3A.3.3.6	Time mask for switching across up to four uplink bands

The switching time mask requirements specified in this sub-clause are applicable for an NR inter-band CA configuration when the capability supportedBandPairListNR-r18 is present, and are only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10].

In the NR inter-band CA configuration, the number of NR uplink bands is up to four. NR UL carrier(s) in each of the up to four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability FeatureSetUplinkPerCC.

The switching time masks in Figure 6.3A.3.3.6-1 and Figure 6.3A.3.3.6-2 are applicable to each of the uplink band pairs in the CA configuration, and are applicable to uplink transmissions when configured with switchedUL or dualUL by the parameter switchingOptionConfigForBandPair. To simplify the figures, the two bands in different band pairs are denoted as NR band X and band Y. The uplink transmission on either band X or band Y is with one or two transmit antenna connector(s),

–	if NR UL carriers in both bands in one band pair are capable of one transmit antenna connector, 1Tx-1Tx switching is supported for the band pair;

–	if NR UL carrier(s) in one band of one band pair is capable of one transmit antenna connector, and NR UL carrier(s) in the other band of the band pair is capable of two transmit antenna connectors, 1Tx-2Tx switching is supported for the band pair;

–	if NR UL carriers in both bands of one band pair are capable of two transmit antenna connectors, 2Tx-2Tx switching is supported for the band pair.

For each band pair, the switching periods described in Figure 6.3A.3.3.6-1 and Figure 6.3A.3.3.6-2 are located in either NR band X or band Y as indicated in RRC signalling uplinkTxSwitchingBandList [7]. For each band pair, the length of uplink switching period X is indicated by RRC signalling switchingPeriodConfigForBandPair. UE shall be capable to transmit until the beginning of the switching period and after the end of switching period with the exception of transient periods.

Figure 6.3A.3.3.6-1: Time mask for switching between band X and band Y, where the switching period is located in band X

Figure 6.3A.3.3.6-2: Time mask for switching between band X and band Y, where the switching period is located in band Y

The following applies for the uplink switching cases specified in Figure 6.3A.3.3.6-1 and 6.3A.3.3.6-2 in a band pair with switchingOptionConfigForBandPair set to either switchedUL or dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching period of X µs indicated by RRC signalling switchingPeriodConfigForBandPairon any of the carriers band X and band Y before T0,

-	the configuration of the location of the switching period by uplinkTxSwitchingBandList is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

In addition to the requirements in Figure 6.3A.3.3.6-1 and Figure 6.3A.3.3.6-2, the requirements in Figure 6.3A.3.3.6-3 to 6.3A.3.3.6-5 are applicable when dualUL are supported on certain band pair(s) in the CA configuration.

The switching time masks in Figure 6.3A.3.3.6-3 and Figure 6.3A.3.3.6-4 are applicable when dualUL is supported for at least two uplink band pairs in the CA configuration. The two band pairs supporting dualUL are denoted as band pairs of {band X and band Z} and {band Y and band Z}. When one transmitter is switched between band X and band Y,

–	As baseline UE behaviour, the UE is not required to transmit on any of the three bands during time period T1 located on band X and band Z, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3A.3.3.6-4.

–	As optional UE behaviours, when the UE indicates band Z in the capability bandIndexUnaffected,

–	if the UE indicates maintainedUL-Trans for band pair of {band X and band Y}, UE shall be capable of uplink transmission on band Z during the switching period that is located on band X, and UE is not required to transmit on band X and Y during time period T1 located on band X, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3A.3.3.6-3;

–	otherwise, the UE is not required to transmit on any of the three bands during the switching period indicated by UE capability periodOnULBands located on band X and band Z, as shown in Figure 6.3A.3.3.6-4.

In Figure 6.3A.3.3.6-3 and Figure 6.3A.3.3.6-4, the uplink transmission on band X, band Y and band Z are all with one transmit antenna connector and one antenna port.

Figure 6.3A.3.3.6-3: Time mask for one transmitter switching between band X and band Y, and UE is capable of uplink transmission on band Z during the switching period

The following applies for the uplink switching cases specified in Figure 6.3A.3.3.6-3 in a band pair with switchingOptionConfigForBandPair set to dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching period X indicated by RRC signalling switchingPeriodConfigForBandPair on any of the carriers in band X and band Y before T0,

-	the configuration of the location of the switching period by uplinkTxSwitchingBandList is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

Figure 6.3A.3.3.6-4: Time mask for one transmitter switching between band X and band Y, and UE is not capable of uplink transmission on band Z during the switching period

The switching time mask in Figure 6.3A.3.3.6-5 is applicable when dualUL is supported for at least one uplink band pair including band X and band Y, and two transmit antenna connectors are supported on at least one uplink band of band Z. When one transmitter is switched between band X and band Z, and across the same time, the other transmitter is switched between Y and band Z, the switching time mask in Figure 6.3A.3.3.6-5 is applicable.

–	As baseline UE behaviour, UE is not required to transmit on any of the three bands during time period with the larger one of swithching period T2 and T3, where T2 is the length of switching period for the band pair of band X and band Z, and T3 is the length of switching period for the band pair of band Y and band Z.

–	As optional UE behaviour, when UE additionally reports band pair of {band X and band Y} and band Z in the capability uplinkTxSwitchingAdditionalPeriodDualUL-List , UE is not required to transmit on any of the three bands during time period indicated by UE capability switchingAdditionalPeriodDualUL.

In Figure 6.3A.3.3.6-5, the uplink transmission on band X and band Y is with one transmit antenna connector and one antenna port, and the uplink transmission on band Z is with two transmit antenna connectors and two antenna ports. The switching period location is configured according to [7], and band Z is with the highest priority according to the RRC configuration uplinkTxSwitchingBandList.

Figure 6.3A.3.3.6-5: Time mask for one transmitter switching between band X and band Z, and one transmitter switching between band Y and band Z

The following applies for the uplink switching case specified in Figure 6.3A.3.3.6-5 and with uplinkTxSwitchingOptionForBandPair set to dualUL for at least one band pair

-	if uplink switching on a band pair is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the maximum of the lengths of uplink switching periods indicated by RRC signalling switchingPeriodConfigForBandPair on any of the carriers in band X, band Y and band Z before T0 on any switched-to carrier

-	the configuration of the location of the switching period and the priority of bands in the uplinkTxSwitchingBandList are ignored by the UE

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the switched-from carrier(s) before T0 on any switched-to carrier and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s)

The requirements in this sub-clause apply for the case of synchronized network deployment for the uplink bands.

<<Next Change>>

6.3C.3.5	Time mask for switching across up to four uplink bands

The switching time mask requirements specified in this sub-clause are applicable for an NR SUL band configuration with inter-band CA when the capability supportedBandPairListNR-r18 is present, and are only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10].

In the NR SUL band configuration with inter-band CA, the number of uplink bands with different carrier frequencies is up to four.  NR UL carrier(s) in each of the up to four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability FeatureSetUplinkPerCC.

The switching time masks in Figure 6.3C.3.5-1 and Figure 6.3C.3.5-2 are applicable to each of the uplink band pairs in the SUL band configuration with inter-band CA, and are applicable to uplink transmissions when configured with switchedUL or dualUL by the parameter switchingOptionConfigForBandPair. To simplify the figures, the two bands in different band pairs are denoted as NR band X and band Y. The uplink transmission on either band X or band Y is with one or two transmit antenna connector(s),

–	if NR UL carriers in both bands in one band pair are capable of one transmit antenna connector, 1Tx-1Tx switching is supported for the band pair;

–	if NR UL carrier(s) in one band of one band pair is capable of one transmit antenna connector, and NR UL carrier(s) in the other band of the band pair is capable of two transmit antenna connectors, 1Tx-2Tx switching is supported for the band pair;

–	if NR UL carriers in both bands of one band pair are capable of two transmit antenna connectors, 2Tx-2Tx switching is supported for the band pair.

For each band pair, the switching periods described in Figure 6.3C.3.5-1 and Figure 6.3C.3.5-2 are located in either NR band X or band Y as indicated in RRC signalling uplinkTxSwitchingBandList [7]. For each band pair, the length of uplink switching period X is indicated by RRC signalling switchingPeriodConfigForBandPair [7]. UE shall be capable to transmit until the beginning of the switching period and after the end of switching period with the exception of transient periods.

Figure 6.3C.3.5-1: Time mask for switching between band X and band Y, where the switching period is located in band X

Figure 6.3C.3.5-2: Time mask for switching between band X and band Y, where the switching period is located in band Y

The following applies for the uplink switching cases specified in Figure 6.3C.3.5-1 and 6.3C.3.5-2 in a band pair with switchingOptionConfigForBandPair set to either switchedUL or dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching period of X µs indicated by RRC signalling switchingPeriodConfigForBandPair on any of the carriers band X and band Y before T0,

-	the configuration of the location of the switching period by [uplinkTxSwitchingBandList-r18] is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

In addition to the requirements in Figure 6.3C.3.5-1 and Figure 6.3C.3.5-2, the requirements in Figure 6.3C.3.5-3 to 6.3C.3.5-5 are applicable when dualUL are supported on certain band pair(s) in the SUL band configuration with inter-band CA.

The switching time masks in Figure 6.3C.3.5-3 and Figure 6.3C.3.5-4 are applicable when dualUL is supported for at least two uplink band pairs in the band configuration. The two band pairs supporting dualUL are denoted as band pairs of {band X and band Z} and {band Y and band Z}. When one transmitter is switched between band X and band Y,

–	As baseline UE behaviour, the UE is not required to transmit on any of the three bands during time period T1 located on band X and band Z, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3C.3.5-4.

–	As optional UE behaviours, when the UE indicates band Z in the capability bandIndexUnaffected,

–	if the UE indicates maintainedUL-Trans for band pair of {band X and band Y} , UE shall be capable of uplink transmission on band Z during the switching period that is located on band X, and UE is not required to transmit on band X and Y during time period T1 located on band X, where T1 is the length of switching period for the band pair of band X and band Y, as shown in Figure 6.3C.3.5-3;

–	otherwise, theUE is not required to transmit on any of the three bands during the switching period indicated by UE capability periodOnULBands [located on band X and band Z, as shown in Figure 6.3C.3.5-4.

In Figure 6.3C.3.5-3 and Figure 6.3C.3.5-4, the uplink transmission on band X, band Y and band Z are all with one transmit antenna connector and one antenna port.

Figure 6.3C.3.5-3: Time mask for one transmitter switching between band X and band Y, and UE is capable of uplink transmission on band Z during the switching period

The following applies for the uplink switching cases specified in Figure 6.3C.3.5-3 in a band pair with switchingOptionConfigForBandPair set to dualUL:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the length of uplink switching periods indicated by RRC signalling switchingPeriodConfigForBandPair on any of the carriers in band X and band Y before T0,

-	the configuration of the location of the switching period by uplinkTxSwitchingBandList is ignored by the UE;

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carrier(s) before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s).

Figure 6.3C.3.5-4: Time mask for one transmitter switching between band X and band Y, and UE is not capable of uplink transmission on band Z during the switching period

The switching time mask in Figure 6.3C.3.5-5 is applicable when dualUL is supported for at least one uplink band pair including band X and band Y, and two transmit antenna connectors are supported on at least one uplink band of band Z. When one transmitter is switched between band X and band Z, and across the same time, the other transmitter is switched between Y and band Z, the switching time mask in Figure 6.3C.3.5-5 is applicable.

–	As baseline UE behaviour, UE is not required to transmit on any of the three bands during time period with the larger one of swithching period T2 and T3, where T2 is the length of switching period for the band pair of band X and band Z, and T3 is the length of switching period for the band pair of band Y and band Z.

–	As optional UE behaviour when UE additionally reports band pair of {band X and band Y} and band Z in the capability uplinkTxSwitchingAdditionalPeriodDualUL-List, UE is not required to transmit on any of the three bands during time period indicated by UE capability switchingAdditionalPeriodDualUL.

In Figure 6.3C.3.5-5, the uplink transmission on band X and band Y is with one transmit antenna connector and one antenna port, and the uplink transmission on band Z is with two transmit antenna connectors and two antenna ports. The switching period location is configured according to [7], and band Z is with the highest priority according to the RRC configuration uplinkTxSwitchingBandList.

Figure 6.3C.3.5-5: Time mask for one transmitter switching between band X and band Z, and one transmitter switching between band Y and band Z

The following applies for the uplink switching case specified in Figure 6.3C.3.5-5 and with uplinkTxSwitchingOptionForBandPair set to dualUL for at least one band pair

-	if uplink switching on a band pair is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the maximum of the lengths of uplink switching periods indicated by RRC signalling switchingPeriodConfigForBandPair on any of the carriers in band X, band Y and band Z before T0 on any switched-to carrier

-	the configuration of the location of the switching period and the priority of bands in the uplinkTxSwitchingBandList are ignored by the UE

-	transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the switched-from carrier(s) before T0 on any switched-to and at the start of the first symbol(s) configured or scheduled at T0 on the switch-to carrier(s)

The requirements in this sub-clause apply for the case of synchronized network deployment for the uplink bands.

<<End of Change>>