3GPP TSG-RAN WG4 Meeting #110-bis	R4-2404530

Changsha, China, Apr 15 – Apr 19, 2024

Source:	Nokia

Title:	On MultiRx Demodulation performance and CSI requirements – PDSCH requirements

Agenda item:	6.3.4.2

Document for:	Discussion

Introduction

In RAN4#110 draftCR [3] was endorsed with requirements agreed in [1][2] and a request for companies to provide updated/new simulation results for the next meeting with a possible reduced span and, to increase the number of results especially where 2 PRTS ports are configured.

This paper contains Nokia’s new and updated simulation results for the sDCI related use cases.

Discussion

Simulation results for sDCI SDM

The following is Nokia’s updated simulation results for sDCI SDM based on the agreed DraftCR from RAN4#110 [3]:

Table 1: Results for sDCI SDM rank 1+1 with MCS17 – Separate processing.

| Simulation Configuration | PTRS Port Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | RB Allocation Size | Cross talk factor (rho) - dB | Fraction of maximum throughput (%) | SNRBB (dB) | SNRBB + impairment (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sDCI SDM Rank 1+1 (-0.0625us, 600Hz) | With 1 PTRS port | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 66 | -12 | 70 | 12.0 | 15.1 |
| sDCI SDM Rank 1+1 (-0.0625us, 600Hz) | With 2 PTRS ports | 100/120 | FR2.120-1 | MCS17 | 1+1 | TDLA30-75 | 66 | -12 | 70 | 11.9 | 15.0 |

In Table 1, we have provided our updated simulation results for sDCI SDM. Based on the collected results from RAN4#110 (see [4]) we see that our new and updated results do not change the span to above 2.5dB when outliers are removed, hence we see that it feasible to update the SNR values for sDCI SDM provided in the endorsed draftCR [3] from RAN4#110 to include our new and updated results.

Our new/updated results do not result in a span >2.5dB when outliers are removed when combined with the RAN4#110 results.

Update sDCI requirements in TS38.101-4 based on the latest provided simulation results.

Conclusion

Within this contribution we have presented our simulation results for the performance alignment of FR2 multi-RX downlink PDSCH demodulation requirements.

We have the following observations and proposals:

- Our new/updated results do not result in a span >2.5dB when outliers are removed when combined with the RAN4#110 results.

- Update sDCI requirements in TS38.101-4 based on the latest provided simulation results.

- References

- R4-2402859 - WF for FR2_multiRX_DL_Demod_v00_mod (RAN4#110)

- R4-2321140 - WF on FR2_Multi-Rx_Demod (RAN4#109)

- R4-2403084 - Draft CR on Minimum requirements and FRC definition for sDCI SDM; Huawei, HiSilicon

- R4-2401139 - Simulation results summary for FR2 multi-Rx performance requirements