3GPP TSG-RAN WG4 Meeting #110bis				                              R4-2404566

Changsha, China, 15th – 19th April, 2024

Agenda Item:	6.5.1.

Title: 	Discussion on RRM requirements for combinations of pre-configured MGs and multiple concurrent MGs

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In the last RAN4 meeting, Rel18 NR measurement gap enhance WI was further discussed. In this paper, the considerations on remaining issues of pre-configured MGs and multiple concurrent MGs under objective 1 [1][2] was provided.

- Discussion on RRM requirements for Case 1

  - Activation delay

For the dynamic collision Scenario 1, one of important open issue is how to define the activation delay requirements [2].

Issue 2-1-1: [Case 1] - [Scenario 1] Further clarification on the agreement from scenario 1? - Background: - Agreements from dynamic collision: - A collision between a change in the status of a pre-configured MG (MG#1) and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG (MG#2) the Pre-MG status and dropping rule shall be applied 5ms after the overlapping MG and UE should continue the measurement within the MG#2 - TBD whether same Pre-MG activation delay requirements as Rel-17 can still be re-used - The collision scenario in this issue is depicted in the figure below:

In TS38.133, the (de)activation delay requirement was composed of several parts:

- The total delay time allowed

- The point of activation/deactivation takes effort

- Some exception conditions to change Pre-MG status

Generally, the (de)activation procedure itself can be same as that of Rel17 Pre-MG. That is:

Observation 1: The overall (de)activation procedure in all dynamic collision scenarios can be consisted with the processing time for the trigger event and other extension for RF processing.

Thus, the total activation time itself defined in Rel17 [3] can be reused for Rel18.However, particularly for the Scenario 1, since agreed in [2] the dropping rule shall be applied 5ms after the overlapping MG and UE should continue the measurement within the MG#2. It will impact the (de)activation delay when the change point taking effect within <5ms transition time window indeed.

Figure 1. Pre-MG activation/deactivation taking effect

Observation 2: Since UE will postpone the status change of Pre-MG in Scenario 1, the Pre-MG status change taking effective time point needs to be updated.

Therefore, we can propose that:

Proposal 1: The time point when Pre-MG activation/deactivation take effects shall explicitly defined in TS38.133 as:

8.19.2 Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch 8.19.2.1 Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC …… Activation of Pre-MG takes effect from the first MG occasion after the activation delay if the time difference between the last overlapping MG and first Pre-MG occasion to be changed is larger than 5ms. Otherwise, the first Pre-MG occasion shall be kept as deactivated also.

The accompanied draftCR to address this issue can be found in [3].

  - Pre-MG association

In the previous meeting, the open issues below was also captured in [2].

Issue 2-2-1: [Case 1] Pre-MG association clarification - Way Forward - Option 1: - When NW configures a Pre-MG1 and a Pre-MG2/Type-2 MG in ConMGs, the MO associated with Pre-MG1 will be measured within activated Pre-MG2/Type-2 MG if Pre-MG1 is deactivated and the MO is fully overlapping with activated Pre-MG2/Type-2 MG. - Option 1a: HW - FFS: whether it need to be captured in spec - Option 2: - RAN4 to discuss options related to UE behaviour, for UE supporting Case 1 requirements, in case of deactivated Pre-MG, i.e. require the UE to perform measurements for MO’s assigned to Pre-MG outside any other MG, or define a priority for deactivated Pre-MG to be compared against priority of any other overlapping MG, or define a Pre-MG association rule by transferring MO’s assigned to Pre-MG to any other active MG (Pre-MG or Type-2 MG) as long as Pre-MG is deactivated

For the cases in which MOs associated with Pre-MG and Tyep2-MG was fully overlapped, there is one important issue below to be clarified.

Observation 3: According to current spec on UE handling the collision in case of concurrent MGs (e.g. 9.1.8.3 in TS38.133 CR[4]), UE is not clear which types of measurement shall be performed (e.g. the measurement within or outside of gap ) when Pre-MG was deactivated if these concurrent MG instances (Pre-MG #1 and Tyep2 MG #2 in Figure 2) was overlapping.

9.1.8.3 Collision between concurrent measurement gaps Collisions between occasions of two concurrent measurement gaps may occur as specified in this clause if the two measurement gaps are - two per-UE measurement gaps, or - two per-FR measurement gaps in the same FR, or - one per-UE measurement gap and one per-FR measurement gap. When UE is configured with concurrent measurement gaps, two measurement gap occasions are considered colliding if at least one of the following conditions is met: - the two occasions are fully or partially overlapping in time domain, or - the distance between the two occasions is equal to or smaller than 4ms. The distance between two measurement gap occasions is defined as the time difference between the ending point of the first occasion and the starting point of the second occasion, where the first measurement gap occasion occurs earlier in time than the second measurement gap occasion. In case of collision between two measurement gap occasions, the UE shall perform measurements in the occasion of the measurement gap with higher priority, and the occasion of the measurement gap with lower priority shall be dropped. The UE shall be able to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots that are not interrupted according to requirements in clause 9.1.8.4. The requirements of concurrent measurement gaps in section 9 shall not apply when a gap without assigned priority is configured simultaneously with any other gap(s) that affect serving carriers in the same FR and the measurement gaps are colliding with each other. The priority for a measurement gap is configured by networks via gapPriority in GapConfig. The requirements with concurrent measurement gaps apply provided that two measurement gaps colliding with each other are configured with different priorities.

Figure 2a. Illustration of Pre-MG association issue (fully overlapping)

In our understanding, there are possible following options:

- Option 1: UE perform measurement for MO#1 within gap as in Rel15 the measurement with gap can be granted with higher priority. And the measurement without gap on MO#2 will be always dropped.

- Option 2: The MO#2 can be associated with type2MG if Pre-MG being deactivated. The corresponding gap sharing mechanism applied.

Furthermore, beside the scenario disused in the previous meeting, the other scenario in which Pre-MG and Tyep2-MG was overlapped but MOs (e.g. SSBs from the different carriers to be measured ) associated with Pre-MG and Tyep2-MG was not overlapped shall be further clarified as in the figure below.

Figure 2b. Illustration of Pre-MG association issue (partially overlapping)

Observation 4: According to Rel17 concurrent MG colliding rules above, Type2 MG and Pre-MG in Figure 2b are collided. UE measurement behavior according to current dropping rules in TS38.133, one of them shall be dropped. As a result, the benefits of concurrent MGs will be vanished.

From the technical perspective, in case of concurrent MG instances overlapping but MOs associated with these gaps are separated enough, UE can perform them one by one. For an example, if SSBs from the different carriers to be measured are configured with different offset within a frame as the figure below, the distance between them can be larger than 1ms definitely.

Figure 3. Possible configuration of SSBs to be measurement of different MOs

Observation 5: SSBs to be measured within the different concurrent gaps which are overlapped can be separated enough.

Therefore, we can propose that:

Proposal 2: The measurement dropping rules when the concurrent measurement gaps are collided can be optimized, e.g.

Option 1). Only there is overlapping among the [SSBs+Xms] to be measured by these collided concurrent gaps, UE needs to drop the measurement with the lower priority gap. Otherwise, UE can perform these measurements sequentially because UE can return to each of carriers one by one.

- Conclusion

In this contribution, serval issues related to the measurement gap enhancement WI in Rel18 are discussed. The proposals can be summarized as:

Observation 1: The overall (de)activation procedure in all dynamic collision scenarios can be consisted with the processing time for the trigger event and other extension for RF processing.

Observation 2: Since UE will postpone the status change of Pre-MG in Scenario 1, the Pre-MG status change taking effective time point needs to be updated.

Proposal 1: The time point when Pre-MG activation/deactivation take effects shall explicitly defined in TS38.133 as:

8.19.2 Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch 8.19.2.1 Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC …… Activation of Pre-MG takes effect from the first MG occasion after the activation delay if the time difference between the last overlapping MG and first Pre-MG occasion to be changed is larger than 5ms. Otherwise, the first Pre-MG occasion shall be kept as deactivated also.

Observation 3: According to current spec on UE handling the collision in case of concurrent MGs (e.g. 9.1.8.3 in TS38.133 CR[4]), UE is not clear which types of measurement shall be performed (e.g. the measurement within or outside of gap ) when Pre-MG was deactivated if these concurrent MG instances (Pre-MG #1 and Tyep2 MG #2 in Figure 2) was overlapping.

Observation 4: According to Rel17 concurrent MG colliding rules above, Type2 MG and Pre-MG in Figure 2b are collided. UE measurement behavior according to current dropping rules in TS38.133, one of them shall be dropped. As a result, the benefits of concurrent MGs will be vanished.

Observation 5: SSBs to be measured within the different concurrent gaps which are overlapped can be separated enough.

Proposal 2: The measurement dropping rules when the concurrent measurement gaps are collided can be optimized, e.g.

Option 1). Only there is overlapping among the [SSBs+Xms] to be measured by these collided concurrent gaps, UE needs to drop the measurement with the lower priority gap. Otherwise, UE can perform these measurements sequentially because UE can return to each of carriers one by one.

- References

- R4-2321604, WF for NR_MGE-2_part1

- R4-2317305, WF on further enhancements on measurement gaps and measurements without gaps

- R4-2401200, Draft CR on activation deactivation delay for Pre-MG within the concurrent MGs

- TS38.133 v18.3.0

- TS38.331 v17.1.0